3GPP TSG-SA5 Meeting #156 	S5-244695

# Maastricht, Netherlands 19 - 23 August 2024                                revision of S5-243747

# Source:	Huawei, Nokia

# Title:	pCR TR 28.915 Solution for using NDT to generate ML training data

# Document for:	Approval

Agenda Item:	6.19.5

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TR 28.915: " Study on management aspects of Network Digital Twin"

3	Rationale

It’s proposed to add solution for use case Using NDT to generate ML training data as described in clause 5.6 of [1]

4	Detailed proposal

This document proposes the following changes in TR 28.915.

1st Change

5.6.3 Potential solutions

Figure 5.6.3-x: procedure of simulated data generation for ML model training

Pre-condition: NDT is created and can support the simulated data generation.

- The MnS consumer, e.g., ML training function, makes preparation of ML model training and decides to collect simulated data to enrich ML model training dataset.

- The MnS consumer requests NDT to generate simulated data. The request parameters may include:

- Object: the managed object which the simulated data is related to, e.g., network slice.

- Data type: the type of data that needs to be generated by NDT for certain managed object, e.g., KPIs/alarms.

- NDT the prepares the simulated data as requested in step 5. NDT simulates the injection and then collects the required simulation data if the injection is contained in step5.

- MnS producer reports the simulated data to MnS consumer. The simulated data is for specific managed object with specific data type as requested in step5.

End of change