3GPP TSG-SA5 Meeting #156 	S5-244858

Maastricht , NL, 19 - 23 Aug 2024

# Source:	China Unicom

# Title:	Add use case, requirements and solution on CN configuration management support for Indirect Network Sharing

# Document for:	Approval

Agenda Item:	6.19.19

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 32.130: "Network sharing; Concepts and requirements".

3	Rationale

As specified in TS 23.501[1]:

"The 5G System may also support Indirect Network Sharing deployment between hosting operator (i.e. shared network operator) and participating operator (see clause 6.21 of TS 22.261 [2], Figure 5.18.1-2 and Annex R), in which the RAN is shared. The communication between the shared RAN and the core network of the participating operator is routed through the core network of the hosting operator that connects to the shared RAN. "

"Multiple PLMN IDs are supported by the serving AMF (i.e. the AMF in the core network of the PLMN representing the hosting operator)."

In TS 32.130[3], Core Network configuration enhancements for Indirect Network Sharing are supposed to be studied.

4	Detailed proposal

1st Change

5 	Use Cases

5.X	Use Case#X: CN configuration management support for Indirect Network Sharing

5.X.1	Description

According to TS 23.501[x], Indirect Network Sharing is supported as follow:

"The communication between the shared RAN and the core network of the participating operator is routed through the core network of the hosting operator that connects to the shared RAN."

To achieve the management of Indirect Network Sharing, first communication between the shared RAN and the core network of each POP is supposed to support configuring, which means that the MOP should support configuring communication routing to POPs and POP should support configuring being routed from the MOP.  Second, AMF of the MOP should allow to configure PLMN ID for each POP, and each POP should allow to configure the inter-plmn discovery.

5.X.2	Potential requirements

REQ-NetShare-CN-Cfg_Req-1: The 3GPP management system of the MOP should have the capability to configure communication to route through the core network for each POP.

REQ-NetShare-CN-Cfg_Req-2: The 3GPP management system of each POP should have the capability to configure communication to route through the core network for the MOP.

REQ-NetShare-CN-Cfg_Req-3: The 3GPP management system of the MOP should have the capability to configure PLMNID of serving AMF individually for each POP.

5.X.3	Potential solutions

.

Following aspects needs to be added or enhanced inin TS 32.130[z]:

- Add descriptions of potential requirements on CN configurations for Indirect Network Sharing.

-	 Add a new workflow that describes the MOP must provide  attribute "PLMNInfoList" and "interPlmnFQDN" of AMFFunction to support configurations in the Indirect Network Sharing scenario.

5.X.4 Evaluation of potential solutions

TBD

End of Change