3GPP TSG-SA5 Meeting #156 	S5-244865

Maastricht , NL, 19 - 23 Aug 2024

# Source:	China Unicom

# Title:	pCR TR 28.876 Add evaluation, conclusion and recommendation for Use case #4

# Document for:	Approval

Agenda Item:	6.19.17

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.876: “Management aspects of RedCap feature”

3	Rationale

It was approved in SP-231734 to study the management of aspects of RedCap features. One of the working taks is to investigate the measurements and KPIs to evaluate the performance of NR networks delivering communication services for RedCap UEs. The corresponding use case, requirement and potential solution on the throughput performance evaluation for RedCap were approved during last two meetings. In order to achieve the study objective, the evaluation, conclusion and recommendation for Use case #4 monitoring of RedCap throughput performance is proposed in this contribution.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

5.4.4 Evaluation of potential solutions

The possibile solution described in clause 5.4.3.1 enhances the existing measurements related to UE throughput in TS 28.552 [10]. Measurements related to RedCap throughput will create a subcounter. The change is lightweight and largely reuses existing measurements.

In summary, the solution described in clause 5.4.3.1 is feasible.

2nd Change

6	Conclusion and Recommendation

6.X	Use case #4: Monitoring of throughput for RedCap service

This use case identifies that existing cell-level measurements realated to throughput may not accurately reflect the performance of network delivering RedCap services, especially in co-existence scenarios. This is because these measurements combine all types of UEs together. To address this issue, the potential solution proposes a new approach to enhance the existing measurements by filtering the RedCap UEs. Detailed description about this solution is shown in clause 5.4.3.

It is recommended to make some enhancements on throughput performance management related to RedCap service according to this solution in the future normative work.

End of changes