3GPP TSG-SA5 Meeting #156 	S5-244872

Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	Ericsson, Deutsche Telekom

# Title:	Discussion paper on generic report header information for MDA

# Document for:	Approval

Agenda Item:	6.4.2

# 1	Decision/action requested

The group is asked to discuss and endorse detailed proposal.

# 2	References

[1]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"

# 3	Rationale

## 3.1	Introduction

The use cases and supported capabilities of an MDA solution that are described in TS 28.104, see reference [1] are associated to each other via the definition of MDAType. An MDAType is defined as string and can have values as shown in Table 3.1.1.

Table 3.1.1: Overview of named MDA Types

| MDA capability | MDA Type |
| --- | --- |
| CoverageAnalytics | CoverageAnalytics.CoverageProblemAnalysis |
|  | CoverageAnalytics.PagingOptimization |
| SLSAnalysis | SLSAnalysis.ServiceExperienceAnalysis |
|  | SLSAnalysis.NetworkSliceThroughputAnalysis |
|  | SLSAnalysis.NetworkSliceTrafficPrediction |
|  | SLSAnalysis.NetworkSliceLoadAnalysis |
| MDAAssistedFaultManagement | MDAAssistedFaultManagement.FailurePrediction |
| MDAAssistedEnergySaving | MDAAssistedEnergySaving.EnergySavingAnalysis |
| MobilityManagementAnalytics | MobilityManagementAnalytics.MobilityPerformanceAnalysis |
| Maintenance | Maintenance.MaintenanceAnalytics |
| ResourceAnalytics | ResourceAnalytics.VirtualizedResourceUtilizationAnalysis |
|  | ResourceAnalytics.PhysicalResourceUtilizationAnalysis |
|  | ResourceAnalytics.ControlPlaneCongestionAnalysis |
| Predictions | Predictions.PMData |

MDAOutputEntry

A single MDARequest may contain a list of MDAType. A single MDAReport references a single MDARequest and the same MDARequest may be referenced by many other MDAReports. One MDAReport reports the output of one MDAType. The information captured by MDAType is therefore not needed to be captured by the individual information elements in the output.

## 3.2	Redundancy in output information elements

As the MDAType is part of the MDA output report and the additional information captured in the output report is per definition only applicable to that MDAType. This means that the names of the information elements do not have to include MDAType information. There where MDAType information is captured in an attribute name, this information would be redundant.

Table 3.2.1 shows the output attribute names without MDAType information in the attribute names. MDAType information has been crossed out.

Table 3.2.1 The output information elements per MDA Type

Editor’s Note: Do not accept change marks, they are shown in this way to avoid having multiple tables (before and after).

| MDA Type (includes MDA Capabilities shown in the left column) |  | Output for a specific MDA Capability |
| --- | --- | --- |
| CoverageAnalytics | CoverageIssueAnalysis | Id IssueType Areas problematicCells recommendedActions radioEnvironmentMap |
|  | PagingOptimization | oOCDuration oOCLocation oOCMap |
| SLSAnalysis | ServiceExperienceAnalysis | Id serviceInformation IssueType affectedObjects Statistics Predictions |
|  | NetworkSliceThroughputAnalysis | Id IssueType UserStatistics TimeStatistics UserPredictions TimePredictions |
|  | NetworkSliceTrafficPrediction | Projections |
|  | E2ELatencyAnalysis | IssueId IssueType affectedObjects |
|  | NetworkSliceLoadAnalysis | IssueId IssueDomain IssuePhase IssueType affectedObjects Distribution |
| MDAAssistedFaultManagement | MDAAssistedFailurePrediction | Object Type eventTime issueID perceivedSeverity recommendedActions |
| MDAAssistedEnergySaving | EnergySavingAnalysis | ProblematicObject IssueType trafficLoadTrends rANenergySavingRecommendations cNenergySavingRecommendations statisticsOfCellsEsState |
| MobilityManagementAnalytics | MobilityPerformanceAnalysis | IssueID IssueRootCause IssueLocation |
|  | HandoverOptimizationAnalysis | hOTarget |
| Maintenance | MaintenanceManagementAnalysis | currentUpgradeOptimal futureUpgradeOptimal gNBID recommendedActions |
| ResourceAnalytics | VirtualResourceUtilizationAnalysis | lowVRUsageNFs highVRUsageNFs predictedVRUsageForNFs recommendedActions |
|  | PhysicalResourceUtilizationAnalysis | lowPRUsageNFs highPRUsageNFs predictedPRUsageForNFs recommendedActions |
|  | ControlPlaneCongestionAnalysis | affectedObjects IssueID recommendedActions |
| Predictions | AnalyticsOutput | pmPredictions |

From table 3.2.1 it can be observed that there is a pattern to the information captured by the MDA output. For example, an MDAType can have multiple issues or problems which can be grouped in multiple types of issues  or problems and the output can identify if an issue or problem is in RAN in the CN or the MDA cannot identify where the issue or problem occurs. In many cases the output includes recommendedActions, predictions/trends and specific MDAType information. A potential set of common information element specific information is shown in Table 3.3.2.

Table 3.2.2 The output information elements split into two groups “header” and “body”

| Grouping | Output applicable to all MDATypes |
| --- | --- |
| Header information: Applicable to most MDATypes | issueId |
|  | issueType |
|  | affectedObjects |
|  | recommendedActions |
| Body information: Applicable to at least one MDAType | problematicCells |
|  | radioEnvironmentMap |
|  | oOCDuration |
|  | oOCLocation |
|  | oOCMap |
|  | serviceInformation |
|  | statistics |
|  | predictions |
|  | userStatistics |
|  | timeStatistics |
|  | userPredictions |
|  | timePredictions |
|  | projections |
|  | issuePhase |
|  | distribution |
|  | eventTime |
|  | perceivedSeverity |
|  | trafficLoadTrends |
|  | statisticsOfCellEsStats |
|  | issueRootCause |
|  | issueLocation |
|  | hOTarget |
|  | currentUpgradeOptimal |
|  | futureUpgradeOptimal |
|  | lowVRUsageNFs |
|  | highVRUsageNFs |
|  | predictedVRUsageForNFs |
|  | lowPRUsageNFs |
|  | highPRUsageNFs |
|  | predictedPRUsageForNFs |
|  | pmPredictions |

Of all the MDATypes the following include header information as shown in Table 3.2.3.

Table 3.2.3 The use of header attributes by the MDATypes.

| MDAType | issueId | issueType | affectedObjects | recommendedActions |
| --- | --- | --- | --- | --- |
| CoverageAnalytics.CoverageProblemAnalysis | X | X | X | X |
| CoverageAnalytics.PagingOptimization | - | - | - | - |
| SLSAnalysis.ServiceExperienceAnalysis | X | X | X | X |
| SLSAnalysis.NetworkSliceThroughputAnalysis | X | X | X | X |
| SLSAnalysis.NetworkSliceTrafficPrediction | - | - | - | - |
| SLSAnalysis.NetworkSliceLoadAnalysis | X | X | X | X |
| MDAAssistedFaultManagement.FailurePrediction | X | X | X | X |
| MDAAssistedEnergySaving.EnergySavingAnalysis | - | X | X | X |
| MobilityManagementAnalytics.MobilityPerformanceAnalysis | X | - | - | - |
| Maintenance.MaintenanceAnalytics | - | - | X | - |
| ResourceAnalytics.VirtualizedResourceUtilizationAnalysis | - | - | X | - |
| ResourceAnalytics.PhysicalResourceUtilizationAnalysis | - | - | X | - |
| ResourceAnalytics.5GCControlPlaneCongestionAnalysis | X | - | X | X |
| Predictions.PMData | - | - | - | - |

## 3.3	Observations, analysis, and conclusions

### 3.3.1	Observations

Observation 1: A group of output attributes could be generalized by removing the MDAType information from the names, this group of attributes is called “header”.

Observation 2: The generalized attributes are identified as issueId, issueType, affectedObjects and recommendedActions as shown in Table 3.2.3. Most of the MDATypes require either all or a partial set of header attributes bar a few exceptions.

Observation 3: Use of common attributes, motivates re-use and significantly reduces the number of attribute that need to be specified (minimum specification level attribute name and type).

### 3.3.2	Analysis

Analysis 1: As some output attributes can be used by multiple MDATypes and the information conveyed by those attributes is agnostic of the MDAType, the attribute name can be common to all MDATypes.

Analysis 2: A common header definition could be defined to be used with all MDATypes.

### 3.3.2	Conclusions

Conclusion 1: The naming of the proposed header attributes can be agnostic of MDA information, making the header re-usable for various MDATypes.

Conclusion 2: The following candidate header attributes can be agnostic of MDAType: issueId, issueType, affectedObjects and recommendedActions.

Conclusion 3:	Use the phrase “issue” where applicable instead of “problem”, an issue may not always be the same as a problem.

## 3.4	Recommendations

Recommendation 1: To introduce a new dataType that represents a common header that is agnostic of what  MDA an output report is used for.

Recommendation 2: void

Recommendation 3: The use of the dataType is defined per MDAType and the use of the attributes in the dataType is optional.

# 4	Detailed proposal

The group is asked to endorse the recommendations in section 3.4