| 3GPP TR 28.858 V0.1.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Concepts and overview	8

5.1	ML model training	8

5.1.1	ML Knowledge Transfer Learning	8

5.1.1.1	Description	8

5.1.1.2	Use cases	9

5.1.1.2.1	Discovering sharable Knowledge	9

5.1.1.2.2	Knowledge sharing and transfer learning	9

5.1.1.3	Potential requirements	11

5.1.1.4	Possible solutions	12

5.1.1.4.1	Evaluation	13

5.1.2 	Pre-training	13

5.1.2.1 	Description	13

5.1.2.2	Use cases	13

5.1.2.2.1	 consumer requested ML pre-training	13

5.1.2.3	Potential requirements	13

5.1.3	ML model training for multiple contexts	13

5.1.3.1	Description	13

5.1.3.2	Use cases	13

5.1.3.2.1	ML model training for multiple contexts	13

5.1.3.3	Potenital Requirements	14

5.1.4	ML training data statistics	14

5.1.4.1	Description	14

5.1.4.2	  Use Cases	14

5.1.4.2.1	 Training data statistical properties for ML training	14

5.1.4.3	  Potential requirements	14

5.1.4.5	  Possible solutions	14

5.1.5	ML model confidence	15

5.1.5.1	Description	15

5.1.5.2	   Use Cases	15

5.1.5.2.1	 Model Confidence Threshold in ML Training	15

5.1.5.3	  Potential requirements	15

5.1.6	Management of Reinforcement Learning	15

5.1.6.1	Description	15

5.1.6.2	Use cases	15

5.1.6.2.1	Exploration in Reinforcement Learning	15

5.1.7 	Sustainable AI/ML	16

5.1.7.1 	Description	16

5.1.7.2	Use cases	16

5.1.7.2.1         AI/ML energy consumption evaluation and reporting for ML model training	16

5.2	ML model testing	16

5.3	AI/ML inference emulation	16

5.3.1	ML inference emulation	16

5.3.1.1	Description	16

5.3.1.2	Use cases	16

5.3.1.2.1	AI/ML inference emulation	16

5.3.1.2.2	Managing ML inference emulation	17

5.3.1.3	Potential requirements	17

5.3.1.4	Possible solutions	18

5.3.1.4.1	Evaluation	19

5.4	AI/ML deployment	19

5.5	AI/ML inference	19

5.5.1	Coordination between the ML capabilities	19

5.5.1.1	Description	19

5.5.1.2	Use cases	20

5.5.1.2.1	 Alignment of the ML capability between 5GC/RAN and 3GPP management system	20

5.5.1.3	Potential requirements	20

5.5.1.4	Possible solutions	20

5.5.1.4.1	Possible solution #1	20

5.5.1.4.2	Evaluation	21

Annex <A>: Change history	21

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and overview

# 5	AI/ML management use cases and requirements

## 5.1	ML model training

### 5.1.1	ML Knowledge Transfer Learning

Editor’s Note: The contention this clause may need to be aligned with the latest version of TS28.105

#### 5.1.1.1	Description

It is known that existing ML capability can be leveraged in producing or improving new or other ML capability. Specifically, using transfer learning knowledge contained in one or more ML entities may be transferred to another ML entity. Transfer learning relies on task and domain similarity to deduce whether some parts of a deployed ML entity can be reused in another domain / task with some modifications. As such, aspects of transfer learning that are appropriate in multi-vendor environments need to be supported in network management systems. However, ML entities are likely to not be multi-vendor objects, i.e. it will in most cases not be possible to transfer an ML entity from function to another. Instead, the knowledge contained in the model should be transferred instead of transferring the ML entity itself. For example, the knowledge contained in an ML entity deployed to perform mobility optimization by day can be leveraged to produce a new ML entity to perform mobility optimization by night. As such and as illustrated by figure 5.1.1.1-1, the network or its management system needs to have the required management services for ML Transfer Learning (MLKLT), where ML Transfer Learning refers to means to allow and support the usage and fulfilment of transfer learning between any two ML entities.

Figure 5.1.1.1-1: ML Knowledge Transfer Learning (MLKLT) flow between the source MLKLT
(which is the entity with the pre-trained ML entity), the peer MLKLT
(which is the entity that shall train a new ML entity) and the MLKLT MnS consumer
(which may be the operator or another management function that wishes to trigger or control MLKLT)

#### 5.1.1.2	Use cases

##### 5.1.1.2.1	Discovering sharable Knowledge

For the transfer learning, it is expected that the source ML Knowledge Transfer Learning MnS producer shares its knowledge with the target ML Training function, either simply as single knowledge transfer instance or through an interactive transfer learning process. The concept of knowledge here represents any experiences or information gathered by the MLEntity in the ML Knowledge Transfer Learning MnS producer through training, inference, updates, or testing. This information or experiences can be in the form of - but not limited to - data statistics or other features of the underlying ML model. It may also be the output of an MLEntity. The 3GPP management systems should provide means for an MnS consumer to discover this potentially shareable knowledge as well as means for the provider of MLKLT to share the knowledge with the MnS consumer.

##### 5.1.1.2.2	Knowledge sharing and transfer learning

The transfer learning may be triggered by a MnS consumer either to fulfil the learning for itself or for it to be accomplished through another ML Training function. The entity containing the knowledge may be an independent managed entity (the ML entity). Alternatively, the ML model may also be an entity that is not independently managed but is an attribute of a managed ML entity or ML function in which case MLKLT does not involve sharing the ML model or parts thereof but may imply implementing the means and services to enable the sharing of knowledge contained within the ML entity or ML-enabled function. The 3GPP management system should provide means and the related services needed to realize the ML transfer learning process.

Specifically, the 3GPP management system should provide means for an MnS consumer to request and receive sharable knowledge as well as means for the provider of MLKLT to share the knowledge with the MnS consumer or any stated target ML Training function. Similarly, the 3GPP management system should provide means for an MnS consumer to manage and control the MLKLT process and the related requests associated with transfer learning between two ML entities or between the two ML entities and a shared knowledge repository.

The two use cases should address the three scenarios represented by figures 5.1.1.2.2-1 to 5.1.1.2-2-4. Note that, the use case and requirements here focus on the required management capabilities. The implementation of the knowledge transfer learning processes are implementation details that are out of the scope of the present document.

Figure 5.1.1.2-2-1: Scenario 1 - Interactions for ML-Knowledge Transfer Learning (MLKLT) to
support training at the ML knowledge Transfer MnS consumer -
the ML knowledge Transfer MnS consumer obtains the ML knowledge
which it then uses for training the new ML entity based on knowledge received
from the MLKLT source MnS producer

Figure 5.1.1.2-2-2: Scenario 2 - interactions for ML-Knowledge Transfer Learning (MLKTL) to
support training at the ML knowledge transfer MnS consumer triggered by the MLKTL Source -
the ML Transfer Learning MnS consumer acting as the MLKTLSource
(the source of the ML knowledge) triggers the training at the ML knowledge Transfer MnS consumer by providing the ML knowledge to be used for the training,
the ML Transfer Learning MnS consumer then undertakes the training

Figure 5.1.1.2-2-3: Scenario 3 - interactions for ML-Knowledge Transfer Learning (MLKLT) to
support training at the Peer ML knowledge Transfer MnS producer who is different from the
ML knowledge Transfer MnS consumer - the ML knowledge Transfer MnS consumer triggers
training at the MLKLT peer MnS producer. The MLKLT MnS consumer then obtains the
ML knowledge from the MLKLT source MnS producer and then uses the knowledge for training
the new ML entity based on knowledge received from the MLKLT source MnS producer

Figure 5.1.1.2-2-4: Scenario 4 - interactions for ML-Knowledge Transfer Learning (MLKLT) to
support training at the Source ML knowledge Transfer MnS producer -
the ML knowledge Transfer MnS consumer triggers training at the MLKLT source MnS producer.
The MLKLT MnS consumer then obtains the ML knowledge from the MLKLT source MnS producer
and then uses the knowledge for training the new ML entity based on knowledge received from
the MLKLT source MnS producer

#### 5.1.1.3	Potential requirements

REQ-MLKLT-1: The 3GPP management system should have a capability enabling an authorized MnS consumer to discover the available shared knowledge from a given MLKLT MnS producer according to a stated set of criteria.

REQ-MLKLT-2: The 3GPP management system should have a capability enabling an authorized MnS consumer to request a MLKLT MnS producer to provide some or all the knowledge available for sharing according to some stated criteria.

REQ-MLKLT-3: The 3GPP management system should have a capability for a MLKLT MnS producer to report to an authorized MnS consumer on the available shared knowledge according to a ReportingCriteria specified in a request for information on available Knowledge.

REQ-MLKLT-4: The 3GPP management system should have a capability enabling an authorized MnS consumer to request a MLKLT MnS producer to trigger and execute a transfer learning instance to a specified ML entity or ML-enabled function. Accordingly, the MLKLT MnS producer receives an instantiation of MLKLT as an MLKLTJob specifying the target MLKLT ML-enabled function or ML entity.

REQ-MLKLT-1: The 3GPP management system should have a capability for an authorized MnS consumer (e.g. an operator or the function/entity that generated the request for available Knowledge or for information thereon) to manage the request for knowledge or its information and subsequent process, e.g. to suspend, re-activate or cancel the MLKnowledgeRequest; or to adjust the description of the desired knowledge.

REQ-MLKLT-1: The 3GPP management system should have a capability for an authorized MnS consumer (e.g. an operator or the function/entity that generated the request for MLKLT) to manage or control a specific MLKLTJob, e.g. to start, suspend or restart the MLKLTJob; or to adjust the transfer learning conditions or characteristics i.e. Modify MLKLTJob attributes.

REQ-MLKLT-1: The 3GPP management system should have a capability enabling an MLEntity to register available knowledge to a shared knowledge repository, e.g. through a MLKnowledgeRegistration process.

REQ-MLKLT-1: The 3GPP management system should have a capability enabling KnowledgeRepo to act as the MLKLT MnS Producer to enable an authorized MnS consumer to request the shared knowledge repository to provide information on the available knowledge according to some given criteria.

REQ-MLKLT-1: The 3GPP management system should have a capability enabling KnowledgeRepo to act as the MLKLT MnS Producer to enable an authorized MnS consumer to request the KnowledgeRepo to provide some or all the knowledge available for sharing according to some given criteria.

REQ-MLKLT-1: The 3GPP management system should have a capability enabling KnowledgeRepo to act as the MLKLT MnS Producer to enable an authorized MnS consumer (e.g. an operator or the function/entity that generated the MLKnowledgeRequest) to manage the request, e.g. to suspend, re-activate or cancel the MLKnowledgeRequest ; or to adjust the description of the desired knowledge.

REQ-MLKLT-1: The 3GPP management system should have a capability enabling KnowledgeRepo to act as the MLKLT MnS Producer to enable an authorized MnS consumer (e.g. an operator) to manage or control a specific MLKLTJob, e.g. to start, suspend or restart the MLKLTJob; or to adjust the transfer learning conditions or characteristics.

#### 5.1.1.4	Possible solutions

Discovering sharable Knowledge

To discover sharable knowledge:

-	The MnS consumer may send a request to the MLKLT MnS producer to provide information on the available sharable knowledge. In other words, the MLKLT MnS producer receives a request to report on the available sharable knowledge.

-	The request may be generic or may state a set of criteria which the knowledge should fulfil.

-	The request may be referred to as MLKnowledgeInfoRequest.

-	The MLKnowledgeInfoRequest must have informational description (Metadata description) of the task and domain related to the required knowledge or given a network problem.

-	An ML entity or a function containing an ML entity may register its available knowledge to a shared knowledge repository, e.g. through a MLKnowledgeRegistration process.

-	The MLKnowledgeRegistration must contain informational description (Metadata description) of the task and domain related to the registered knowledge or suitable network problem.

Knowledge sharing and transfer learning

To share knowledge:

-	Introduce an IOC for an ML Knowledge request. The MnS consumer may send a request to the MLKLT MnS producer to share a specific kind of knowledge. i.e. the MLKLT MnS producer receives a request to provide sharable knowledge, The request may be referred to as MLKnowledgeRequest.

-	Introduce an IOC for an ML transfer learning process or job which is instantiated for any request for transfer learning or ML knowledge transfer. The MLKLT MnS producer instantiates a ML transfer learning process. The process may be referred to as MLKLTJob.

-	The MLKLTJob is responsible for adapting the required knowledge into a shareable format with the MLKLT consumer.

-	MLKLTJob may be a continuous process where knowledge is shared with the MLKLT consumer frequently to account for updates in the knowledge.

NOTE:	It may also be the case that the consumer directly instantiates the MLKLTJob without a separate request.

##### 5.1.1.4.1	Evaluation

The solution described in clause 5.1.1.4 proposes simple information objects that can enable ML functions to exchange their knowledge to be used towards transfer learning but in a way that enables the vendor specific aspects of the ML models not to be exposed. Therefore, the solution described in clause 5.1.1.4 is a feasible solution to be developed further in the normative specifications.

### 5.1.2 	Pre-training

#### 5.1.2.1 	Description

A pre-training is an ML model training for training an ML model with domain related dataset that covers more than one inference type. For example, an ML model could be pre-trained with the dataset from SLS analysis capability group covering inference types including ServiceExperienceAnalysis, NetworkSliceThroughputAnalysis, NetworkSliceTrafficAnalysis, NetworkSliceLoadAnalysis and E2ElatencyAnalysis [see TS 28.104 [3]].

The pre-training is not aimed to support a specific inference type but focuses on commonality in use cases.

ML Model training process defined in TS 28.105 covers pre-training.

#### 5.1.2.2	Use cases

##### 5.1.2.2.1		consumer requested ML pre-training

The pre-training type follows the common procedure of ML training, however the information in training request is different.

#### 5.1.2.3	Potential requirements

REQ-ML_ TRAIN -MLPT-1: The ML training MnS producer shall have a capability to enable an authorized consumer to request a pre-training of an ML model.

Editor note: It is for further discussion if the same requirements in existing TS 28.105 Table 6.2a.1.3-1 related to training function also applies for pre-training.

### 5.1.3	ML model training for multiple contexts

#### 5.1.3.1	Description

ML models can be trained for similar contexts allowing for efficient management of these ML models. If an ML model that is trained for a particular context needs to be updated, it can be realized using an ML model that was trained for different context as the baseline.

#### 5.1.3.2	Use cases

##### 5.1.3.2.1	ML model training for multiple contexts

Although the ML model may provide an AI/ML inference service for multiple scenarios, there are similarities in the contexts where ML models operate and perform AI/ML inferences. For e.g., two ML model instances for the same inference type in urban or rural areas would have significant overlap in their contexts. The context similarity can be leveraged in forming a cluster of ML models, where ML model instances in the cluster are either trained from the same previously trained ML model or from an ML model previously trained for another similar context as the baseline. The training of an ML model for multiple contexts allows for efficiency by cluster training rather than individually training each one of them. ML training needs to support the capability to train the cluster of ML models from the same pre-trained ML model or from an ML model previously trained for another similar context as the baseline. As input to the training, the different contexts and clustering criteria needed to distinguish the ML model instances may be provided by the MnS consumer.

In the case of degradation of ML models, updating of ML models is expected to be triggered. For ML models created through cluster training, the retraining of a degraded ML model could be triggered to start from another member of the cluster, i.e. start from an ML model with another context to create a new ML model with the desired context.

#### 5.1.3.3	Potenital Requirements

REQ-ML_CLUSTER-TRAIN-1: The MLT MnS producer should have a capability for an authorized MnS consumer to request training, update or loading of a cluster of ML models associated to a set of multiple contexts from a previously trained ML model.

### 5.1.4	ML training data statistics

#### 5.1.4.1	Description

During ML training, it is important to ensure that training data is as uniform and representative as possible, and outliers are handled appropriately during the data preprocessing so as to train robust ML entities that do not require frequent re-training.

#### 5.1.4.2			Use Cases

##### 5.1.4.2.1		Training data statistical properties for ML training

Non-uniform distribution of training data can significantly degrade the performance of trained ML entities. The ML model may learn and reflect this non-uniformity, rather than the true underlying patterns in the data. This could lead to inaccurate predictions when the ML model is deployed for inference. Furthermore, an ML model trained on non-uniform data may not generalize well to new unseen data, as the training data might not accurately represent the full range of possible inputs the ML model may encounter during inference. This can limit the usefulness of the ML model.

Similarly, outliers in training data can significantly degrade the performance of trained ML model. ML models learn to make predictions based on the patterns they identify in the training data. Outliers can skew these patterns and lead to an ML model that is biased towards these extreme values, rather than accurately reflecting the majority of the data.

Therefore, it is crucial to ensure that training data is as uniform and representative as possible, and outliers are handled appropriately during the data preprocessing so as to train robust ML models that do not require frequent re-training.

#### 5.1.4.3	Potential requirements

REQ-DATA-STAT-1: The 3GPP management system should enable an authorized consumer to provide information on the training dataset distribution that the ML Training MnS producer should take into account for training an ML model.

REQ-DATA-STAT-2: The 3GPP management system should enable an authorized consumer to provide information on the usage of outliers in the training dataset that the ML Training MnS producer should take into account for training an ML model.

#### 5.1.4.4	Possible solutions

1) Introduce a new attribute of type, e.g., datasetStatisticalProperties, in the MLTrainingRequest IoC requested by an authorized MnS Consumer to the ML Training MnS Producer. This information influences the training data selection mechanisms to be used for training an ML model, assuring that the trained ML model reflects the statistical characteristics of the data that are relevant for the inferenceType. The proposed datasetStatisticalProperties datatype may include the following two attributes:

- UniformlyDistributedTrainingData - It indicates the need for using training data that are uniformly distributed according to the different aspects of the inferenceType. The attribute is of type Boolean. For e.g., in case of coverage problem analytics, data samples for gNBs belonging to each geographical location area needs to be uniformly distributed for the ML model to correctly determine in which geographical location areas the coverage problem occurred during inference.

- TrainingDataWithOrWithoutOutliers – It indicates that the training data samples should consider or disregard data samples that are at the extreme boundaries of the value range. The attribute is of type Boolean. For e.g., in case of coverage problem analytics, if there are only say few training data samples belonging to a particular geographical location, they may be considered as outliers w.r.t. the other training data samples.

### 5.1.5	ML model confidence

#### 5.1.5.1	Description

Confidence is a measure of the probability that a prediction made by the ML model is correct. It is often expressed as a percentage, with 100% being absolute certainty and 0% being absolute uncertainty. Confidence score can be useful in prediction-making processes, where a certain level of confidence might be required before taking action based on the ML model’s prediction. However, it is important to note that a high confidence score does not guarantee the the ML model’s prediction is correct, but it only indicates the ML model belief in its prediction. In simpler terms, confidence is a measure of the ML model’s certainty about its prediction. It is not a measure of the ML model’s accuracy or correctness. An ML model can be very confident about a prediction and still be wrong. Conversely, an ML model can be less confident about a prediction and still be correct.

#### 5.1.5.2		Use Cases

##### 5.1.5.2.1		Model Confidence Threshold in ML Training

In the ML training phase, the ML training function may support the capability to evaluate the confidence value of the trained ML model. The ML model confidence value indicates the average confidence value (in unit of percentage) that the ML model would perform for data with the same distribution as training data. Essentially, this is a measure of degree of the convergence of the trained ML model. Therefore, the MnS consumer may prefer to indicate the confidence threshold value that the trained ML model should satisfy before the ML model is deployed for inference.

#### 5.1.5.3			Potential requirements

REQ-MODEL_CONF-1: The AI/ML training MnS producer should have a capability allowing an authorized MnS consumer to indicate the confidence threshold that the AI/ML MnS producer should meet on the trained ML model.

### 5.1.6	Management of Reinforcement Learning

#### 5.1.6.1	Description

Reinforcement Learning (RL) is a type of machine learning where an agent learns to make decisions by taking actions in an environment to achieve a goal. The agent learns from the consequences of its actions, rather than from being explicitly taught and it selects its actions based on its past experiences (exploitation) and also by new choices (exploration), which is essentially a trial-and-error approach. In RL, an agent interacts with the environment in discrete time steps. At each time step, the agent receives the current state of the environment and selects an action. The environment, in turn, provides a reward and the new state. The goal of the agent is to learn a policy, which tells it what action to take under what circumstances, that maximizes the sum of rewards. The main advantage of this approach is in the ability to automatically adapt to the characteristics of the environment, making it suitable approach for handling dynamic environments such as mobile networks.

#### 5.1.6.2	Use cases

##### 5.1.6.2.1	Exploration in Reinforcement Learning

Reinforcement Learning (RL) has the ability to learn and adapt itself to dynamic environments and thus finds the near optimal solution to the problem. This makes the RL-based approaches very interesting for applications in the mobile networks. However, the potential negative impact to the mobile network caused by RL is still the main drawback. In particular, during the exploration step performing trials and learning from errors may have an impact on the operational network and may result in unsafe operations causing network performance degradations. Therefore, the exploration step in RL needs to be under a controlled environment that are not supposed to violate system performance requirements. If the RL agent behaves in an unexpected manner, there needs to be a set of fall-back actions in place, e.g., to switch from RL-based solution to non-RL-based solution.

### 5.1.7 	Sustainable AI/ML

#### 5.1.7.1 	Description

Sustainable AI/ML aims to reduce energy usage in the ML model training step. It is important to investigate effective methods for evaluating AI/ML energy consumption and efficiency for ML model training which will aid in decreasing energy consumption, enhancing energy efficiency, and promoting sustainable development of AI/ML in 5G systems.

#### 5.1.7.2	Use cases

##### 5.1.7.2.1         AI/ML energy consumption evaluation and reporting for ML model training

AI/ML energy consumption is a significant concern, especially as ML models become more complex and data-intensive. The ML model training is typically the most energy-intensive part since it involves processing large datasets and performing numerous calculations to adjust the ML model parameters.

The MnS consumer may have concern on the energy consumption for ML model training and the MnS Producer should report the related information. MnS Consumer may ask the MnS Producer to report the energy consumption of training each ML model. To satisfy such query request from the MnS consumer, methods should be investigated to measure AIML energy consumption. Then, MnS Consumer can use the energy consumption information and compare with the network performance gain due to ML model to evaluate the total benefit of using the Ml model.

Note: the granularity for energy consumption of ML model training is FFS.

## 5.2	ML model testing

## 5.3	AI/ML inference emulation

### 5.3.1	ML inference emulation

Editor’s Note: The contention this clause may need to be aligned with the latest version of TS28.105

#### 5.3.1.1	Description

A trained ML entity can be used for inference within the stated scope e.g. on a managed function or in a management function. Accordingly, there may be an AI/ML inference MnS producer that is responsible for executing the inference.

#### 5.3.1.2	Use cases

##### 5.3.1.2.1	AI/ML inference emulation

After an ML entity is trained, validation is done to ensure the training process is completed successfully. Typically, validation is done by preserving part of the training data set and using it after training to check whether the ML entity has been trained correctly or not. However, even after the ML entity is validated during development, inference emulation is necessary to check if the ML entity containing the ML entity is working correctly under certain runtime context or using certain inference emulation data set. In principle, the two operations are similar on a functional level, where both of them check the ML performance against given context or data to ensure the ML functionality is functioning correctly. But inference emulation involves interaction with third parties, e.g. the operators who use the ML entity or third-party systems that may rely on the results computed by the ML entity. For these reasons, it is necessary to support inference emulation, specifically to support means:

-	For a given MnS consumer to request for a specific AI/ML capability to be executed in ML inference emulator environment.

-	For a given MnS consumer to request a specific ML inference emulator to execute a given AI/ML capability.

-	For a managed function to act as a ML inference emulator and execute AI/ML capabilities in a controlled way.

The network or its management system needs to have the capabilities and provide the services needed to enable the MnS consumer to request inference emulation and receive feedback on the inference emulation of a specific ML entity or of an application or function that contains an ML entity.

##### 5.3.1.2.2	Managing ML inference emulation

The 3GPP management system may have resources for multiple emulation environments to be used depending on need. These may include simulation environments, a digital twin of the network, a test network or the real network under curtain constrained conditions, e.g. for a selected set of UEs. The multiple emulation environments may represent different levels of trust that the operator or management system has in the ML entity or AI/ML inference functions. Correspondingly, 3GPP management system needs to have means and method for Orchestrating the inference emulation i.e. say called the inference emulation orchestrator or inference emulation function. Accordingly:

-	the emulation progression process involves choosing the right type and instance of an emulation environment to which an ML entity, AI/ML inference function or the action thereof may be tested depending on the needs of the function to be tested and the available emulation environments and their resources;

-	the emulation process may also involve executing the ML entity, AI/ML inference function or its action on the real network but in a controlled fashion, e.g. only within certain hours or only on cells with a particular kind of load or only on cells in a particular area or in limited subscriber groups.

Relatedly, the actions taken by the inference emulation function may include:

-	Controlling the allowed parameter space/ranges of the parameters optimized by the ML entity or AI/ML inference function depending on the emulation environment to which the ML entity, AI/ML inference function or the actions are being executed.

-	Adjusting the parameter space in consideration of the observed behaviour of the ML entity or AI/ML inference function.

-	Deploying the actions of the ML entity or AI/ML inference function on a selected emulation environment or on the real network.

-	Blocking the ML entity or AI/ML inference function from being used on the network.

#### 5.3.1.3	Potential requirements

REQ-AI/ML_EMUL-1: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to query the available emulation environment(s).

REQ-AI/ML_EMUL-2: The MnS producer for AI/ML inference emulation should have a capability to inform an authorized MnS consumer of the available emulation environment(s).

REQ-AI/ML_EMUL-3: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to request an ML inference emulation for a specific ML entity or entities.

REQ-AI/ML_EMUL-4: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to request for ML Inference Emulation for a specific ML entity using specified data or data with specifically stated characteristics and inference emulation features.

REQ-AI/ML_EMUL-5: The MnS producer for AI/ML inference emulation should have a capability to inform authorized MnS consumer about the status of the emulation of an ML entity under emulation.

REQ-AI/ML_EMUL-6: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer (e.g. an operator) to manage or control a specific ML inference emulation process, e.g. to start, suspend or restart the inference emulation; or to adjust the inference emulation conditions or characteristics.

REQ-AI/ML_EMUL-7: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to request reporting, and receive reports on the progress and outcome of an emulation process.

REQ-AI/ML_EMUL-8: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to configure an ML entity or AI/ML inference function supporting with the level of trust that expresses the degree to which the ML entity or AI/ML inference function or the different action thereof have been confirmed as trusted.

REQ-AI/ML_EMUL-9: The MnS producer for AI/ML inference emulation should have a capability to graduate an ML entity, AI/ML inference function or the different action thereof through different levels of trust each expressing a different degree to which the ML entity, AI/ML inference function or action has been confirmed as trusted.

#### 5.3.1.4	Possible solutions

1)	Introduce an IOC with the properties of the ML inference emulation function. This may be termed as an MLInferenceEmulationFunction to be name-contained in either a Subnetwork, a ManagedFunction or a ManagementFunction. The MLInferenceEmulationFunction may be a separate function or may be name-contained in an inference function.

This IOC contains attributes including the following:

-	list of the ML entities which can be emulated and possibly in which emulation environments;

-	indication of progression of the MLEntity or AI/ML inference function that is under emulation to indicate the degree to which the inference has been emulated and trusted;

-	different characteristics for which different emulations may be supported;

-	a hierarchy for different emulation environments, e.g. an emulator that uses only a test network vs. an emulator that activates the actions in the real network at specified time such as the maintenance window.

2)	Introduce an IOC representing an available emulation environment, e.g. a new IOC named as AvailableEmulationEnvironment, or EmulationSubNetwork, or the existing SubNetwork IOC with an attribute indicating it is a subnetwork used for emulation.

The instance of this IOC is created by the MnS producer to allow the consumer to query, or be informed of, the information of the available emulation environments. This IOC contains the following properties (e.g. the subordinated IOC or attributes):

-	inference functions or ML entities under emulation.

3)	Introduce an IOC for the request for ML inference emulation which shall capture the consumer's requirements for inference emulation. This may be named as an MLInferenceEmulationRequest to be name-contained by the MLInferenceEmulationFunction.

The MLInferenceEmulationRequest shall be associated with at least 1 MLEntity for which the inference emulation is being executed.

This IOC contains attributes including the following:

-	identifier of the ML entities requested for emulation;

-	identifier of the selected emulation environment;

-	time window for the emulation.

Each MLInferenceEmulationRequest may have a RequestStatus field that is used to track the status of the specific MLInferenceEmulationRequest or the associated MLInferenceEmulationProcess. The RequestStatus is an enumeration with the possible values as: "Pending" if no action has been taken for the request; "Triggered" when an MLInferenceEmulationProcess has been instantiated; "Suspended" when the request or its job has been suspended by MnS consumer or producer and "Served" when the job has run to completion.

4)	Introduce an IOC for the process of ML inference emulation from the objects for inference emulation shall be instantiated. This may be named as an MLInferenceEmulationProcess to be name-contained by the MLInferenceEmulationfunction.

The MLInferenceEmulationProcess shall be associated with at least one MLEntity for which the inference emulation is being executed.

This IOC contains the following attributes:

-	progress indicator;

-	identifier of the corresponding emulation request.

5)	Introduce a report on ML inference emulation, which may provide reporting on ML emulation for one or more MLInferenceEmulationRequests or MLInferenceEmulationProcesses. This report may be equivalent to the inference report, and:

-	the ML inference emulation report may indicate the emulation environments in which the entity has been emulated and passed;

-	the ML inference emulation report may also include the specific performance metrics for that emulation environments.

May also introduce the IOCs and datatypes for request, process and report on ML Inference and then or update these IOCs with the features of the ML Inference Emulation as introduced above.

6)	The IOCs, attributes and performance measurements for the solutions of performance evaluation for AI/ML inference for inference phase, as described in clause 5.2.6.4, which can be reused for monitoring the inference performance of the ML entities during the emulation.

7)	The IOCs and attributes for configuration management of 5G system, as defined in 3GPP TS 28.541 [20], which can be reused for configuring the emulation environment.

##### 5.3.1.4.1	Evaluation

The solution described in clause 5.2.5.4 reuses the existing provisioning MnS operations and notifications in combination with extensions of the NRM. Requests for inference emulation for a given ML entity may be instantiated using provisioning management service implemented via CRUD (Create, Read, Update, Delete) operations on the request objects. The solution provides the flexibility to allow any function that can execute n ML entity to be the MnS producer for ML inference emulation, e.g. an inference function or a generic sandbox function.

Therefore, the solution described in clause 5.2.5.4 is a feasible solution to be developed further in the normative specifications.

## 5.4	AI/ML deployment

## 5.5	AI/ML inference

### 5.5.1	Coordination between the ML capabilities

Editor’s Note: The contention this clause may need to be aligned with the latest version of TS28.105

#### 5.5.1.1	Description

For ML in 5GC or RAN, the ML capabilities in 5GC or RAN may be needed to coordinate with 3GPP management analytics and possibly other aspects in order to improve the overall performance.

Typically, due to the type of the collected data used for model training/inference for 5GC or RAN, the performance of a model in 5GC or RAN may be biased in some respects. On the other hand, the 3GPP management system collects data of longer range and from a wide scope of RAN nodes/5GC, which consequently implies that the predictions calculated by the management system will be unbiased towards the overall RAN nodes/5GC. However, 3GPP management system predictions may lack insight of patterns related to specific node behaviour or finer granularity of time. Hence with coordination or alignment of the ML capability among 5GC/RAN and 3GPP management system, the overall performance may be improved.

To enable the coordination between the ML capabilities, the configuration (e.g. a triggering condition, i.e. when a result is needed from the RAN analytics to MDA) may be needed. On the other hand, the result of the coordination may be communicated towards the consumer(s) and hence there is a need to enhance the reporting in order to capture the deviation of predictions (and/or the related context) so the consumer can gain a better understanding regarding the coordination of ML capabilities.

#### 5.5.1.2	Use cases

##### 5.5.1.2.1		Alignment of the ML capability between 5GC/RAN and 3GPP management system

Generally, the typical data from 5GS data is measurements (PM, KPI), which may be modeled as "time series data" and the analytics of "time series data" is normally to learn the seasonality, trend, etc., patterns. Different types of seasonality patterns exist, e.g. daily, weekly, monthly, seasonally, annually, etc. A RAN node collects finer granularity data for short duration. The finer seasonality pattern will be well captured in a timely manner, while the 3GPP management system with longer range of data (which is more aggregated) will more accurately capture the higher level of seasonality patterns. Hence, combing the analytics results from RAN, 5G core and 3GPP management system or between RAN and 5G core may improve the accuracy for overall predictions.

On another matter, the data collected from one RAN or 5G core node would tend to be biased for that specific RAN node or NF, which implies that the prediction with the data learned and inferred will tend to be biased for that specific node. On the other hand, the 3GPP management system collects data of longer range and from a large amount of different RAN nodes or NFs, which consequently implies that the prediction will be unbiased towards the overall RAN nodes or 5G core area. Hence combining the results from both the RAN or 5G core and 3GPP management system, or between NWDAF and RAN may also improve the overall predictions accuracy (and mitigate the bias).

#### 5.5.1.3	Potential requirements

REQ-AIML_COORD-01: 3GPP management system should have the capability to allow an authorized consumer to configure an ML capability regarding the correlation of predictions and statistics between MDAS and RAN function, or MDAS and NWDAF.

REQ-AIML_COORD-02: 3GPP management system should have the capability to report the result of correlation of predictions and statistics between MDAS and RAN function, or MDAS and NWDAF.

#### 5.5.1.4	Possible solutions

##### 5.5.1.4.1	Possible solution #1

1)	Introduce the information Elements (e.g. instance of IOC or a dataType) for interaction between ML MnS producer and consumer (e.g. the RAN analytics or NWDAF, or entity consuming the RAN analytics or NWDAF) to support coordination of the ML capability between 5GC/RAN and 3GPP management system: MLCapabilityCoordinationRequest and a response informant element MLCapabilityCoordinationResponse. This information element may represent the triggering configuration (or a triggering policy) for predictions coordination from two ML capabilities. This information Element may allow an MOI (or MOI using this information element) to be created on the ML (inference) MnS Producer and may contain the following attributes:

-	The analytics deviation indicator, such as a threshold (determining that the prediction calculated by a data analytics function exceeds a configurable certain value, corresponding to the prediction available at a different data analytics function, by a "threshold").

-	The requested analytics (analytics type name, list of analytics values or output, time intervals, confidence degree, etc.).

-	The target objects, e.g. gNBs, and the related characteristics.

-	Area of interest, geographical area or TA.

2)	MLCapabilityCoordinationResponse- this information element may represent the response indicating the analytics or data obtained according to the MLCapabilityCoordinationRequest. This information Element may be created by the ML MnS (inference) producer towards the MnS consumer and includes output analytics which can be statistics or predictions. The MLCapabilityCoordinationResponse and MLCapabilityCoordinationRequest may also be data attribute in analytic request and response or analytics report.

Figure 5.x.5.1-1: Interaction between ML MnS producer and consumer
to support coordination of the ML capability

##### 5.5.1.4.2	Evaluation

The solution described in clause 5.x.5.1 proposes simple information elements and procedure that may enable MnS Producer to trigger configuration to be used for analytic coordination. This NRM based solution reuses the existing provisioning MnS Operations and notifications for control and reporting. Therefore, the solution described in clause 5.x.5.1 is a feasible solution to be developed further in the normative specifications.

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | SA5#156 | S5-243521 |  |  |  | Initial skeleton | 0.0.0 |
| 2024-08 | SA5#154 | S5-242150 |  |  |  | Rel-19 pCR TR28.9xx R19 Study usecases for continuation from 28.908 | 0.1.0 |
| 2024-08 | SA5#155 | S5-243096 |  |  |  | Rel-19 Add use case of ML pre-training | 0.1.0 |
| 2024-08 | SA5#155 | S5-243118 |  |  |  | Rel-19 pCR TR28.9xx Clusters for Joint Training of ML models | 0.1.0 |
| 2024-08 | SA5#155 | S5-243115 |  |  |  | Rel-19 pCR TR28.9xx Training Data Statistical Properties | 0.1.0 |
| 2024-08 | SA5#155 | S5-243116 |  |  |  | Rel-19 pCR TR 28.9xx Confidence Threshold | 0.1.0 |
| 2024-08 | SA5#155 | S5-243314 |  |  |  | Rel-19 pCR TR28.9xx Management of Reinforcement Learning | 0.1.0 |
| 2024-08 | SA5#155 | S5-243434 |  |  |  | Rel-19 pCR TR 28.9xx AI/ML energy consumption and energy efficiency | 0.1.0 |