3GPP TSG-SA5 Meeting #156 	S5-243555

Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	Ericsson

# Title:	pCR TR 28.871 Alarm definition

# Document for:	Approval

Agenda Item:	6.19.8

# 1	Decision/action requested

Approve the proposal.

# 2	References

[1]	3GPP TR 28.871 Study on Service Based Management Architecture enhancement phase 3 (Release 19) v0.2.0

[2]	S5-238120	Discussion paper on Alarm definition

# 3	Rationale

The Study on Service Based Management Architecture enhancement phase 3 [1] includes the work task:

WT-3.1	Study whether a new 3GPP alarm type in Alarm definition is needed.

This document specifies the use-cases and potential requirements and solution for this topic. The main goals include:

- Making it possible to list alarming conditions supported by the producer, each alarming condition is defined as an alarm definition. The set of alarming conditions can change with configuration, licensing, etc.

- Supplying supporting data for each alarming conditions in the form of an Alarm Definition

- Allowing the consumer to set the perceivedSeverity associated with each alarming condition

- Allowing alarm filtering based on the 3 parameters defining the alarming-condition

# 4	Detailed proposal

First change

## 5.X	Use case #<X>: List and handle Alarming-Conditions

### 5.X.1	Description

Observation 1: Alarm capability

Currently, there is no mechanism defined that a Fault MnS consumer can use to retrieve the set of all alarming-conditions that a network resource could notify about.

The concept of “Alarm definition” specifying an alarming-condition should have a representation in an NRM. This would provide a readable list of alarm notifications that this network resource can generate.

Observation 2: Context-sensitive alarm capability

The set of alarming-conditions that a particular network resource can notify about, i.e., its set of supported “Alarm definitions”, could change depending on the current state of that network resource, e.g., configuration, software composition, licences.

For example, a particular gNB could provide gNB CU-CP, gNB CU-UP and gNB DU functionality, but may currently be providing gNB DU functionality only. In this case, the gNB will only send notifications about alarmed conditions that are relevant for the gNB DU.

Likewise, a gNB DU may support optional features, such as Carrier Aggregation, which may be configured or not at any given time. The gNB DU will then only send notifications about alarmed conditions that are relevant for the features currently configured.

Observation 3: Overriding vendor defined perceived severity

Some alarming-conditions might have multiple associated perceivedSeverity values. E.g. an alarm based on temperature may be considered MAJOR over 90 ℃ but over 100 ℃ it becomes CRITICAL.

Most other alarming-conditions have a single associated severity e.g. "Power Problem" is always MAJOR. Equipment vendors usually hardcode the perceivedSeverity for such alarms. However, network operators often have a different view of the severity of specific alarming-conditions. E.g. an operator that invested in very robust, redundant power supply might categorize "Power Problem" as MINOR.

It should be possible for the operator to change the severity assigned to specific alarming-conditions. The operator may modify the perceivedSeverity in the AlarmDefinition. After this change all new alarms raised for that alarming-condition should be raised with the new, configured perceivedSeverity. Alarms already in the AlarmList should not be affected.

### 5.X.2	Potential requirements

| Requirement label | Description | Related use case(s)/Motivation |
| --- | --- | --- |
| REQ-MS-FMAL-1 | A producer should be able to provide a list of all the alarming-conditions it might raise an alarm for. |  |
| REQ-MS- FMAL-2 | The list of alarming-conditions should be adjusted according to run-time conditions, configuration etc. | The list shall reflect any conditions that affect potential alarming-conditions e.g. licensing SW updates, etc. |
| REQ-MS- FMAL-3 | Every alarming-condition should have a set of associated data that describes the alarming-condition. The information set should include: alarmType, probableCause, specificProblem The information set may include description, perceivedSeverity | Additional information helps the user correctly handle an alarm |
| REQ-MS- FMAL-4 | The producer should make it possible for the consumer to set the perceivedSeverity the producer should use for alarming-condition. | Most alarming-conditions have single perceivedSeverity, but some may have multiple perceivedSeverity values depending on e.g. thresholds. For the latter type perceived severity may be immutable. |
| REQ-MS- FMAL-5 | It should be possible to filter alarm related notifications based on the alarming-condition. | Motivation: Management systems may not want to handle all alarming-conditions. Operators might handle high and lower priority alarming-conditions in separate subscriptions according to separate operational processes. |

### 5.X.3	Potential solutions

#### 5.X.3.i	Potential solution #1: AlarmDefinition IOC

##### 5.X.3.i.1	Introduction

An AlarmDefinition is an identified alarming-condition together with a set of associated information. Alarm definitions shall be part of the NRM, allowing the consumer to list the alarming-conditions supported by the producer.

As specified in TS 28.111: alarmType, probableCause and specificProblem (if present) identify an alarming condition.

The AlarmDefinition MOIs should be updated whenever the run-time conditions (e.g. licensing, SW-updates) change the alarming-conditions that may be notified.

The consumer may set the perceivedSeverity attribute of an AlarmDefinition. After this change all new alarms raised for that alarming-condition should be raised with the newly configured perceivedSeverity. The change shall have no effect on alarms already raised.

##### 5.X.3.i.2	AlarmDefinition IOC

An IOC listing all AlarmDefinition shall be defined. The IOC shall contain the following attributes:

| Attribute name | S | isReadable | isWritable | Comment |
| --- | --- | --- | --- | --- |
| alarmType | M | T | F |  |
| probableCause | M | T | F |  |
| specificProblem | M | T | F |  |
| description | M | T | F |  |
| perceivedSeverity | O | T | T |  |

Additional attributes may be defined later.

##### 5.X.3.i.2	Alarm notifications shall use all "alarm identifying attributes"

To allow filtering on alarming conditions all alarm related notifications shall carry the parameters alarmType, probableCause and specificProblem. SpecificProblem shall be added to the input parameters of notifyClearedAlarm, notifyChangedAlarm, notifyAckStateChanged, notifyComments. All 3 parameters shall be added to notifyCorrelatedNotificationChanged.

### 5.X.4	Evaluation of potential solutions

The proposed solutions satisfiy the requirements.

End of change