3GPP TSG-SA5 Meeting #156 	S5-243621

Maastricht, Netherlands, 19 – 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.405 | CR | 0034 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.405 MDT alignment procedure enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As defined in TS 38.300, section 21.5, the UE configured with QoE measurements sends an indication to inform the gNB about the start or the stop of a QoE measurement session of configured QoE measurements. The gNB can activate the MDT measurements that are to be aligned with the QoE measurements performed by the UE upon/after receiving the QoE measurement session start indication from the UE. The gNB may activate the MDT measurements upon/after receiving the MDT activation message from the OAM. The gNB can deactivate the aligned MDT measurements according to an OAM command which may, e.g., be triggered by the session stop indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the missing steps in both signalling based QoE activation and management based QoE activation procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligment with RAN3 specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.1, 4.6.1.1, 4.6.1.2, 4.6.2.1, A.1, A.2, A.3, A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 4.5.1	Activation of measurement collection job and reporting of collected information in NR

Management Based Activation enables collection of application layer measurements from the UEs in the specified area for specified end user service type.

The parameters for the network request session are sent from the MnS Consumer via the management system to the gNBs that host the cells that are included in the QoE Measurement Collection job request in the Create MOI QMC operation [15]. A QMC Job is activated by creating a QMCJob object instance in the MnS producer.

Figure 4.5.1-1 and the text below describe the activation of QoE measurement collection.

Figure 4.5.1-1: QMC activation example for Management Based Activation and Reporting in NR

1)	A QoE Measurement Collection Job begins when the MnS Consumer sends createMOI request for QMCJob to the MnS Producer serving the impacted gNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, qoEReference, mDTAlignmentInformation, availableRANqoEMetrics and qMCConfigFile, and mBSCommunicationServiceType. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

NOTE: The RAN visible QoE measurements are supported for the DASH streaming and VR services.

2)	Application level measurement configuration. The Application in the UE shall send AT command +CAPPLEVMCNR [5] containing the parameter <n> set to 1 to the Access Stratum. This enables the the Access Stratum to present an unsolicited result code to the Application at a later time. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

3)	UE registration procedure. The UE shall register QMC capability with the gNB by setting qoe-Streaming-MeasReport to “supported” in UE-NR-Capability [11]. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

4)	The gNB starts a network request session, with the QoE Reference [3] given in createMOI for QMCJob [3]. For the duration of the network request session, the gNB(s) checks for connections where the UE has qoe-Streaming-MeasReport set to “supported” [11] in step 2.

5)	If a UE has the wanted capability, the gNB stores the associated qoECollectionEntityAddress and starts a UE request session by sending RRCReconfiguration to the UE including serviceType, measConfigAppLayerId (corresponding to the value of qoEReference from step 4), measConfigAppLayerContainer, ran-VisibleParameters and availableRANqoEMetrics.

If QoE measurement configuration pertains to MBS communication service, the gNB translates the qoECollectionEntityAddress into qoECollectionEntityIdentity and includes qoECollectionEntityIdentity in the RRCReconfiguration.

NOTE: The IE measConfigAppLayerId indicates the identity of the application layer measurement configuration, see [11].

6)	The access stratum in the UE sends an unsolicited response to the Application including app-meas_service_type, meas_config_app_layer_id and app-meas_config_file. These IEs map directly to the IEs in step 5. The unsolicited response is for the AT command +CAPPLEVMCNR [5] which is sent from Application in step 2.

7)	The QMC Job remains inactive until the Application for the specified serviceType initiates service.

8)	When the Application begins service, it sends the AT command +CAPPLEVMRNR [5], including meas_config_app_layer_id and qoe_measurement_status to indicate a recording session has started, to the access stratum.

9)	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, and qoECollectionEntityIdentity to the gNB to indicate a recording session has started. If mDTAlignmentInformation is received in step 1, the gNB may activate the MDT measurements that are to be aligned with the QoE measurements performed by the UE.

10)	While active, the Application collects measurement reports. When the QMC is completed or at the end of period for periodic report, the recorded information is formatted in a QMC report, see [6] for DASH, [7] for MTSI or [13] for VR.

11)	When a formatted measurement report is available, the Application sends the AT command +CAPPLEVMRNR [5] including meas_config_app_layer_id, qoe_measurement_status, and app-meas_report to the Access Stratum. If the QMC Job has ended, qoe_measurement_status is set to indicate the Job has ended.

12)	When the AT command +CAPPLEVMRNR is received from the Application, the Access Stratum sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, measReportAppLayerContainer, and qoECollectionEntityIdentity to the gNB.

13)	The gNB translates the qoECollectionEntityIdentity into qoECollectionEntityAddress if it is needed, and sends the QMCRecord to the MCE associated to the qoECollectionEntityAddress. The report contains the QoE Reference from step 4, a time stamp, and measReportAppLayerContainer from step 12.

*** START OF NEXT CHANGE ***

#### 4.6.1.1	Activation of QoE measurement task after completion of UE registration procedure

Figure 4.6.1.1-1 and the text below describe the activation of QoE measurement collection after completion of UE registration procedure.

Figure 4.6.1.1-1: QMC activation and reporting example in NR after UE is registered

1.	The MnS Consumer sends createMOI request for QMCJob to UDM that controls the impacted gNB(s), and includes the parameters: serviceType, sliceScope,  areaScope, qoECollectionEntityAddress, qoETarget, qoEReference, mDTAlignmentInformation, availableRANqoEMetrics, qMCConfigFile, and mBSCommunicationServiceType.

2.	The UDM inserts subscriber related data and forwards it to the AMF.

3.	The AMF forwards the configuration parameters serviceType, areaScope, sliceSupportListQMC, measCollEntityIPAddress, qoEReference, nGRANTraceId, availablerANqoEMetrics and containerForAppLayerMeasConfig in message UE Context Modification Request to the impacted gNB.

4.	The gNB checks if the UE capability matches the criteria for serviceType in the QoE measurement configuration information.

5.	If the UE has the wanted UE capability, the gNB starts a UE request session and stores the associated qoECollectionEntityAddress, sends the message RRCReconfiguration to the UE including serviceType, measConfigAppLayerId, transmissionOfSessionStartStop ,ran-VisibleParameters, measConfigAppLayerContainer, and availableRANqoEMetrics.

If QoE measurement configuration pertains to MBS communication service, the gNB translates the qoECollectionEntityAddress into qoECollectionEntityIdentity and includes qoECollectionEntityIdentity in the RRCReconfiguration.

NOTE:	The IE measConfigAppLayerId indicates the identity of the application layer measurement configuration, see [11].

6.	The access stratum in the UE sends an unsolicited response to the application level including app-meas_service_type, meas_config_app_layer_id, transmission_of_session_start-end, ran_visible_periodicity, number_of_buffer_level_entries, report_playout_delay_for_media_startup, and app-meas_config_file. The unsolicited response is for the AT command +CAPPLEVMCNR which is sent from UE Application Level to UE Access Stratum during Registration procedure.

7.	When the application for the specified serviceType starts, the QMC is initiated. To specify the session which is started, the application generates a recordingSessionId.

8.	The application layer sends the AT command +CAPPLEVMRNR [7] including meas_config_app_layer_id and qoe_measurement_status that indicates that a session is started to the access stratum.

9.	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId,  appLayerSessionStatus and qoECollectionEntityIdentity to the gNB. If mDTAlignmentInformation is received in step 1, the gNB may activate the MDT measurements that are to be aligned with the QoE measurements performed by the UE.

10.	When the QMC is completed or at the end of period for periodic report, the recorded information is collected in a QMC report, see [6], [7] or [13].

11.	The application layer sends the AT command +CAPPLEVMRNR [7] including meas_config_app_layer_id, playout_delay_for_media_startup, number_of_buffer_level_entries, qoe_measurement_status indicating that session has ended and app-meas_report including recordingSessionId to the access stratum.

12.	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, ran-VisibleMeasurements and measReportAppLayerContainer including recordingSessionId and qoECollectionEntityIdentity to the gNB.

13.	The gNB translates the qoECollectionEntityIdentity to the qoECollectionEntityAddress, if it is needed, and sends the QMC report to the MCE associated to the qoEReference. The report contains the qoEReference, a time stamp, and the RAN transparent container including the recordingSessionId and slice, which contains only the S-NSSAI used. Note that the qoEReference is mapped to the measConfigAppLayerId at gNB on the previous step and is included in QMC report.

*** START OF NEXT CHANGE ***

#### 4.6.1.2	Activation of QoE measurement task before UE Registration procedure to the network

Figure 4.6.1.2-1 and the text below describe the activation of QoE measurement collection before UE registration procedure to the network.

The AMF receives and stores QoE measurement collection job as part of API: Nudm_SDM. Then the same procedure for activation of measurement collection job is applied as after completion of UE registration procedure, except that instead of UE Context Modification Request the message Initial Context Setup Request is sent, see 4.6.1.1.

Figure 4.6.1.2-1: QMC activation and reporting example in NR before UE Registration procedure to the network

*** START OF NEXT CHANGE ***

#### 4.6.2.1	NG Based Handover for Signalling Based Activation

The figure 4.6.2.1-1 and the text below describe the handling at NG Based handover between gNBs for Signalling Based Activation case.

Figure 4.6.2.1-1: Handling of QMC activation example in case of NG Based handover in NR for Signalling Based Activation

1. Source gNB sends the message HANDOVER REQUIRED to AMF. The message includes the Source to Target Transparent Container which contains qoEReference, serviceType, measConfigAppLayerId, areaScope, measCollEntityIPAddress, qoEMeasurementStatus, sliceSupportListQMC, nGRANTraceId, availableRANVisibleqoEMetrics, and containerForAppLayerMeasConfig.

2. AMF sends the message HANDOVER REQUEST to the Target gNB. The message includes the Source to Target Transparent Container which contains the same parameters as in step 1.

3. The target gNB checks if the measurements will be continued or not.

4. The target gNB sends the message HANDOVER REQUEST ACKNOWLEDGE to the AMF.

5. The AMF sends the message HANDOVER COMMAND to the source gNB.

6. The source gNB sends the message RRCReconfiguration to the UE.

7. The UE sends the message RRCReconfigurationComplete to the gNB.

8. When the ongoing QMC is completed or end of periodic reporting is reached, the recorded information is collected in a QMC report, see [6], [7] or [13], at UE Application Level.

9. The application layer sends the AT command +CAPPLEVMRNR including meas_config_app_layer_id, qoe_measurement_status playout_delay_for_media_startup, number_of_buffer_level_entries, report_initial_playout_delay, and the app-meas_report to the access stratum.

10. The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, measReportAppLayerContainer, and qoECollectionEntityIdentity to the gNB.

11. The gNB translates the qoECollectionEntityIdentity to the qoECollectionEntityAddress, if it is needed, and sends the QMC report to the MCE associated to the qoEReference. The report contains the qoEReference, a time stamp, and the RAN transparent container including the recordingSessionId and slice (which contains only S-NSSAI that is used). Note that the qoEReference is mapped to the measConfigAppLayerId at gNB on the previous step and is included in QMC report.

*** START OF NEXT CHANGE ***

## A.1	QMC activation and reporting example in NR after UE is registered

The following PlantUML source code is used to describe QMC activation and reporting example in NR after UE is registered. As depicted by Figure 4.6.1.1-1:

@startuml

hide footbox

participant "MCE" #white

participant "MnS Consumer" #white

participant "UDM" #white

participant "AMF" #white

participant "gNB" #white

participant "UE Access Stratum" #white

participant "UE Application Level" #white

rnote over UDM, "UE Access Stratum"#white

UE access to network by Registration procedure

endnote

"MnS Consumer" -> "UDM" :1. Create MOI QMCJob\n-  serviceType\n-  sliceScope\n-  qoECollectionEntityAddress\n-  qoETarget\n-  qoEReference\n-  qMCConfigFile\n-  mDTAlignmentInformation\n-  availableRANVisibleqoEMetrics\n-  mBSCommunicationServiceType\n-  areaScope

"UDM" -> "AMF" :2. Nudm_SDM Data Change Notificationn\n-  serviceType\n-  sliceScope\n-  qoECollectionEntityAddress\n-  qoETarget\n-  qoEReference\n-  mDTAlignmentInformation\n-  availableRANVisibleqoEMetrics\n-  containerForApplicationLayerMeasurement\n  Configuration\n-  mBSCommunicationServiceType\n-  areaScope

"AMF" -> "gNB": 3. UE Context Modification Request\n-  serviceType\n-  sliceSupportListQMC\n-  measCollEntityIPAddress\n-  qoEReference\n-  nGRANTraceId\n-  availableRANVisibleqoEMetrics\n-  containerForAppLayerMeasConfig\n-  mBSCommunicationServiceType\n-  areaScope

rnote over "gNB" #white

4. UE capability match the criteria

for ServiveType

end note

"gNB" -> "UE Access Stratum" :5. RRCReconfiguration\n-  serviceType\n-  measConfigAppLayerId\n-  measConfigAppLayerContainer\n-  transmissionOfSessionStartStop\n-  ran-VisibleParameters\n-  qoECollectionEntityidentity\n-  availableRANqoEMetrics

"UE Access Stratum" -> "UE Application Level": 6. +CAPPLEVMCNR\n-  meas_config_app_layer_id\n-  app_meas_config_file\n-  transmission_of_session_start-end\n-  ran_visible_periodicity\n-  number_of_buffer_levels_entries\n-  app_meas_service_type\n-  report_playout_delay_for_media_startup

rnote over "UE Application Level" #white

7. Application starts and QoE measurement

collectoin is initiated

end note

rnote over "UE Access Stratum", "UE Access Stratum" #lightgray

UE may go to idle and come back to active at any time

end note

"UE Application Level" -> "UE Access Stratum": 8. +CAPPLEVMRNR\n-  meas_config_app_layer_id\n-  qoe_measurement_status\n    -  Session Start

"UE Access Stratum" -> "gNB": 9. MeasurementReportAppLayer\n-  meansConfigAppLayerID\n-  applayerSessionStatus\n    - Session Start\n-  qoECollectionEntityIdentity

rnote over "UE Application Level" #white

10. Measurement collection completed or

end of period for periodic report

end note

"UE Application Level" -> "UE Access Stratum": 11. +CAPPLEVMRNR\n-  meas_config_app_layer_id\n-  playout_delay_for_media_startup\n-  number_of_buffer_level_entries\n-  qoe_measurement_status\n    -  Session Stop\n-  app_meas_report\n    -  recordingSessionID

"UE Access Stratum" -> "gNB": 12. MeasurementReportAppLayer\n-  meansConfigAppLayerID\n-  ran-VisibleMeasurements\n-  applayerSessionStatus\n    - Session Stop\n-  measReportAppLayerContainer\n    - recordingSessionID\n-  qoECollectionEntityIdentity

"gNB" -> "MCE": 13. Report-Container\n- qoEReference\n- time stamp\n-  Report\n    -  slice\n    -  recordingSessionId

@enduml

*** START OF NEXT CHANGE ***

## A.2	QMC activation and reporting example in NR before UE Registration procedure to the network

The following PlantUML source code is used to describe QMC activation and reporting example in NR before UE Registration procedure to the network. As depicted by Figure 4.6.1.2-1:

@startuml

hide footbox

participant "MCE" #white

participant "MnS Consumer" #white

participant "UDM" #white

participant "AMF" #white

participant "gNB" #white

participant "UE Access Stratum" #white

participant "UE Application Level" #white

"MnS Consumer" -> "UDM" :Create MOI QMCJob\n-  serviceType\n-  sliceScope\n-  qoECollectionEntityAddress\n-  qoETarget\n-  qoEReference\n-  qMCConfigFile\n-  mDTAlignmentInformation\n-  availableRANVisibleqoEMetrics\n-  mBSCommunicationServiceType\n-  areaScope

"UE Access Stratum" -> "gNB" :Registration Request

"gNB" -> "AMF":Registration Request

"AMF" -> "UDM":Nudm_UEContextManagement_Register

|||

"UDM" -> "AMF" :API: Nudm_SDM\n-  serviceType\n-  sliceScope\n-  qoECollectionEntityAddress\n-  qoETarget\n-  qoEReference\n-  mDTAlignmentInformation\n-  availableRANVisibleqoEMetrics\n-  containerForApplicationLayerMeasurementConfiguration\n-  mBSCommunicationServiceType\n-  areaScope

"AMF" -> "gNB" :Initial Context Setup Request\n-  serviceType\n-  sliceSupportListQMC\n-  measCollEntityIPAddress\n-  qoEReference\n-  nGRANTraceId\n-  availableRANVisibleqoEMetrics\n-  containerForAppLayerMeasConfig\n-  mBSCommunicationServiceType\n-  areaScope

rnote over "gNB" #white

UE capability match the

criteria for serviceType

end note

"gNB" -> "UE Access Stratum" :RRCReconfiguration\n-  serviceType\n-  measConfigAppLayerId\n-  measConfigAppLayerContainer\n-  transmissionOfSessionStartStop\n-  ran-VisibleParameters

"UE Access Stratum" ->"UE Application Level":+CAPPLEVMCNR\n-  meas_config_app_layer_id\n-  app_meas_config_file\n-  transmission_of_session_start-end\n-  ran_visible_periodicity\n-  number_of_buffer_level_entries\n-  app_meas_service_type\n-  report_playout_delay_for_media_startup

rnote over "UE Application Level"#white

Application starts and QoE

measurement collection is initiated

end note

"UE Application Level" ->"UE Access Stratum":+CAPPLEVMRNR\n-  meas_config_app_layer_id\n-  qoe_measurement_status\n    -  Session Start

"UE Access Stratum" -> "gNB": MeasurementReportAppLayer\n-  measConfigAppLayerId\n-  appLayerSessionStatus\n    -  Session Start

rnote over "UE Application Level"#white

Measurement collection completed or end

of period for periodic report

end note

"UE Application Level" ->"UE Access Stratum":+CAPPLEVMRNR\n-  meas_config_app_layer_id\n-  playout_delay_for_media_startup\n-  number_of_buffer_level_entries\n-  qoe_measurement_status\n    -  Session Stop\n-  app_meas_report\n    -  recordingSessionID

"UE Access Stratum" -> "gNB": MeasurementReportAppLayer\n-  measConfigAppLayerId\n-  ran-VisibleMeasurements\n-  appLayerSessionStatus\n    -  Session Stop\n-  measReportAppLayerContainer\n    -  recordingSessionId

"gNB" -> "MCE":Report-Container\n-  qoEReference\n-  time stamp\n-  Report\n    -  slice\n    -  recordingSessionId

@enduml

*** START OF NEXT CHANGE ***

## A.3	Handling of QMC activation example in case of NG Based handover in NR for Signalling Based Activation

The following PlantUML source code is used to describe the procedure for SNPN provisioning with 3GPP segments only, as depicted by Figure 4.6.2.1-1

@startuml

hide footbox

participant "MCE" #white

participant "MnS Consumer" #white

participant "AMF" #white

participant "Source gNB" #white

participant "Target gNB" #white

participant "UE Access Stratum" #white

participant "UE Application Level" #white

"Source gNB" -> "AMF":1. Handover Required\nsourceToTargetTransparentContainer\n-  qoEReference\n-  serviceType\n-  measConfigAppLayerId\n-  qoECollectionEntityAddress\n-  qoEMeasurementStatus\n-  sliceSupportListQMC\n-  nGRANTraceId\n-  availableRANVisibleqoEMetrics\n-  containerForAppLayerMeasConfig\n-  mBSCommunicationServiceType\n-  areaScope

"AMF" -> "Target gNB":2. Handover Request\nsourceToTargetTransparentContainer

rnote over "Target gNB" #white

3. Target gNB checks if

measurements to be

continued or not

end note

"Target gNB" -> "AMF":4. Handover Request Acknowledge

"AMF" -> "Source gNB":5. Handover Command

"Source gNB" -> "UE Access Stratum":6. RRCReconfiguration

"UE Access Stratum" -> "Target gNB":7. RRCReconfigurationComplete

rnote over "UE Application Level" #white

8. Measurement collection completed or

end of period for periodic report

end note

"UE Application Level" -> "UE Access Stratum":9. +CAPPLEVMRNR\n-  meas_config_app_layer_id\n-  playout_delay_for_media_startup\n-  number_of_buffer_level_entries\n-  report_initial_playout_delay\n-  qoe_measurement_status\n    -  Session Stop\n-  app_meas_report\n-  recordingSessionId

"UE Access Stratum" -> "Target gNB":10. MeasurementReportAppLayer\n-  measConfigAppLayerId\n-  appLayerSessionStatus\n    -  Session Stop\n-  measReportAppLayerContainer\n    -  recordingSessionId\n-  qoECollectionEntityIdentity

"Target gNB" -> "MCE":11. Report-Container\n-  qoEReference\n-  time stamp\n-  Report\n    -  slice\n    -  recordingSessionId

@enduml

*** START OF NEXT CHANGE ***

## A.x	QMC activation example for Management Based Activation and Reporting in NR

The following PlantUML source code is used to describe QMC activation and reporting example in NR after UE is registered. As depicted by Figure 4.5.1.1-1:

@startuml

hide footbox

participant "MCE" #white

participant "MnS Consumer" #white

participant "gNB" #white

participant "UE Access Stratum" #white

participant "UE Application Level" #white

"MnS Consumer" -> "gNB" :1. Create MOI (QMCJob)\n-serviceType\n- areaScope\n- qoECollectionEntityAddress\n- qoEReference\n- mDTAlignmentInformation\n- availableRANqoEMetrics\n- qMCConfigFile\n- mBSCommunicationServiceType.

"UE Application Level" -> "UE Access Stratum": 2. AT command +CAPPLEVMCNR

group UE registration procedure

"UE Access Stratum" -> "gNB": 3. UE capacity information

end

rnote over "gNB" #white

4. UE capacity match the criteria for serviceType

end note

"gNB" -> "UE Access Stratum" :5. RRCReconfiguration\n-  serviceType\n- measConfigAppLayerId\n- measConfigAppLayerContainer\n- ran-VisibleParameters\n- availableRANqoEMetrics

"UE Access Stratum" -> "UE Application Level": 6. Response+CAPPLEVMCNR\n- meas_config_app_layer_id\n- app_meas_config_file\n- app_meas_service_type

rnote over "UE Access Stratum", "UE Access Stratum" #lightgray

UE may go to idle and come back to active at any time

end note

rnote over "UE Application Level" #white

7. Application starts and QoE measurement

collectoin is initiated

end note

"UE Application Level" -> "UE Access Stratum": 8. +CAPPLEVMRNR\n-  meas_config_app_layer_id\n-  qoe_measurement_status

"UE Access Stratum" -> "gNB": 9. MeasurementReportAppLayer\n-  meansConfigAppLayerID\n-  applayerSessionStatus\n-  qoECollectionEntityIdentity

group [loop] as long as application runs

rnote over "UE Application Level" #white

10. Application makes QoE measurement

end note

"UE Application Level" -> "UE Access Stratum": 11. +CAPPLEVMRNR\n-  meas_config_app_layer_id\n-  qoe_measurement_status\n-  app_meas_report

"UE Access Stratum" -> "gNB": 12. MeasurementReportAppLayer\n-  meansConfigAppLayerId\n-  applayerSessionStatus\n-  qoECollectionEntityIdentity

end

"gNB" -> "MCE": 13. Report-Container\n- qoEReference\n-  time stamp\n-  measReportAppLayerContainer

@enduml

*** END OF CHANGE ***