3GPP TSG-SA5 Meeting #156 	S5-243696

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	Huawei

# Title:	Adding a new solution for signaling storm

# Document for:	Approval

Agenda Item:	6.19.5

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP draft TR 28.915: “Management and orchestration; Study on management aspects of Network Digital Twin v0.3.0”.

[2]	SP-231727 "New Study on management aspects of Network Digital Twin"

3	Rationale

This contribution proposes to add a new solution for signaling storm analysis for TR 28.915 based on [1]

4	Detailed proposal

It proposes to make the following changes to TR 28.915[1].

1st Change

5.2.3 Potential solutions

5.2.3.x solution x

In this solution, a NDT is created to model (either using emulation method or simulation method) a specific network scenario and its possible impact on the network. It can be used to determine whether a signaling storm occurs when such network scenario happens.

Figure 5.2.3.x-1: procedure of NDT for sinaling storm analysis

- MnS consumer requests MnS producer (the entity who provides the NDT management service) to create an NDT instance (i.e., a new IOC , may be called NetworkDigitalTwin, is required. It may be name contained in a subnetwork or managed function ) with modeling requirements. The modeling requirements are used to specify the scope of the network to be modelled and the to be modelled network scenario , which may include,

- NDT scope: the area of actual mobile network or the managed object that needs to be simulated or emulated in NDT. For instance, a geography area, a network slice, etc.

- Modeling scenario: the network scenario required to be modelled, e.g., the number of PDU session received by AMF increases to 10 times.

- Modeling data: the selected data to be modelled by NDT, e.g., 5GC related PM data as defined in TS 28.552/28.554, CM data as defined in TS 28.541/28.622, etc.

- Based on the modeling requirements given in step 1, the MnS producer collects the data from the network and creates a NDT instance.

- MnS producer notifies MnS consumer that the NDT instance is created.

- MnS consumer requests NDT to evaluate the impact on the network (e.g. whether a signalling storm occrus). The request parameters may include:

- Evaluation event: in this use case, signalling storm is evaluated.

- Impact detectors: specified performance metrics and/or alarm types that needs to be collected and reported by NDT after the behaviour happens in NDT.

- NDT simulates/emulates the network behaviour and generate network performance data and/or alarm data from the NDT.

- MnS producer reports the result to MnS consumer. The report content may include the result of the evaluation and impact which is a key-value list where the keys contain the impact detectors specified in step 4. Alarms are reported if any raised.

End of Changes