3GPP TSG SA WG5 Meeting #156	S5-243807

Maastricht, Netherlands, 19 - 23 August, 2024

Source:	C4-241558

Title:	LS on 5G Trace to support UE level measurement

Document For:	Information

Agenda Item:	6.1

3GPP TSG SA WG5 Meeting #155	S5-242351

Jeju, South Korea, 27 - 31 May 2024

Source:	C4-241558

Title:	LS on 5G Trace to support UE level measurement

Document For:	Information

Agenda Item:	6.1

3GPP TSG-CT WG4 Meeting #122	C4-241558

# Changsha, P.R.China; 15th – 19th April 2024                                       was1548 was1373

# Title:	LS on 5G Trace to support UE level measurement

# Response to:	LS (S5-241086) on improved KPIs involving end-to-end data volume transfer time analytics from SA5

# Release:	Release 18

# Work Item:	TEI18

Source:	CT4

To:	SA5

Cc:	CT3

Contact Person:

Name:	Rong Wang

Tel. Number:	+86 13810948370

E-mail Address:	wangrongyjy@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C4-241532, C4-241533, C4-241333

1. Overall Description:

CT4 thanks SA5 for their LS on improved KPIs involving end-to-end data volume transfer time analytics.

As for the actions to CT4, please find CT4’s reply below inline:

a)	SA5 respectfully requests SA2, CT3 and CT4 to take the information mentioned above into consideration and extend the Trace activation messages in 5GC to support the signalling-based activation of 5GC UE level measurements collection as replied to question c) mentioned above.

CT4 answer: CT4 agreed the attached CRs to extend the trace configuration parameters in 3GPP TS29.571 and 3GPP TS29.503 to support the signalling-based activation of 5GC UE level measurements collection.

But still some questions remained to be clarified:

Q1:	How is the 5GC UE level measurements collection on SMF triggered/activated?

According to 3GPP TS28.558:

“The QoS monitoring can be initiated on SMF by management system via QFQoSMonitoringControl MOI (see TS 28.541 [3]), or by PCF via QoS Monitoring policy included in the PCC rule provisioning (see TS 23.503 [4]).”

Based on the definition, the 5GC UE level measurements collection on SMF is triggered/activated by management system or by PCF.

According to clause 4.1.2.18 of 3GPP TS32.422:

“For the signaling Trace Session activation for UE level measurements collection in 5GC as part of the PDU Session Establishment procedure for the UE that has already been registered where SMF obtains trace control and configuration parameters from UDM via Nudm_UECM_Registration procedure”

However, it means the 5GC UE level measurements collection on SMF is triggered/activated by obtaining trace control and configuration parameter from UDM.

Q2:	What is the meaning of the following Information:

-	IE named “Trace reporting format” for 5GC UE level measurements collection? CT4 cannot find any definition on the “Trace reporting format” in 3GPP TS 32.422.

-	NE Types (CM) for UE level measurement, i.e. whether EP_N3 and EP_N9 are additional types of NE in the 3GPP TS 32.422 .

-	N4 Session Identifier. How will this IE be used in signalling-based activation of 5GC UE level measurements collection?

Q3:	How is the 5GC UE level measurements collection on UPF triggered/activated? Can SMF re-use the existing Per QoS Flow per QoS Measurement mechanism as defined in clause 5.33.3.2 of 3GPP TS 23.501?

2. Actions:

To SA5

ACTION: 	CT4 kindly asks SA5 to take the above information into consideration and provide answers to all the questions.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm