3GPP TSG-SA5 Meeting #156	S5-243836

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.111 | CR | 0012 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.111 Add missing example for sending an alarm |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Examples on how to use CRUD operations in the HTTP/JSON solution are provided. However, there is no example for sending an alarm. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An example for sending an alarm is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing example for sending an alarm may result in interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Begin of modifications

### A.1.4	Examples

Sending alarm notifications

This example shows how a "notifyNewAlarm" notification is sent.

POST /3gpp-management/alarm-notification-sink HTTP/1.1 Host: example.org Content-Type: application/json { "href": "example.org/SubNetwork=SN1/ManagedElement=ME1", "notificationId": 123456789, "notificationType": "notifyNewAlarm", "eventTime": "Tue, 06 Aug 2019 16:50:26 GMT", "systemDN": "DC=example.org,SubNetwork=SN1,MnsAgent=MA1", "alarmId": "alarm-id-1", "alarmType": "EQUIPMENT_ALARM", "probableCause": "Indeterminate", "perceivedSeverity": "CRITICAL" }

Retrieving alarms

This example shows how to retrieve an alarm based on its "alarmId".

GET /SubNetwork=SN1/AlarmList=AL1?\ fields=/attributes/alarmRecords/alarmId1 HTTP/1.1

Multiple alarms can be retrieved with the following request.

GET /SubNetwork=SN1/AlarmList=AL1?\ fields=/attributes/alarmRecords/(alarmId1 ¦ alarmId2) HTTP/1.1

The next example shows how all alarms with a perceived severity of major or critical can be retrieved.

GET /SubNetwork=SN1/AlarmList=AL1?\ selection=/AlarmList[id="AL1"]/attributes/alarmRecords\ /*[perceivedSeverity="MAJOR" or perceivedSeverity="CRITICAL"] HTTP/1.1

To retrieve all alarms for a specific managed object instance identified by "DN1" the MnS consumer may send the following request.

GET /SubNetwork=SN1/AlarmList=AL1?\ selection=/AlarmList[id="AL1"]/attributes/alarmRecords\ /*[objectInstance="DN1"] HTTP/1.1

A MnS consumer wants to retrieve often all alarms from one Managed Element. A Manged Element is modelled in the management system by an object tree whose base object is a "ManagedElement" instance. In the example below this instance is identified by the DN "example.com/SubNetwork=SN1/ManagedElement=ME1". The Jex expression in the query parameter "selection" evaluates to true for all DNs, that contain (start) with this DN, i.e. for all objects in the object subtree of interest.

GET /SubNetwork=SN1/AlarmList=AL1?\ selection=/AlarmList[id="AL1"]/attributes/alarmRecords\ /*[contains(objectInstance,"example.com/SubNetwork=SN1/ManagedElement=ME1")]

Acknowledging alarms

To acknowledge an alarm a MnS consumer has multiple alternatives. With JSON Patch the request may look as follows.

PATCH /SubNetwork=SN1/AlarmList=AL1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "add", "path": "/attributes/alarmRecords/alarmId1/ackUserId", "value": "userId1" }, { "op": "add", "path": "/attributes/alarmRecords/alarmId1/ackSystemId", "value": "systemId1" }, { "op": "replace", "path": "/attributes/alarmRecords/alarmId1/ackState", "value": "ACKNOWLEDGED" } ]

3GPP JSON Patch allows for a more compact request.

PATCH /SubNetwork=SN1/AlarmList=AL1 HTTP/1.1 Host: example.org Content-Type: application/vnd.3gpp.json-patch+json [ { "op": "merge", "path": "#/attributes/alarmRecords/alarmId1", "value": { "ackUserId": "userId1", "ackSystemId": "systemId1", "ackState": "ACKNOWLEDGED" } } ]

Also JSON Merge Patch is quite compact.

PATCH /SubNetwork=SN1/AlarmList=AL1 HTTP/1.1 Host: example.org Content-Type: application/merge-patch+json { "id": "AL1", "attributes": { "alarmRecords": { "alarmId1": { "ackUserId": "userId1", "ackSystemId": "systemId1", "ackState": "ACKNOWLEDGED" } } } }

Commenting alarms

In this example a comment is added to an alarm identified with its "alarmId".

PATCH /SubNetwork=SN1/AlarmList=AL1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "add", "path": "/attributes/alarmRecords/alarmId1/comments/-", "value": { "commentUserId": "userId1", "commentSystemId": "systemId1", "commentText": "Here is the comment text" } } ]

The MnS producer adds the "commentTime" attribute to the alarm record. The response may be as follows.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "commentTime": "2019-08-06T16:50:26Z", "commentUserId": "id", "commentSystemId": "id", "commentText": "Here is the comment text" }

End of modifications