3GPP TSG-SA5 Meeting #156 	S5-243872

Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	Ericsson Hungary Ltd.

# Title:	Rel-19 pCR TR 28.871 Request atomic operation

# Document for:	Approval

Agenda Item:	6.19.8

# 1	Decision/action requested

Approve the proposal.

# 2	References

[1]	3GPP TR 28.871 Study on Service Based Management Architecture enhancement phase 3

[2]	3GPP TS 32.532 Generic management services

# 3	Rationale

For simple consumers handling non-atomic operation results with output status partial failure presents a complex problem. It should be possible for a consumer to requests atomic handling of an operation. See details in description clause.

# 4	Detailed proposal

First change

# 5.a Request atomic operation

### 5.a.1 Description

The Provisioning MnS operations modifyMOIAttributes, deleteMOI, changeMOIs may end with the status PARTIALLY_FAILED. (Curiously the createMOI operation does not allow this result.) For simple consumers handling partial failure (partial success) presents a complex problem. They need to

- analyze complex error messages,

- check which parts of the operation succeeded,

- make a decision for each combination of failed/succesful partial results

- potentially roll back the partially successful parts

While this might be possible for advanced consumers, it is too expensive for simple consumers.

To avoid such complicated handling, it should be possible for a consumer to require atomic handling of an operation, only accepting SUCCEEDED or FAILED as a status.

5.a.2 Potential requirements

REQ-rel-atomic-1: The producer shall make it possible for the consumer to request atomic handling of ProvMns operations createMOI, deleteMOI, modifyMOIAttributes and changeMOIs.

### 5.a.3 Potential solutions

A new optional input parameter should be added to the operations deleteMOI, modifyMOIAttributes and changeMOIs.

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| atomicHandlingRequired | O | Indicates whether the consumer requires atomic handling of the operation. If True, the producer shall either fully execute the operation or not execute it at all. The output status PARTIALLY_FAILED shall not be returned. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: False |

Producers that support this input parameter shall provide atomic operation handling when requested.

Note: the current createMOI operation does not allow the output status PARTIALLY_FAILED, so it is already atomic today.

Impacted specifications are: 28.532

### 5.a.4 Evaluation of potential solutions

The solution fulfils the proposed requirements.

End of change