3GPP TSG-SA5 Meeting #156	S5-243877

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0392 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.623 Update ConditionMonitor for YANG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The content for the ConditionMonitor.condition attribute was not specified well enough in the YANG solution set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description of the ConditionMonitor.condition attribute in YANG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Underspecified attribute “condition” leading to incompatible implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge only |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1220 at commit c02a08f714047a6b2ef0353756d187120ab6d31a |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-util.yang ***

<CODE BEGINS>

module _3gpp-common-util {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-common-util;

prefix "util3gpp";

import ietf-yang-types { prefix yang ; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp ; }

import _3gpp-common-managed-element { prefix me3gpp ; }

import _3gpp-common-subnetwork { prefix subnet3gpp ; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines Utility IOCs: ConditionMonitor, Scheduler

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623";

revision 2024-07-30 { reference "CR-0392 CR-0393"; }

revision 2024-05-06 { reference CR-0359; }

feature SchedulerUnderManagedElement {

description "Scheduler shall be contained under ManagedElement.";

}

feature SchedulerUnderSubNetwork {

description "Scheduler shall be contained under SubNetwork.";

}

grouping TimeIntervalGrp {

description "This data type defines a time interval within one day.

If the whole day shall be selected, intervalStart shall be set to

00:00:00 and intervalEnd shall be set to 23:59:59.";

leaf intervalStart {

type types3gpp:FullTime;

mandatory true;

description "It indicates the time (in 'full-time' format) when the

service shall be started.

Data type 'FullTime' defines the time as specified by 'full-time'

in RFC3339.";

}

leaf intervalEnd {

type types3gpp:FullTime;

mandatory true;

description "It indicates the time (in 'full-time' format) when the

service shall be stopped.

'FullTime' defines the time as specified by 'full-time' in RFC3339.

intervalEnd should use the same timezone as intervalStart and shall

be later then intervalStart";

}

}

grouping SchedulingTimeGrp {

description "This <<choice>> defines the scheduling time and allows to

configure one of four possible scheduling methods:

- One time interval: The attribute timeWindow presents the active

scheduling time. A duration more than one day may be configured.

- Daily periodicity: Several active intervals per day can be configured

in attribute timeIntervals. The active scheduling times recur each day.

- Weekly periodicity: Several active intervals for one day can be

configured in attribute timeIntervals. The active scheduling times recur

on the days of the weeks configured by attribute daysOfWeek

- Monthly periodicity: Several active intervals for one day can be

configured in attribute timeIntervals. The active scheduling times

recur on the days of the months configured by attribute daysOfMonth.";

choice SchedulingTime {

mandatory true;

case One-time-interval {

uses types3gpp:TimeWindowGrp;

}

case periodic {

list timeIntervals {

description "List of intervals within one day for which the service

shall be active.";

key "intervalStart intervalEnd";

min-elements 1;

uses TimeIntervalGrp;

}

leaf daysOfWeek {

must 'not(../daysOfMonth)';

description "Shall be present in case of Weekly periodicity,

otherwise not";

type types3gpp:DayOfWeek;

}

leaf daysOfMonth {

must 'not(../daysOfWeek)';

description "Shall be present in case of Monthly periodicity,

otherwise not";

type types3gpp:DaysOfMonth;

}

}

}

}

grouping SchedulerGrp {

description "Represents the Scheduler IOC.";

list schedulingTimes {

description "It defines the active scheduling times.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses SchedulingTimeGrp;

}

leaf schedulerStatus {

type boolean;

mandatory true;

config false;

description "Switches between TRUE and FALSE depending upon whether the

configured time constraints are fulfilled or not.";

}

}

grouping SchedulerSubtree {

list Scheduler {

description "This IOC defines a time scheduler.

It can be name-contained by SubNetwork or ManagedElement.

The attribute schedulingTimes allows to configure one or several

active time intervals. The active intervals can be configured to occur

once or recurring periodically.

The boolean attribute schedulerStatus switches between TRUE and FALSE

depending upon whether the configured time constraints are fulfilled

or not. This attribute makes the internal Scheduler status observable.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SchedulerGrp;

}

}

}

augment /me3gpp:ManagedElement {

if-feature SchedulerUnderManagedElement;

uses SchedulerSubtree;

}

augment /subnet3gpp:SubNetwork {

if-feature SchedulerUnderSubNetwork;

uses SchedulerSubtree;

}

///////////////////////////////////////////////////////////////////////////////

feature ConditionMonitorUnderManagedElement {

description "ConditionMonitor shall be contained under ManagedElement.";

}

feature ConditionMonitorUnderSubNetwork {

description "ConditionMonitor shall be contained under SubNetwork.";

}

grouping ConditionMonitorGrp {

description "Represents the Scheduler IOC.";

leaf condition {

type yang:xpath1.0 {

length 1..max;

}

mandatory true;

description "A logical condition containing an XPath expression.

The leaf contains an XPath 1.0 string with the following restrictions

- The accessible tree is the full Network Resource Model

- The root node is the list representing the top level MOI

(E.g. ManagedElement)

- In the accessible tree, all leafs and leaf-lists with default values

in use exist

- The data tree has no concept of document order except when 'ordered-by

user' is declared.

- The set of variable bindings is empty.

- The function library is the core function library defined in XPATH

- Namespaces should be ignored when evaluating the expression

An empty string is not allowed.";

}

leaf conditionStatus {

type boolean;

mandatory true;

config false;

description "The boolean result of the condition.

Switches between TRUE and FALSE depending upon whether the

configured constraints are fulfilled or not.";

}

}

grouping ConditionMonitorSubtree {

list ConditionMonitor {

description "This IOC defines one or several conditions and monitors

whether these conditions are satisfied. It can be name-contained by

SubNetwork or ManagedElement.

The attribute condition allows to configure one or several conditions.

Possible conditions include but are not limited to scheduling

requirements or parameter settings e.g. evaluation if a configuration

parameter is above a certain threshold or has a certain values.

The boolean attribute conditionStatus switches between TRUE and FALSE

depending upon whether the configured conditions are fulfilled or not.

This attribute makes the internal ConditionMonitor status observable.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses ConditionMonitorGrp;

}

}

}

augment /me3gpp:ManagedElement {

if-feature ConditionMonitorUnderManagedElement;

uses ConditionMonitorSubtree;

}

augment /subnet3gpp:SubNetwork {

if-feature ConditionMonitorUnderSubNetwork;

uses ConditionMonitorSubtree;

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***