3GPP TSG-SA5 Meeting #156	S5-243923

# Maastricht, Netherlands, 19 – 23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0585 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 New measurements to monitor Paging Load in Non-Terrestrial Network deployments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Paging Load is the number of UEs that receive a page. In NTN (Non-Terrestrial Network) deployments where cell sizes are large, the paging load could be very high leading to large resource usage of satellite beams. On the other hand, smaller cell sizes would mean more frequent TAU procedures. NTN deployments will need to monitor both these metrics (ie paging load and rate of TAU procedures) to dimension the cell size leading to optimal signalling and radio resource usage. The paging load can be measured either at different instants within a measurement. And the paging load could be defined for both core initial page (for Idle UEs) and RAN initiated page (for UEs in RRC-INACTIVE state). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 2 new measurements are proposed - Instantaneous Paging Load for Core Initiated Page - Instantaneous Paging Load for RAN Initiated Page |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN operators will not be able to measure the paging load in their deployements – this could lead inefficient radio resource usage and service degradation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2 Below new clauses are added 5.1.1.27.a, 5.1.1.27.b, A.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[9]	3GPP TS 32.425: "Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements".

[11]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[12]	Void.

[13]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".[14]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[15]	Void.

[16]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[17]	ETSI GS NFV-IFA027 v2.4.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Performance Measurements Specification".

[18]	Void.

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[21]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[22]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[23]	3GPP TS 29.122: "Technical Specification Group Core Network and Terminals; T8 reference point for Northbound APIs".

[24]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[25]	ETSI ES 202 336-12 V1.2.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[26]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[27]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS); Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[28]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[29]	3GPP TS 38.314: "NR; layer 2 measurements".

[30]	3GPP TS 38.313: "Self-Organizing Networks (SON) for 5G networks".

[31]	3GPP TS 38.415: "NG-RAN; PDU session user plane protocol".

[32]	3GPP TS 38.321: "NR MAC protocol specification".

[33]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[34]	3GPP TS 38.215: "NR; Physical layer measurements".

[35]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[36]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[37]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[38]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[39]	3GPP TS 29.507: "5G System; Access and Mobility Policy Control Service; Stage 3".

[40]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[41]	3GPP TS 29.531: "5G System; Network Slice Selection Services".

[42]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[43]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[44]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[45]	3GPP TS 29.541: "5G System; Network Exposure FunctionServices for Non-IP Data Delivery (NIDD); Stage 3".

[46]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[47]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[48]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[49]	3GPP TS 38.300: "NR and NG-RAN Overall description; Stage-2".

[50]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[51]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 23.273: "5G System (5GS); Location Services (LCS); Stage 2".

[54]	3GPP TS 29.572: "5G System (5GS); Location Management Services; Stage 3".

[55]	Void

[56]	3GPP TS 38.425: "NG-RAN; NR User plane protocol".

[57]	3GPP TS 36.425: "(E-UTRAN); X2 interface user plane protocol".

[58]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[59]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[60]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[61]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[62]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[63]	3GPP TS 28.318: "Management and Orchestration; Network and Service Operations for Energy Utilities (NSOEU)".

[64]	3GPP TS 38.213: "NR;Physical layer procedures for control".

[65]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[66]	3GPP TS 38.401: "NG-RAN; Architecture description".

[67]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting (ATSSS); Stage 3".

[x]	3GPP TR 38.821: "Solutions for NR to support Non-Terrestrial Networks (NTN) "

Start of next change

#### 5.1.1.27.a Distribution of Instantaneous Paging Load for Core Initiated Page in NTN Deployments

a)	This measurement provides distribution of the instantaneous paging load applicable for core-initiated paging in NTN deployments. The measurement is provided per TAI and only for NTN.

b)	CC

c)	Each sample is obtained with an internal sampling period (e.g. one second) as the number of UEs in Idle state in the list of TAIs (comprising the Registration Area) in which a page is sent out by the Core Network as specified in TS 38.300 [49], TS 38.401 [66] and TS 23.501 [4]. The number of UEs in Idle is obtained using the running count of UEs which are incremented whenever a UE is moved from RRC Connected to RRC Idle in a given TAI and summing up the counts per TAI across all the TAIs in which the page is being sent.

d)	Each measurement is an integer.

e)	The measurement name has the form PAG.PagingLoadInstantCore.Bin, where Bin indicates a data volume range which is vendor specific.

f)	NRCellCU

GNBCUCPFunction

g)	Valid for packet switched traffic.

h)	5GS

#### 5.1.1.27.b Distribution of Instantaneous Paging Load for RAN Initiated Page in NTN Deployments

a)	This measurement provides distribution of the of instantaneous paging load applicable to RAN initiated paging in NTN deployments. The measurement is provided per RNA and only for NTN.

b)	CC

c)	Each sample is obtained with an internal sampling period (e.g. one second) as the number of UEs in RRC-INACTIVE state for each RAN based Notification Area (RNA) in the RAN as specified in TS 38.300 [49], TS 38.401 [66] and TS 23.501 [4]. The number of UEs in RRC-INACTIVE is obtained using the running count of UEs which is incremented whenever a UE is moved from RRC Connected to RRC INACTIVE in a given RNA.

d)	Each measurement is an integer.

e)	The measurement name has the form PAG.PagingLoadInstantRAN.Bin, where Bin indicates a data volume range which is vendor specific.

f)	NRCellCU

GNBCUCPFunction

g)	Valid for packet switched traffic.

h)	5GS

Start of next change

# A.x	Monitoring of Paging Load for NTN Deployments

Non-Terrestrial Networks (NTN) typically need to cover large areas to provide their service. One of the challenges due to this large area of coverage is the dimensioning of NTN Cells. This is captured in the NTN TR 38.821 [x] clause 7.3.1 Idle Mode Mobility Enhancements.

Each NTN gNB covers a large geographical area (each cell could cover area with radius of hundreds of kms). The cell area dimensioning poses a problem due to this requirement. Very large area cells would mean large Registration Area where a 5GC initiated page should be broadcast. A Registration Area contains several Tracking Areas each of which in turn may have multiple cells. Since this geographical area is huge in NTN, the number of UEs covered will be very high and this leads to large paging load. On the other hand, small area cells would mean small Registration Areas. This would lead to a larger number of Registration Area updates for UEs that are moving in the overlapping areas.

This consideration leads to the need for NTN operators to monitor both paging load and Registration Area Update rates. The cell sizes should be dimensioned based on the monitoring such that both the paging load and Registration Updates are kept at manageable levels. Currently, RA updates are measured in clause 5.2.2 and paging related measurements are specified in clause 5.1.1.27, but there exists no measurement for paging load monitoring. Without this measurement, NTN deployments may not be able to know how many UEs are being paged in a given RA and hence, may end up over or under dimensioning cell area sizes.

In order to accurately characterize the paging load, two categories of paging load are defined – instantaneous load and average load. The former gives a picture of the paging load at sub intervals of a measurement interval whereas the latter provides the average paging load over the measurement interval. These two categories are defined for UEs in RRC idle state per TAI (for core initial pages) and UEs in RRC inactive state per RNA (for RAN initiated pages).

Once implemented, these paging load measurements could be used in tandem with the TAU related measurements (already defined in clause 5.2.2) to ensure that optimal usage of radio resources are done in NTN deployments.

End of change