# 3GPP TSG-SA5 Meeting #156	S5-243937

# 19 - 23 August 2024, Maastricht, Netherlands

# Source:	Nokia

# Title:	Rel-19 pCR New use case on Pre-processing event data

# Document for:	Approval

# Agenda Item:	6.19.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.866-000 “Study on Management Data Analytics (MDA) – Phase 3”.

# 3	Rationale

The use case on Pre-processing event data for ML training was discussed and agreed in the AI/ML management study TR28.908. It was however observed that although the preprocessing of data is done to support AIML training the use case is more suited to analytics since the pre-processed events can be used for other purposes besides AI/ML training. This pCR transfers to MDA the portion of the use case, requirements and solution that is fitted to the analytics discussion.

# 4	Detailed proposal

Start of first change

## 5.3	Data correlation analytics

### 5.3.Y	Use case X:	 Pre-processing event data

#### 5.3.Y.1	Description

For AI/ML algorithms, a large amount of data points to be stored for eventual training. However, it is possible that not all recorded data is useful as the derived events, may have loss of information or misinformation e.g. with respect to time of the event. So, it may be that not all the availed data necessarily adds value, e.g. if most of it includes biased data which ends up getting discarded. The AI/ML algorithms need to have information-rich events data that is condensed but mostly useful for the required training. For example, one could train an interference optimization solution that learns the best way to combat interference by looking at correlated events which are derived from PM counters of handover failures correlated with signal quality. As such the collected raw data may be pre-processed both in terms of volume and quality to derive events that provide optimised/accurate training data.

Here, a network event represents a specific combination of activities executed on or occurring in the network identified say by a set of instances of threshold crossings. MDA can be used to obtain event data (e.g. to be stored), the data containing information rich network events in the network, to ensure that minimizing storage and processing costs by discarding the unnecessary/redundant raw data does not compromise the ability to and still avails adequate historical information to adequately train ML entities.

For event-like data, there could be multiple sources for the same event data. It may be necessary to eliminate duplications of events or event data that is exposed by the MnS producer. The MDA can support a capability for network events aggregation through which the MnS producer exposes data from multiple sources but without duplications.

The performance measurements and KPIs defined in [5] and [6] are used as inputs along with the Threshold crossing MOIs. Appropriate measurements, KPIs and threshold crossings are used to extract the proposed analytics.

#### 5.3.2.2	Potential requirements

REQ-EVENT-DATA-1: The MnS for MDA should include the capability for network events processing that enables an authorized consumer to request for network events corresponding to the data produced by a specified set of network data producers.

REQ-EVENT-DATA-2: The MnS for MDA should include the capability for network events aggregation through which the MDA MnS producer exposes event data from multiple sources in an aggregated way that eliminates duplications.

#### 5.3.Y.3	Potential solutions

- A new analytics type is proposed for this use case, “MDAAssistedDataCorrelation.NetworkEventsProcessing”

- Enhance the MDARequest with an optional datatype for network events processing, called networkEventsProcessing. The data taype contains the events that has to be monitored from the requested Managed Functions, i.e., an MnS consumer can configure the networkEventsProcessing to compile and generate network events with certain characteristics.

  - For example, the network events processing mentioned may include High Call Drop Rate event, Low Availability KPIs event, Low Retainability KPIs event , High Traffic event , Interference event.

- Enhance MDAReport for the analytics type “MDAAssistedDataCorrelation.NetworkEventsProcessing” which can report the below described events.

  - A new data type for the Metric Threshold Crossing events derived from a threshold or a combination of multiple thresholds is proposed, i.e. the Metric Threshold Crossing events are the events captured when one or more metrics cross the one or more configured thresholds. The definition of the Metric Threshold Crossing events may reuse the threshold monitoring MnS defined in 3GPP TS 28.550 as well as the related NRMs defined in 3GPP TS 28.622 [13]. This analytics outpur may contain the following attributes.

    - The proposed data type may contain the following attributes.

      - Event name

      - Corresponding NFs (DNs)

      - Event description

      - Metric (e.g. availability, retainability, etc.)

      - Condition

      - Threshold

      - Monitoring period

  - A new datatype containing Object Status Change Events, these are the events captured when the status of a managed object instance changes be it due to internal processes within the MOI or due to external actors doing something on the MOI.

    - The proposed datatype may contain following attributes.

      - Event name

      - Corresponding NFs

      - Event description

      - Affected parameter

      - Attribute changed

#### 5.3.Y.4	Evaluation of solutions

The solution proposed in 5.3.Y.3 requires management data from indicated managed functions and provides analytics on the events observed for metric threshold crossing and object status changes. From the mentioned input, MDA may be able to derive the indicated output. Hence this is a feasible solution.

Start of next change

# 6	Conclusions and Recommendations

The technical report identified and documented a range of use cases to leverage the capabilities of management data analytics, derived the corresponding potential requirements, and developed and evaluated the possible solutions.

For enhancement of MDAS in normative work in 3GPP Rel-19, it is recommended to describe the use cases, the requirements, input data and analytics output for the following use cases as applicable:

- Pre-processing event data

End of changes