3GPP TSG-SA5 Meeting #156	S5-243984

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0590 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add UE-initiated PLR measurements for ATSSS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  | Date: |  | 2024-08-09 |
| Category: | B |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |

| Reason for change: | Add Number of PLR measurement requests/responses in UE-initiated procedure |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Add measurements of PLR in UE-initiated procedure |  |  |  |
| Consequences if not approved: | Can not get enough data for analyse network failure and network optimization |  |  |  |
| Clauses affected: | 5.4.13.a, 5.4.13.b, 5.4.13.c, 5.4.13.d |  |  |  |
|  | Y | N |  |  |
| Other specs |  | X | Other core specifications | TS/TR ... CR ... |
| affected: |  | X | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

Start of change

#### 5.4.13.a	Number of PMFP PLR count requests (UE-initiated procedure)

a)	This measurement provides the number of PMFP PLR count requests over the access of the MA PDU session (procedure initiated by UE).

b)	CC.

c)	On receipt of PMFP PLR COUNT REQUEST message by the UPF from UE (see TS 24.193 [67] clause 5.4).

d)	A single integer value.

e)	GTP.PLRCntMeasUEInitReq.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.13.b	Number of PMFP PLR count responses (UE-initiated procedure)

a)	This measurement provides the number of PMFP PLR count responses over the access of the MA PDU session (procedure initiated by UE).

b)	CC.

c)	On transmission of PMFP PLR COUNT RESPONSE message with the same EPTI as the allocated EPTI value and with the RI value of a sent PMFP PLR COUNT REQUEST message (see TS 24.193 [67] clause 5.4) by UE from the UPF.

d)	A single integer value.

e)	GTP.PLRCntMeasUEInitSucc.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.13.c	Number of PMFP PLR report requests (UE-initiated procedure)

a)	This measurement provides the number of PMFP PLR report requests over the access of the MA PDU session (procedure initiated by UE).

b)	CC.

c)	On receipt of PMFP PLR REPORT REQUEST message by the UPF from UE (see TS 24.193 [67] clause 5.4).

d)	A single integer value.

e)	GTP.PLRRptMeasUEInitReq.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.13.d	Number of PMFP PLR report responses (UE-initiated procedure)

a)	This measurement provides the number of PMFP PLR report responses over the access of the MA PDU session (procedure initiated by UE).

b)	CC.

c)	On transmission of PMFP PLR REPORT RESPONSE message with the same EPTI as the allocated EPTI value and with the RI value of a sent PMFP PLR REPORT REQUEST message (see TS 24.193 [67] clause 5.4) by UE from the UPF.

d)	A single integer value.

e)	GTP.PLRRptMeasUEInitSucc.

f)	EP_N3 (contained by UPFFunction); 
EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

End of change