3GPP TSG SA WG5 Meeting #156	S5-244069

# Maastricht, The Netherlands 19 - 23 August 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0238 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.290 Clarification on RSU of quota management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the EPC charging, GGSN/P-GW sends the charging data request to OCS for the requested units. - For different rating groups, the P-GW sends independently the request message. - For the same rating group, the P-GW sends the next requested unit of the rating group after receiving a response from the OCS. The corresponding mechanism should be clarified in the 5G Charging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the related description for the request unit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The quota management mechanism is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.290 … CR 0239 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.4.8.1	General

The quota can be consumed in the network e.g., seconds, bytes. Quota management applies for charging per rating group, including requested quota, granted quota and used units.

The following applies for quota management:

-	NF consumer shall request units via charging data request.

-	CHF as NF producer may either grant or deny the request for units via charging data response.

-	NF consumer shall report the used units via charging data request with the quota management indicating online.

The following applies when the quota management indicates online:

-	NF consumer shall, if quota management is still applicable for the rating group, include requested units.

-	NF consumer shall return all unused granted units to the CHF.

-	NF consumer shall not send a new requested unit for the rating group before receiving the response of previous requested unit for the rating group.

End of change