3GPP TSG-SA5 Meeting #156	S5-244087

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	NTT DOCOMO

# Title:	Add use case of Fault injection analysis for TR 28.915

# Document for:	Approval

Agenda Item:	6.19.5

1	Decision/action requested

The group is asked to discuss and approval

2	References

[1] ETSI GR ZSM-15: "Zero Touch Network and Service Management (ZSM); Network Digital Twin".

3	Rationale

This contribution proposes to add use case of Fault injection analysis.

4	Detailed proposal

5.X Use case#A: Fault injection analysis

5.X.1 Description

Mobile network has become crucial infrastructure and the impact of network failures on society can be substantial. Therefore, it is important to consider measures against potential future network failures. To this end, it is useful to proactively analyse potential future network failures (e.g., analyse potential network failures caused by component failures like VMs or containers turned down). It is impossible to cause issues in actual commercial networks because it would affect users. Therefore, the use of Digital Twin can be investigated for this purpose.

NDT fault injection analysis is described in ETSI GR ZSM 015 clause 5.13 [1]. In this use case, it is described as non-disruptive way of doing fault injection studies. Network digital twin simulates potential fault scenarios and provide results. This enables operator to learn network anomaly patterns of different faults.

For example, mobile networks are built on physical resources and can be built on virtualization technologies. Predicting the impacts on the network induced by the failures on physical resources (e.g., CPU, memory, storage failure of physical servers or port and transmission error of physical networks) or virtual resources (e.g., stopping of VMs or containers, host OS failures) can be challenging. Additionally, with the introduction of technologies like containers, the number of components in a mobile network increase, making it difficult to analyse the network-wide impacts of a single component failure. In the increasingly complex modern networks, it can also be difficult to ascertain the effectiveness of the countermeasures to those failures.

By using NDT, operator can verify the impacts on the network when failures occur. Furthermore, during the verification of countermeasures, operator can simulate a failure on the network within NDT by applying the countermeasures, and observe the results, which can confirm the effectiveness of the countermeasures. Also, as the mobile networks continue to evolve, by continuously verifying the effectiveness of these measures against the changes, a consistently stable mobile network operation can be expected.

The consumer can request NDT to simulate failures and receive the simulation results regarding the impact on the network.

Moreover, network failures can sometimes cause issues originating in a part of the network to propagate throughout the entire network. In such cases, it is necessary to simulate not just individual components, but the entire network or a significant part as network digital twin. Additionally, as networks are structured in layers, an issue in one layer can impact other layers. Therefore, to accurately reproduce the entire network or its parts, it is crucial to retrieve the network topology including how network elements are interconnected and elements/connection capability.

5.X.2 Potential requirements

REQ-NDT-NFS-01 The NDT should have the capability allowing the consumer to request simulation of network failures in the NDT.

REQ-NDT-NFS-02 The NDT should have the capability to induce network failures and analyse the behaviour of the network in the NDT with topology data.

REQ-NDT-NFS-03 The NDT should have the capability to provide the results of simulation of network failures.

REQ-NDT-NFS-04 The management system should have a capability to provide topology data for simulating and/or emulating network failures in NDT.

5.X.3 Potential solutions

5.X.3.1 Solution1: Introduce Network Digital Twin IOC

Introduce NDT IOC with the following attributes:

- Network topology data type including:

- Node information data type including:

- Reference to node (e.g., gNB, AMF)

- List of connected nodes

- Network failure details data type including:

- Type of failure (e.g., CPU, memory, storage failure of physical servers or port and transmission error of physical networks, stopping of VMs or containers, host OS failures)

- Reference to node where failure to be induced

- Network simulation output data type (e.g., node performance data, node fault data)

5.X.4Evaluation of potential solutions

The solution described in clause 5.X.3.1 adopts NRM-based approach. This solution can reuse current specification. Therefore, this is a feasible solution.