3GPP TSG-SA5 Meeting #156 	S5-244090

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	Huawei

# Title:	Editorial improvements

# Document for:	Approval

Agenda Item:	6.19.2

# 1	Decision/action requested

For approval

# 2	References

[1] 	3GPP TR 28.866: “Study on Management Data Analytics (MDA) – Phase 3”.

# 3	Rationale

This contribution proposes editorial improvements.

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.866 [1].

1st change

#### 5.3.2.1	Description

For a ML model training, a large amount of measurement data instances may be collected and does not necessarily add value to training performance. The collected measurement data for ML training can be highly correlated (linear or non-linear), hence there may be high redundancy among the collected measurement data. Hence using all measurement data for a model training (and inference) can be a waste of computing resources and energy. Therefore, it’s necessary to optimise the training data preparation based on the correlation analytics and redundancy information. A correlation analytics may help to detect the redundancy pattern among the measurement data for ML training, such as:

-	For a given task (e.g., analytics, model training), analyses the correlation among the given set of measurement data, the output can be a (much) smaller set of measurement data, with which ML model (re-)training could be much more efficient with limited (or managed) impact to model training performance (comparing to use full set of data). The output may include a recommendation, for example recommendation to optimize measurement data collection for the model training.

-	Regularly renew the correlation analytics as time progresses, since the correlation relationship might change; this is especially useful when there is a need to regularly re-train the ML model.

2nd change

#### 5.7.2.1	Description

This use case is to enhance the existing MDA capability for failure prediction. The enhancements to the existing use case on failure prediction is targeted towards effective correlation of the predicted alarms and the actual alarms which has resulted as predicted.

The existing use case of failure prediction provides a prediction of failure with some additional information, see TS 28.104 [2]. This includes information such as predicted time of failure occurrence, perceived severity, and the recommended actions. These additional information helps the consumer to understand the effect of the failure in the network to a basic extent. However, the current solution does not provide additional information (e.g. predicted failure recovery time or the trend of the problem causing the failure) which may enable the consumer to anticipate the impacts to the network more effectively.

The predicted failure recovery time indicates the time by which the target entity recovers from the predicted failure. For instance, MDA can predict the time to recover for a given failure given the predicted root cause of the predicted failure and the history information.

The attribute “trend” indicates the trend of the failure, whether the failure will intensify or weaken in the future. In case of periodic analytics reports, the trend information can provide information on how the identified issue might transform in the future. This is similar to the “trend” attribute in the alarm solution in 28.532 [3].

Also, for the consumers to effectively use the output of MDA assisted failure prediction, it is required by the consumer to seamlessly correlate the predicted failure and the actual failure when it occurs in the system. The alarm stage 2 solution is defined in 28.532 [3]. To utilize the failure prediction output effectively for this purpose, it is required to enhance the existing failure prediction to include additional information that may help in effective correlation. To achieve this, it is required to enhance the existing solution towards the alarm solution in 28.532 [3].

3rd change

#### 5.7.5.1	Description

The MDARequest IOC as defined in clause 9.3.2 in TS 28.104 [2] is used to request MDA analysis service by the consumer. In the MDARequest, it includes analyticsScope attribute used to indicate the scope of the analytics requested by the MnS consumer. The analyticsScope attribute can be expressed as managedEntitiesScope which possibly can carry the DN of one or more ManagedFunction (see Table 9.5.1 in TS 28.104 [2]). In case the analyticsScope only contains the DN of a ManagedFunction, it means that the analytics only relate to a specific ManagedFunction.

The existing MDA capability for failure prediction is mainly used to analysis and predict whether a fault will occur in the 5G network. The analytics output for fault prediction analysis mainly contains failurePredictionObject, potentialFailureType, eventTime, perceivedSeverity and recommendedActions (see table 8.4.3.1.3-1 in TS 28.104 [2]). Those information elements in the analytics output are about to express the predicted faults that may occur. As described in the above scenario that the analyticsScope only contains the DN of a ManagedFunction and the target ManagedFunction running well, then the current analytics output is not applicable.

When consumer requests fault prediction analysis for a specific ManagedFunction, it is not clearly specified in TS 28.104 [2] how the MDA report can indicate that no fault is predicted. In addition, when a fault is predicted to occur, quick and accurate identification of the affected users, services, cells, and geographical areas is also crucial for fault management and subsequent handling. The MDAS producer needs to identify the affected network entities, users, and geographical areas related to the predicted fault by correlating with the analysing data from multiple sources. The analysis results can assist the consumer to quickly assess the impact of faults and take appropriate actions for fault recovery and service assurance, thereby minimizing the negative impact caused by the fault.

4th change

#### 5.8.1.3	Potential solutions

The solution requires collecting the performance data of the target NF (i.e the NF which got software upgraded) analysing it using time series model by deploying appropriate algorithms; and predict the PM data. If the predicted PM data degrades the software upgrade is considered un-successful.

The solution proposes a new MDA type for software upgrade validation.

The analytics scope of this MDA type would indicate the following:

- The DNs of the NF (NRCellCU, AMF, SMF etc) for which the upgrade validation analytics is requested.

The enabling data for this MDA type would include the following:

- Performance Data (performance measurement and KPI) for the entities, provided by the analytics scope, as defined in 3GPP TS 28.552 [5] and TS 28.554 [6].

- Provisioning Data:  This refers to the operation state of a cell. It shows whether the cell is Enabled or Disabled for service as defined in 3GPP TS 28.541 [13].

- Alarm Information: This refers to the active alarm information (AlarmInformation as defined in 3GPP TS 28.532 [3]) in the network.

NOTE: aforementioned data will be collected for the new software.

The analytics output for this MDA type would include the following:

- UpgradeStatus: This indicates if the upgrade is predicted to be successful in future. This will be a Boolean attribute with default value as TRUE. The value FALSE indicate the un-successful upgrade

End of changes