3GPP TSG-SA5 Meeting #156	S5-244104

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	Huawei

# Title:	Add new solution for NDT support to network automation

# Document for:	Approval

Agenda Item:	6.19.5

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP draft TR 28.915: “Management and orchestration; Study on management aspects of Network Digital Twin v0.3.0”.

[2]	SP-231727 "New Study on management aspects of Network Digital Twin"

3	Rationale

This contribution proposes to add a new solution for NDT support to network automation for TR 28.915 based on [1].

NDT has scenarios such as modeling and evaluation which are related to automation function. Based on the discussion in previous meetings, NDT can be external to MnF or integrated into MnF. But there is no discussion about the detailed solution for the scenario of NDT integrated into MnF. In this tdoc, a solution is provided to try to illustrate the possible procedure of NDT enable the network automated functions (e.g. analytics) to support modeling and evaluation scenarios.

4	Detailed proposal

It proposes to make the following changes to TR 28.915[1].

1st Change

5.5 Use case 5: NDT support to network automation

5.5.1	Description

NDTs may be used to support many automation use cases. An NDT may be integrated into a network automation function, or it may be external to the network automation function. In the case where the NDT is external to the network automation function, it should be possible for the network automation function to define and configure into the NDT the scenario that should be modelled and simulated by the NDT. Then the NDT should implement the defined scenario, simulate it, and subsequently provide an output representing the statues of different network metrics for the simulated scenario

In the case where the NDT is integrated into a network automation function, the NDT can be embedded in the analysis phase of automation process which includes monitoring, analysis, decision and execution. For example, some capabilities (e.g, NDT modeling and evaluation) can be part of MDA. Existing MDA capability can be enhanced by NDT through verifying the analytics results or new MDA capability can be defined. In this case, the modeling or evaluation results may be included in the analytics output provided by the network automation function.

5.5.2	Potential Requirements

REQ-NDT-1: The NDT should support a capability to model the behavior of the network and provide the outcomes of such modelling to consumers.

REQ-NDT-2: The NDT should support a capability enabling an Mns consumer to define the network scenario that should be modelled and simulated.

REQ-NDT-3: The NDT should support a capability to provide an output representing the statues of different network metrics for the simulated scenario

REQ-NDT-4: The NDT should support a capability to enable the network automation function to provide new or enhanced MDA capabilities.

5.5.3	Potential Solutions

5.5.3.x solution x

In this solution, NDT and its related capabilities are integrated into a network automation function to support modeling and evaluation scenarios derived from those scenarios related to NDT. The network automation function in this solution acting as analytics function can receive analytics request and provide analytics output.

Figure 5.5.3.x-1: procedure of NDT support to network automation

- MnS consumer sends management function (e.g. MDAF) an analytics request with analytics type, reporting target and reporting method, etc. The analytics type can be a specific capability, such as signaling storm modeling, failure prediction, and configuration verification in a specific management scenario.

- Based on the analytics request, the management function creates an NDT instance if no NDT instance exists in the management function.

-	NDT scope: the area of actual mobile network or the managed object that needs to be simulated or emulated in NDT. For instance, a geography area, a network slice, etc.

-	Modeling data: the data that collected for NDT, e.g., PM data as defined in TS 28.552/28.554, CM data as defined in TS 28.541/28.622, etc.

- In parallel, the management function collects the data from the managed entities.

- The management function enabled by NDT performs evaluation (signaling storm, failure prediction, or configuration verification) according to the analytics request sent by MnS consumer.

- The management function reports the analytics result to MnS consumer. The report content may include modeling, evaluation, or verification results of NDT.

End of Changes