3GPP TSG-SA5 Meeting #156 	S5-244106

Maastricht, The Netherlands 19 - 23 August 2024

# Source:	Discussion Paper on Overview of Charging Specifications

# Title:	Huawei

# Document for:	Approval

Agenda Item:	7.1

1	Decision/action requested

This is the discussion paper on overview of charging specifications.

2	References

[1]	3GPP TS 32.240 Telecommunication management; Charging management; Charging architecture and principles

[2]	S5-242606	Discussion paper on overview of aom specifications

[3]			Discussion on Information about the SA5 specification structure  https://nwm-trial.etsi.org/#/documents/8902

3	Rationale

An overview of all SA5 charging specifications for the 5G Service Based Architecture (SBA) converged charging and the legacy charging system can be found in TS 32.240 Scope as a diagram. Some delegates from other 3GPP Working group think that the charging specifications are confusing. The non SA5 charging readers give some feedbacks that it is unclear and difficult to understand charging specifications.

-	The diagram uses colour boxes to show the specification is related to general service charging or 5G specific network slice charging, which is unclear the difference between the different colours.

-	For a non SA5 expert, the diagram and introduce is difficult to understand how the charging specifications relates to each other and where to begin.

A clearer guideline is required for understanding and quickly obtain the charging specification needed.

Observation 1: The diagram in the TS 32.240 includes too much information with the different principles, which is not easy to understand.

The charging specifications can be divided into the 3 categories for the development of charging system, from legacy online and offline charging to 5G converged charging system:

-	The common charging specifications referred by all other charging specifications which cover all the 3GPP releases, i.e. TS 32.240 (Common charging architecture and charging principle) and TS 32.298 (CDRs and ASN.1).

-	The special charging specifications only for 5G Charging from Release 15, e.g.  TS 32.255 (SMF Charging) and TS 32.256(AMF Charging).

-	The special charging specifications provide the service charging for 4G/5G charging, e.g. TS 32.254 (SCEF in 4G and NEF in 5G), TS 32.277 (EPC prose charging and 5G prose charging).

The charging specifications can be divided into the 2 categories for the stages of charging normative works:

-	The stage 2 charging specifications: e.g. TS 32.251 (PS domain charging), TS 32.255 (5G data connectivity charging), TS 32.290 (Common 5G Charging operation) and TS 32.297 (Common CDR files format).

-	The stage 3 charging specifications: TS 32.299 (Diameter protocol for legacy Charging), TS 32.291 (SBI for converged charging), TS 32.298 (CDRs and ASN.1).

Analysis: The figure 3-1 is present to clearly specify the relationship between all the charging specifications.

Potential Proposals:

Question 1: How to describe the charging specifications relationship.

Proposal 1: Provide the separately diagram to describe the legacy charging and 5G Charging.

-	Pro: Clearly present 5G charging specifications and legacy charging specifications.

-	Con: Maintain two figures.

Proposal 2: Nothing To DO.

Question 2: Where to present the charging specifications relationship.

Proposal 1: Describe the dependencies and relationships of charging specifications in the TS 32.240.

-	Pro: Non SA5 experts have an easier way of understanding charging specifications relate to each other. Since this would be a clearly specified in the TS 32.240, which is the umbrella of charging specifications and the start of charging.

-	Con: it is a risk for inconsistent because forget to update the Annex.

Proposal 2: Create a new type of document, such as a web page, to document the charging specifications dependencies and relationships.

-	Pro:  Could be easier to maintain and have least impact on existing specs.

-	Con:  Separation of the information from the actual specs could lead to inconsistencies.

4	Detailed proposal

For the implementation of the recommendation the CR will be provide in the next meetings.