3GPP TSG-SA5 Meeting #156 	S5-244110

# Maastricht, NL, 19 - 23 Aug 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | XXX | rev | 0598 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add PLMN granularity for total available PRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | DL total available PRB and UL total available PRB have been defined in TS 28.552 to represent the available physical resource for a NRCellDU. In RAN network sharing scenario, each participant operator has the requirement to be aware of their available physical resource respectively. Currently, there are some PLMN specific measurements related PRB defined in TS 28.552, such as DL Total PRB Usage, Mean DL PRB used for data traffic, etc. To satisfy the resource evaluation requirement in RAN sharing scenario and keep alignment with other PRB related measurements, this contribution proposes to add PLMN gruanularity for DL total available PRB and UL total available PRB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN gruanularity for DL total available PRB and UL total available PRB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PRB related measurement can’t be applied well in RAN sharing scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.6, 5.1.1.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.1.2.6	DL total available PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) in average available downlink. The measurement is optionally split into subcounter per supported PLMN ID.

b)	SI.

c)	Each  measurement is obtained as the average (arithmetic mean) of total availible count of PRBs available for DL traffic transmission per PLMN ID during time period T.

d)	Each measurement, (average number of DL PRBs) is a single integer value. If the optional measurements are performed, the number of measurements is equal to the number of supported PLMN.

e)	RRU.PrbAvailDl, or optionally RRU.PrbAvailDl.PLMN, where PLMN identifies the PLMN ID.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in average for DL traffic.

2nd Change

5.1.1.2.8	UL total available PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) available uplink. The measurement is optionally split into subcounter per supported PLMN ID.

b)	SI.

c)	Each measurement is obtained as the average number (arithmetic mean) of total available  count of PRBs available for UL traffic transmission per PLMN ID during time period T.

d)	Each measurement, (average of total number of UL PRBs) that is a single integer value. If the optional measurements are performed, the number of measurements is equal to the number of supported PLMN.

e)	RRU.PrbAvailUl, which indicates the UL PRB available. Or optionally RRU.PrbAvailUl.PLMN, where PLMN identifies the PLMN ID.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in average UL.

End of Change