3GPP TSG-SA5 Meeting #156 	S5-244112

Maastricht, NL, 19 - 23 Aug 2024

# Source:	China Unicom

# Title:	pCR TR 28.876 Add evaluation, conclusion and recommendation for Use case #1

# Document for:	Approval

Agenda Item:	6.19.17

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.876: “Management aspects of RedCap feature”

3	Rationale

It was approved in SP-231734 to study the management of aspects of RedCap features. One of the working taks is to study enhancement on management of RedCap related initial BWP. The corresponding use case, requirement and potential solution on the multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario were approved during last two meetings. In order to achieve the study objective, the evaluation, conclusion and recommendation for Use case #1 multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario are proposed in this contribution.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

5.1.4 Evaluation of potential solutions

The possible solution described in clause 5.1.3.1 enhances the allowedValues of isInitialBwp attribute of BWP IOC with a new allowedValue INITIAL-REDCAP indicating that an initial BWP can be configured for RedCap UEs. The change is lightweight and it is backward compatible, therefore it is a feasible solution.

2nd Change

6	Conclusion and Recommendation

6.X	Use case #1: Multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario

This use case identifies that access bandwidth of RedCap UEs is different from conventional UEs. An initial BWP can be configured for RedCap UEs’ initial access instead of initial BWP that may exceed the (e)RedCap UE maximum bandwidth. In TS 38.331 [2], the DownlinkConfigCommon and UplinkConfigCommon provide initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap parameters, respectively. However, in TS 28.541[3], only two types of BWP are defined: INITIAL, OTHER, and the INITIAL-REDCAP type is missing. In order to address this issue, the potential solution proposes an approach to configure Multi-Initial BWP under RedCap and Non-RedCap UEs co-existence scenario, and the existed isInitialBwp attribute can be modified and configured for RedCap, or other UEs. Detailed description about this solution is in clause 5.1.3.

It is recommended to make some enhancements on management of RedCap related initial BWP according to this solution in the future normative work to satisfy the requirements for the access bandwidth of RedCap UEs.

End of changes