3GPP TSG-SA5 Meeting #156 	S5-244114

Maastricht , NL, 19 - 23 Aug 2024

# Source:	China Unicom

# Title:	pCR TR 28.876 Add evaluation, conclusion and recommendation for Use case #3

# Document for:	Approval

Agenda Item:	6.19.17

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.876: “Management aspects of RedCap feature”

3	Rationale

It was approved in SP-231734 to study the management of aspects of RedCap features. One of the working taks is to investigate the measurements and KPIs to evaluate the performance of NR networks delivering communication services for RedCap UEs. The corresponding use case, requirement and potential solution on the UE numbers evaluation for RedCap were approved during last two meetings. In order to achieve the study objective, the evaluation, conclusion and recommendation for use case #3 monitoring of RRC connection number for RedCap service is proposed in this contribution.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

5.3.4 Evaluation of potential solutions

The possibile solution described in clause 5.3.3.1 enhances the existing measurements related to RRC connection number defined in TS 28.552[10]. A new filter is introduced, which can be optionally used to separate the measurments into subcounters to represent RedCap UEs number. The change is lightweight and and largely reuses existing measurements.

In summary, the solution described in clause 5.3.3.1 is feasible.

2nd Change

6	Conclusion and Recommendation

6.X	Use case #3: Monitoring of RRC connection number for RedCap service

This use case identifies that existing cell-level measurements can hardly determin the number of RedCap UEs covered by 5G NR accurately, because the current measurements related to UE numbers in TS 28.552 are all performed without recognizing UE type. To address this issue, the potential solution proposes a new approach to enhance the existing measurements by separating the RedCap UEs. Detailed description about this solution is shown in clause 5.3.3.

It is recommended to make some enhancements on the measurements related to RRC number management according to this solution in the future normative work to satisfy the requirements for the evaluation of RedCap UE number and density.

End of changes