3GPP TSG-SA5 Meeting #156 	S5-244118

# Maastricht, NL, 19 - 23 Aug 2024

# Source:	China Unicom

# Title:	pCR TR 28.876 Update requirement and add potential solution for EE KPI of RedCap

# Document for:	Approval

Agenda Item:	6.19.17

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.876: “Management aspects of RedCap feature”

[2]	3GPP TS 28.552: “Management and orchestration;5G performance measurements”

[3]	3GPP TS 28.554: “Management and orchestration; 5G end to end Key Performance Indicators (KPI)”

3	Rationale

It was approved in SP-231734 to study the management of aspects of RedCap features. One of the working taks is to investigate the method to evaluate the energy efficiency for RedCap network. In order to achieve the objective mentioned above, potential solution for EE KPI of RedCap is proposed in this contribution.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

## 5.2	Use case #2: EE KPI for RedCap

### 5.2.1	Description

RedCap is a lightweight network access solution aiming at scenarios with low-cost, low-power, low-energy-consumption and low-speed requirements, mainly applying to industrial sensors, video surveillance, wearable scenarios.

As stated in TS 28.554 [7] clause 6.7, the network energy efficiency (EE) KPI is related to the performance and the energy consumption of the network providing the specific service. When network delivers communication services for RedCap UEs, there has some impacts on the evaluation of EE.

- In the aspect of performacne, the key performance of RedCap service is different from the services such as eMBB, URLLC and MIoT (higher than MIoT but lower than URLLC and eMBB).

- In the aspect of energy consumption, 3GPP has introduced some complexity reduction features (such as bandwidth reduction and eDRX) for RedCap. These new features may have some impacts on the energy consumption of network which should be evaluated separately.

However, TS 28.554 [7] only provides EE KPIs for eMBB, URLLC and MIoT. How to evaluate the EE for RedCap service should be invetigated.

### 5.2.2	Potential requirements

REQ-RedCap-Perf-EE-1: The 3GPP management system should have capability to evaluate the energy efficiency for RedCap service.

REQ-RedCap-Perf-EE-2: The 3GPP management system should have capability to provide the estimation value of energy consumption for RedCap service.

5.2.3	Potential Solutions

5.2.3.1	Potential solution #1

TR 38.875[6] clause 5 specifies the specific requirements for RedCap service in the following use cases: industrial wireless sensors, video surveillance and wearables. Although the performance requirements for RedCap use cases are not identical, they all necessitate considerations regarding throughput, latency or reliability, similar to the performance metrics of network slices for EE.

Therefore, this solution proposes to refer to the calculation method of network slice level energy consumption and EE that defined in TS 28.554, with some enhancements on the granularity which can be performed for RedCap service.

In the meantime, RAN1/RAN2 specifies bandwith reduction for RedCap UEs. Since the operation bandwidth is a dependency of energy consumption, the reduced BWP of the network for RedCap services leads to a decrease on overall energy consumption. Therefore, it is recommended to consider the impact of BWP when calculating the energy consumption of RedCap services by introducing a weight factor.

To sum up:

- This solution proposes to reuse the network slice energy consumption approach to estimate the energy consumption of RedCap with some enhancements on the performed granularity and the enhancement on calculation the proportion taking BWP into consideration.

- This solution proposes to reuse the EE KPIs for network slice with the enhancements on the granularity for RedCap.

5.2.4 Evaluation of potential solutions

TBD

End of changes