3GPP TSG-SA5 Meeting #156 	S5-244129

Maastricht, Netherlands, 19 - 23 August 2024

# Source:CSCN

# Title: pCR TR28.846 Charging for satellite resource rental between satellite network operator and terrestrial network operator

# Document for:	Approval

Agenda Item:	7.5.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TS 23.501 V19.0.0 System architecture for the 5G System (5GS)

[2]		3GPP TS 32.255 V18.4.0 5G data connectivity domain charging

[3]		S5-243007 DP on business scenarios and charging requirements for satellite resource rental between satellite network operator and terrestrial network operator

[4]		3GPP TR 23.700-27 V18.0.0 Study on 5G System with Satellite Backhaul

3	Rationale

3.1Business scenario for satellite resource rental.

As discussed in S5-243007[3], Satellite network operator can lease satellite resources to a terrestrial network operator, Satellite network operator and terrestrial operator should be able to generate billing information based on different rental arrangements.

3.1.1Satellite Backhaul

Terrestrial operators rent satellites which are to be used as a part of the backhaul between (R)AN and 5GC.

Figure 3.1.1-1: Example scenario that gNB has multiple candidate satellite backhauls

3.1.2Edge computing via UPF deployed on the satellite

In order to enable GEO satellite edge computing, a UPF can be deployed on a satellite. Following figure shows a high level architecture of Satellite Edge Computing via UPF on board.

Figure 3.1.2-1: Satellite Edge Computing via UPF on-board

3.1.3Local switch via UPF deployed on the satellite

For UEs in a communication that is served by satellite backhaul, if local data switching via UPF on-board can be enabled, then the communication path between two UEs can be significantly shortened by avoiding using ISL and feeder link on the way towards the PSA on the ground.

Figure 3.1.3-1: Local Data Switching via a UPF on-board

Editor's note:	The above three figures are sourced from 3GPP TR 23.700-27 [4]

3.2Potential charging requirements

The charging mechanism in terrestrial network operator should support conveying charging information on usage of satellite backhaul to the satellite network operator.

Terrestrial network operator could be charged by Satellite network operator based on usage of satellite backhaul per EAS related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes).

The charging mechanism in terrestrial network operator should support conveying charging information on usage of satellite for 5G VN group related to the data volume to the satellite network operator.

3.3 Existing interface for CHF to CHF

N107 Reference point has been introduced for CHF to CHF in TS32.255[2].

Figure 3.3-1 depicts the 5G data connectivity converged charging architecture for roaming local breakout with V-CHF to H-CHF in reference point representation:

Figure 3.3-1: 5G data connectivity converged charging architecture in Local Breakout V-CHF to H-CHF scenario reference point representation

The N40 reference point is defined for the interactions between V-SMF and V-CHF, the N107 reference point is defined for the interactions between V-CHF and H-CHF.

For scenarios with MVNO (owning a CHF referred to as A-CHF) non-roaming, the N40 reference point is defined for the interactions between SMF and CHF owned by MNO, the N107 reference point is used for the interactions between CHF owned by the MNO and A-CHF owned by the MVNO.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.846.

1st change

6	Charging scenarios and key issues

6. X	Topic 1 Charging for satellite resource rental between satellite network operator and terrestrial network operator

6. X.1	Use cases

6. X.1.1	Use Case #1.1: satellite network operator charging terrestrial network operator for backhal

Terrestrial network operator has an agreement to use the satellite from satellite network operator for backhaul, the charging party and charged party can be:

-	Charged party: terrestrial network operator who using the satellite from the Satellite Service Provider(satellite network operator)

-	Charging party: satellite network operator who provides the satellite to terrestrial network operator.

The inter-provider charging could be based on the total data volume transferred via the satellite.

6. X.1.2	Use Case #1.2: satellite network operator charging terrestrial network operator for edge computing

Satellite network operator provides the satellite infrastructure resources to terrestrial network operator to enable the EAS to be running on the satellite. The charging party and charged party can be:

-	Charged party: terrestrial network operator who using the satellite from the Satellite Service Provider (satellite network operator)

-	Charging party: satellite network operator who provides the satellite to terrestrial network operator

This inter-provider charging may be based on usage of satellite backhaul per EAS related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes).

6. X.1.3	Use Case #1.3: satellite network operator charging terrestrial network operator for local switch

Satellite network operator can lease satellite to a terrestrial network operator for local switch via UPF deployed on the satellite.

The MNO has an agreement to use the satellite from satellite provider, the charging party and charged party can be:

-	Charged party: terrestrial network operator who using the satellite from the Satellite Service Provider (satellite network operator)

-	Charging party: satellite network operator who provides the satellite to terrestrial network operator

The inter-provider charging by MNO could be based on usage of satellite for 5G VN group related to the data volume.

6. X.2	Potential charging requirements

REQ-CH_ SATBH_SMNO-01: The 5GS should support collecting and converging charging information related to satellite backhaul service, edge computing or local switch.

6. X.3	Key issues

Key Issue #1a: Aggregation of charging information in terrestrial network operator.

Key Issue #1b: Conveying of charging information from terrestrial network operator to satellite network operator.

- X.4	Possible solutions

6. X.4.1	Solution #1.X: CHF to CHF communication for satellite backhaul wholesale charging

The architecture for terrestrial network operator-CHF to satellite network operator-CHF communication to support satellite backhaul B2B charging is expected as illustrated by the figure:

Figure6. X.4.X-1: CHF to CHF communication for satellite backhaul B2B charging

The Nx reference point is used for the interactions between CHF owned by the terrestrial network operator and CHF owned by the satellite network operator.

The SMF and CEF with CTF interact with CHF using Nchf interface.

SMF capture charging data related to satellite backhaul (satellite backhaul category, data volumes) for each subscriber.

SMF capture charging data related to Local switch (satellite backhaul category, data volumes) for each subscriber.

CEF can capture charging data related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes).

The CDRs produced by terrestrial network operator CHF may be sent to satellite network operator CHF for interconnect charging through Nx interface, which then can be used to aggregate and calculate the amount of data in the terrestrial network operator that can be used for wholesale B2B charging.

The Nx interface can reuse the 107 interface.

6. X.5	Evaluation

Solution1.X using the CHF-CHF interface in the 5G system to provide the aggregated and conveying charging information to support the satellite resource rental charging.

6. X.6	Conclusion

End of changes