3GPP TSG-SA5 Meeting #156	S5-244133

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	Ericsson LM

# Title:	Potential solution of energy consumption metrics

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.880 Study on energy efficiency and energy saving aspects of 5G networks and services

[2]		3GPP TS 28.554 Management and orchestration; 5G end to end Key Performance Indicators (KPI)

# 3	Rationale

This contribution is the potential solution for the use case multi-dimensional network energy efficiency metrics.

The existing energy consumption metrics in TS 28.554 [2] do not cover differentiation on data traffic, number of PDU sessions, and register subscribers, that are needed to support things like energy efficiency for these cases.

# 4	Detailed proposal

This document proposes the following changes in TR 28.880 [1].

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[3]	ETSI GR NFV-IFA 029 V3.3.1 (2019-11): "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS"".

[4]	ETSI GS NFV-IFA 040 V4.3.1 (2022-05): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Requirements for service interfaces and object model for OS container management and orchestration specification".

[5]	ETSI GS NFV-IFA 027 V4.3.1 (2022-06): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[6]	ETSI GS NFV-IFA 027 V5.1.1 (2024-4): "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

[7]		3GPP TS 22.261: Service requirements for the 5G system; Stage 1

[8]	ETSI GS OEU 020: "Operational energy Efficiency for Users (OEU); Carbon equivalent Intensity measurement; Operational infrastructures; Global KPIs; Global KPIs for ICT Sites".

[9]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[10]	ISO/IEC 30134-3:2016: Information technology - Data centres - Key performance indicators - Part 3: Renewable energy factor (REF)

[11]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[12]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G"

[13]	3GPP TS 28.313: " Management and orchestration; Self-Organizing Networks (SON) for 5G networks ".

[14]                      3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[x]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

Editor’s note: reference [6] is not published yet, will be published soon, the latest draft of DGS/NFV-IFA027 is available in the following location: https://docbox.etsi.org/ISG/NFV/Open/Drafts/IFA027ed451

Second change

#### 5.6.3.X	Potential solution #X: Potential Solution X Energy consumption metrics

##### 5.6.3.X.1	Introduction

Adding more 5G energy consumption KPIs in TS 28.554 [x] will allow also for more energy efficiency KPIs as well as estimations of energy consumption in CHF.

##### 5.6.3.X.2	Description

The current energy consumption KPI in TS 28.554 [x] have defined how the calculation may be done for parts that are usage dependent in the case of shared NFs, however for non-shared NF it’s not defined on how the energy consumption can be related to usage. The proposal is therefore to split the energy consumption for a network slice into the different parts that may be dependent on usage and have the energy efficiency include parts it requires for calculating the total energy efficiency. The following Energy Consumption KPIs would be beneficial, they are all based on the ECns defined in clause 6.7.3.3 in TS 28.554 [x]:

-	ECns,DV: This KPI describes the Energy Consumption (EC) of the network slice dependent on data volume. It is obtained similarly to the ECns, where only NFs or nodes that have the energy consumption depending on data volume (i.e. gNB and UPF) are included. The unit of this KPI is J/bit.

ECns,DV=gNBECgNBDRB.PdcpSduVolumeUl+DRB.PdcpSduVolumeDl (1000×1000×NbOfgNB)+UPFECUPFGTP.InDataOctN3UPF+GTP.OutDataOctN3UPF(8×NbOfUPF)+PsaUpfECPsaUpfGTP.InDataOctN9PsaUpf+GTP.OutDataOctN9PsaUpf(8×NbOfPsaUpf)

-	ECns,PDUSes: This KPI describes the Energy Consumption (EC) of the network slice dependent on number of PDU sessions. It is obtained similarly to the ECns, where only NFs or nodes that have the energy consumption depending on number of PDU session (i.e. SMF) are included (i.e. SMF). The unit of this KPI is J/PDUSess.

ECns,PDUSess=SMFECSMFSM.SessionNbrMeanNbOfSMF

-	ECns,RegSub: This KPI describes the Energy Consumption (EC) of the network slice dependent on number of registered subscribers. It is obtained similarly to the ECns, where only NFs or nodes that have the energy consumption depending on number of registered subscribers (i.e. AMF) are included. The unit of this KPI is J/RegSub.

ECns,RegSub=AMFECAMFRM.RegisteredSubNbrMeanNbOfAMF

The calculation of Energy Efficiency for a network slice depending on data volume (i.e. EEeMBB,DV) would look as follows:

EEeMBB,DV=UPFGTP.InDataOctN3UPF.SNSSAI+GTP.OutDataOct.SNSSAI×8(ECns, PDUSes×1)+(ECns,RegSub×1)+(ECns,DV×UPFGTP.InDataOctN3UPF.SNSSAI+GTP.OutDataOct.SNSSAI×8)

End of change