3GPP TSG-SA5 Meeting #156	S5-244158

Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 1013 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.298 Improvement of generic data types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are several data types in the ASN.1 that could be used by more than one record types, these are moved to the generic part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move several data types to the generic part form CHF part. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reuse between records will be more difficult. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1 and 5.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.2.1	Generic ASN.1 definitions

This subclause contains generic CDR syntax definitions, where the term "generic" implies that these constructs are applicable for more than one domain/service/subsystem. Examples of this are syntax definitions that are imported from non-charging 3GPP TSs, e.g. TS 29.002 [214].

.$GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AddressString,

ISDN-AddressString,

LCSClientExternalID,

LCSClientInternalID

FROM MAP-CommonDataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version21 (21)}

-- from TS 29.002 [214]

PositionMethodFailure-Diagnostic,

UnauthorizedLCSClient-Diagnostic

FROM MAP-ER-DataTypes { itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-ER-DataTypes (17) version21 (21)}

-- from TS 29.002 [214]

ObjectInstance

FROM CMIP-1 {joint-iso-itu-t ms (9) cmip (1) modules (0) protocol (3)}

-- from Rec. X.711 [304]

ManagementExtension

FROM Attribute-ASN1Module {joint-iso-itu-t ms (9) smi (3) part2 (2) asn1Module (2) 1}

-- from Rec. X.721 [305]

AE-title

FROM ACSE-1 {joint-iso-itu-t association-control (2) modules (0) apdus (0) version1 (1) };

-- Note that the syntax of AE-title to be used is from

-- ITU-T Rec. X.227[306) / ISO 8650 corrigendum and not "ANY"

--

--  Generic Data Types

--

--

-- A

--

AgeOfLocationInformation 	::= INTEGER

AccessType	::= ENUMERATED

{

threeGPPAccess					(0),

nonThreeGPPAccess				(1)

}

AMFID	::= OCTET STRING (SIZE(3..6))

-- See subclause 2.10.1 of 3GPP TS 23.003 [7] for encoding.

-- Any byte following the 3 first shall be set to ”F”

--

#### -- B

--

BCDDirectoryNumber		::= OCTET STRING

--

-- This type contains the binary coded decimal representation of

-- a directory number e.g. calling/called/connected/translated number.

-- The encoding of the octet string is in accordance with the

-- the elements "Calling party BCD number", "Called party BCD number"

-- and "Connected number" defined in TS 24.008 [208].

-- This encoding includes type of number and number plan information

-- together with a BCD encoded digit string.

-- It may also contain both a presentation and screening indicator

-- (octet 3a).

-- For the avoidance of doubt, this field does not include

-- octets 1 and 2, the element name and length, as this would be

-- redundant.

--

Bitrate	::= OCTET STRING

--

--  See 3GPP TS 29.571 [249] Bitrate data type.

--

--

#### -- C

--

CallDuration 			::= INTEGER

--

-- The call duration is counted in seconds.

-- For successful calls /sessions / PDP contexts, this is the chargeable duration.

-- For call attempts this is the call holding time.

--

CalledNumber			::= BCDDirectoryNumber

CallingNumber	::= BCDDirectoryNumber

CellGlobalId	::= SEQUENCE

{

plmnId					[0] PLMN-Id,

lac						[1] Lac,

cellId					[2] CellIdString

}

CellId	::= OCTET STRING (SIZE(2))

--

-- Coded according to TS 24.008 [208]

--

CellIdString		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

ChargeIndicator			::= INTEGER

{

noCharge			(0),

charge				(1)

}

CauseForRecClosing	::= INTEGER

--

-- Cause codes 0 to 15 are defined 'CauseForTerm' (cause for termination)

-- There is no direct correlation between these two types.

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214]

--

-- In PGW-CDR and SGW-CDR the value servingNodeChange is used for partial record

-- generation due to Serving Node Address list Overflow

-- In SGSN servingNodeChange indicates the SGSN change

--

-- sWGChange value is used in both the S-GW, TWAG and ePDG for inter serving node change

--

{

normalRelease					(0),

partialRecord					(1),

abnormalRelease					(4),

cAMELInitCallRelease			(5),

volumeLimit						(16),

timeLimit						(17),

servingNodeChange				(18),

maxChangeCond					(19),

managementIntervention			(20),

intraSGSNIntersystemChange		(21),

rATChange						(22),

mSTimeZoneChange				(23),

sGSNPLMNIDChange 				(24),

sGWChange						(25),

aPNAMBRChange					(26),

mOExceptionDataCounterReceipt	(27),

unauthorizedRequestingNetwork	(52),

unauthorizedLCSClient			(53),

positionMethodFailure			(54),

unknownOrUnreachableLCSClient	(58),

listofDownstreamNodeChange		(59)

}

CauseForTerm			::= INTEGER

--

-- Cause codes from 16 up to 31 are defined as 'CauseForRecClosing'

-- (cause for record closing).

-- There is no direct correlation between these two types.

--

-- LCS related causes belong to the MAP error causes acc. TS 29.002 [214].

--

{

normalRelease						(0),

partialRecord						(1),

partialRecordCallReestablishment	(2),

unsuccessfulCallAttempt				(3),

abnormalRelease						(4),

cAMELInitCallRelease				(5),

unauthorizedRequestingNetwork		(52),

unauthorizedLCSClient				(53),

positionMethodFailure				(54),

unknownOrUnreachableLCSClient		(58)

}

ChargingID	::= INTEGER (0..4294967295)

--

-- Generated in P-GW, part of IP-CAN bearer

-- 0..4294967295 is equivalent to 0..2**32-1

--

CivicAddressInformation		::= OCTET STRING

--

-- as defined in subclause 3.1 of IETF RFC 4776 [409] excluding the first 3 octets.

--

CNIPMulticastDistribution		::= ENUMERATED

{

nO-IP-MULTICAST				(0),

iP-MULTICAST				(1)

}

--

#### -- D

--

DataNetworkNameIdentifier	::= IA5String (SIZE(1..63))

-- Network Identifier part of DNN in dot representation.

-- For example, if the complete DNN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- The Identifier is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

DataVolumeOctets		::= INTEGER

--

-- The volume of data transferred in octets.

--

DynamicAddressFlag	::= BOOLEAN

Diagnostics						::= CHOICE

{

gsm0408Cause								[0] INTEGER,

-- See TS 24.008 [208]

gsm0902MapErrorValue						[1] INTEGER,

-- Note: The value to be stored here corresponds to the local values defined in the MAP-Errors

-- and MAP-DialogueInformation modules, for full details see TS 29.002 [214].

itu-tQ767Cause							[2] INTEGER,

-- See Q.767 [309]

networkSpecificCause						[3] ManagementExtension,

-- To be defined by network operator

manufacturerSpecificCause				[4] ManagementExtension,

-- To be defined by manufacturer

-- May be used for CHF generated diagnostics

positionMethodFailureCause				[5] PositionMethodFailure-Diagnostic,

-- See TS 29.002 [214]

unauthorizedLCSClientCause				[6] UnauthorizedLCSClient-Diagnostic,

-- See TS 29.002 [214]

diameterResultCodeAndExperimentalResult	[7] INTEGER

-- See TS 29.338 [230], TS 29.337 [231], TS 29.128 [244]

-- May be used for Nchf received diagnostics

}

DiameterIdentity		::= OCTET STRING

--

#### -- E

--

Ecgi	::= SEQUENCE

{

plmnId					[0] PLMN-Id,

eutraCellId			[1] EutraCellId,

nid						[2] Nid OPTIONAL

}

EnhancedDiagnostics					::= SEQUENCE

{

rANNASCause						[0] SEQUENCE OF RANNASCause

}

EutraCellId		::= UTF8String

--

-- See 3GPP TS 29.571 [249] for details

--

EutraLocation	::= SEQUENCE

{

tai							[0] TAI OPTIONAL,

ecgi						[1] Ecgi OPTIONAL,

ageOfLocationInformation	[3] AgeOfLocationInformation OPTIONAL,

ueLocationTimestamp			[4] TimeStamp OPTIONAL,

geographicalInformation		[5] GeographicalInformation	OPTIONAL,

geodeticInformation			[6] GeodeticInformation OPTIONAL,

globalNgenbId				[7] GlobalRanNodeId OPTIONAL,

globalENbId					[8] GlobalRanNodeId OPTIONAL

}

--

#### -- G

--

GCI		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

GeodeticInformation 	::= UTF8String

-- See 3GPP TS 29.571 [249] for details

GeographicalInformation ::= UTF8String

-- See 3GPP TS 29.571 [249] for details

GeographicalLocation ::= SEQUENCE

{

geographicalCoordinates			[0] GeographicalCoordinates OPTIONAL,

civicLocation					[1] OCTET STRING OPTIONAL

}

GeographicalCoordinates::= SEQUENCE

{

latitude			[0] INTEGER OPTIONAL,

longitude			[1] INTEGER OPTIONAL

}

GeraLocation	::= SEQUENCE

{

locationNumber              [0] LocationNumber OPTIONAL,

cgi							[1] CellGlobalId OPTIONAL,

sai							[2] ServiceAreaId OPTIONAL,

lai							[3] LocationAreaId OPTIONAL,

rai							[4] RoutingAreaId OPTIONAL,

vlrNumber					[5] VlrNumber OPTIONAL,

mscNumber					[6] MscNumber OPTIONAL,

ageOfLocationInformation	[7] AgeOfLocationInformation OPTIONAL,

ueLocationTimestamp			[8] TimeStamp OPTIONAL,

geographicalInformation		[9] GeographicalInformation	OPTIONAL,

geodeticInformation			[10] GeodeticInformation OPTIONAL

}

GLI		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

GSNAddress	::= IPAddress

--

#### -- H

--

HFCNodeId		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

--

#### -- I

--

InternalGroupIdentifier		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

InvolvedParty ::= CHOICE

{

sIP-URI		[0] GraphicString, -- refer to rfc3261 [401]

tEL-URI		[1] GraphicString,	-- refer to rfc3966 [402]

uRN			[2] GraphicString,	-- refer to rfc5031 [407]

iSDN-E164 	[3] GraphicString,	-- refer to ITU-T Recommendation E.164[308]

externalId 	[4] UTF8String		-- refer to clause 19.7.2 TS 23.003 [200]

}

IPAddress	::= CHOICE

{

iPBinaryAddress				IPBinaryAddress,

iPTextRepresentedAddress	IPTextRepresentedAddress

}

IPBinaryAddress	::= CHOICE

{

iPBinV4Address			[0] IPBinV4Address,

iPBinV6Address			    IPBinV6AddressWithOrWithoutPrefixLength

}

IPBinV4Address	::= OCTET STRING (SIZE(4))

IPBinV6Address	::= OCTET STRING (SIZE(16))

IPBinV6AddressWithOrWithoutPrefixLength ::= CHOICE

{

iPBinV6Address				[1] IPBinV6Address,

iPBinV6AddressWithPrefix	[4] IPBinV6AddressWithPrefixLength

}

IPBinV6AddressWithPrefixLength ::= SEQUENCE

{

iPBinV6Address				IPBinV6Address,

pDPAddressPrefixLength		PDPAddressPrefixLength DEFAULT 64

}

IPTextRepresentedAddress	::= CHOICE

--

-- IPv4 address are formatted in the "dotted decimal" notation according to IETF RFC 1166 [411].

-- IPv6 address are formatted according to clause 4 of IETF RFC 5952 [412]. The mixed IPv4 IPv6

-- notation according to clause 5 of IETF RFC 5952 [412] is not used.

-- IPv6 address prefix are formatted in the "/" notation and according to clause 4 of

-- IETF RFC 5952 [412].

--

{

iPTextV4Address		[2] IA5String (SIZE(7..15)),

iPTextV6Address		[3] IA5String (SIZE(2..45))

}

--

#### -- L

--

Lac		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

LCSCause				::= OCTET STRING (SIZE(1))

--

-- See LCS Cause Value, TS 49.031 [227]

--

LCSClientIdentity 		::= SEQUENCE

{

lcsClientExternalID	[0] LCSClientExternalID OPTIONAL,

lcsClientDialedByMS	[1] AddressString OPTIONAL,

lcsClientInternalID	[2] LCSClientInternalID OPTIONAL

}

LCSQoSInfo				::= OCTET STRING (SIZE(4))

--

-- See LCS QoS IE, TS 49.031 [227]

--

LevelOfCAMELService		::= BIT STRING

{

basic						(0),

callDurationSupervision		(1),

onlineCharging				(2)

}

LineType		::= ENUMERATED

{

dSL 	(0),

pON		(1)

}

LocalSequenceNumber ::= INTEGER (0..4294967295)

-- Sequence number of the record in this node

-- 0.. 4294967295 is equivalent to 0..2**32-1, unsigned integer in four octets

LocationAreaAndCell		::= SEQUENCE

{

locationAreaCode	[0] LocationAreaCode,

cellId				[1] CellId,

mCC-MNC				[2] MCC-MNC OPTIONAL

}

LocationAreaCode		::= OCTET STRING (SIZE(2))

-- See TS 24.008 [208]

--

#### -- M

--

ManagementExtensions	::= SET OF ManagementExtension

MBMS2G3GIndicator		::= ENUMERATED

{

twoG				(0),	-- For GERAN access only

threeG			(1),	-- For UTRAN access only

twoG-AND-threeG	(2) 	-- For both UTRAN and GERAN access

}

MBMSInformation			::= SET

{

tMGI							[1] TMGI OPTIONAL,

mBMSSessionIdentity			[2] MBMSSessionIdentity OPTIONAL,

mBMSServiceType				[3] MBMSServiceType OPTIONAL,

mBMSUserServiceType			[4] MBMSUserServiceType OPTIONAL, -- only supported in the BM-SC

mBMS2G3GIndicator			[5] MBMS2G3GIndicator OPTIONAL,

fileRepairSupported			[6] BOOLEAN OPTIONAL,			  -- only supported in the BM-SC

rAI							[7] RoutingAreaCode OPTIONAL,	  -- only supported in the BM-SC

mBMSServiceArea				[8] MBMSServiceArea OPTIONAL,

requiredMBMSBearerCaps		[9] RequiredMBMSBearerCapabilities OPTIONAL,

mBMSGWAddress				[10] GSNAddress OPTIONAL,

cNIPMulticastDistribution	[11] CNIPMulticastDistribution OPTIONAL,

mBMSDataTransferStart		[12] MBMSTime OPTIONAL,

mBMSDataTransferStop			[13] MBMSTime OPTIONAL

}

MBMSServiceArea		::= OCTET STRING

MBMSServiceType		::= ENUMERATED

{

mULTICAST		(0),

bROADCAST		(1)

}

MBMSSessionIdentity		::= OCTET STRING (SIZE (1))

-- This octet string is a 1:1 copy of the contents of the MBMS-Session-Identity

-- AVP specified in TS 29.061 [82]

MBMSTime	::= OCTET STRING (SIZE (8))

-- This value indicates the time in seconds relative to 00:00:00 on 1 January 1900 (calculated as

-- continuous time without leap seconds and traceable to a common time reference) where binary

-- encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in

-- the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second as

-- specified in TS 29.061 [82].

MBMSUserServiceType		::= ENUMERATED

{

dOWNLOAD		(0),

sTREAMING	(1)

}

MCC-MNC		::= OCTET STRING (SIZE(3))

--

-- See TS 24.008 [208]

--

MessageClass		::= ENUMERATED

{

personal			(0),

advertisement		(1),

information-service	(2),

auto				(3)

}

MessageReference		::= OCTET STRING

-- The default value shall be one octet set to 0

MSCAddress		::= AddressString

MscNo					::= ISDN-AddressString

-- See TS 23.003 [200]

MscNumber	::= UTF8String

-- See 3GPP TS 29.571 [249] for details

MSISDN					::= ISDN-AddressString

-- See TS 23.003 [200]

MSTimeZone	::= OCTET STRING (SIZE (2))

-- 1. Octet: Time Zone and 2. Octet: Daylight saving time, see TS 29.060 [215]

--

#### -- N

--

N3gaLocation	::= SEQUENCE

{

n3gppTai		[0] TAI OPTIONAL,

n3IwfId			[1] N3IwFId OPTIONAL,

ueIpv4Addr		[2] IPAddress OPTIONAL,

ueIpv6Addr		[3] IPAddress OPTIONAL,

portNumber		[4] INTEGER OPTIONAL,

tnapId			[5] TNAPId OPTIONAL,

twapId			[6] TWAPId OPTIONAL,

hfcNodeId		[7] HFCNodeId OPTIONAL,

w5gbanLineType	[8] LineType OPTIONAL,

gli				[9] GLI OPTIONAL,

gci				[10] GCI OPTIONAL

}

Ncgi	::= SEQUENCE

{

plmnId					[0] PLMN-Id,

nrCellId			[1] NrCellId,

nid						[2] Nid OPTIONAL

}

Nid		::= UTF8String--

-- See 3GPP TS 29.571 [249] for details.

NodeID		::= IA5String (SIZE(1..20))

NodeAddress ::= CHOICE

{

iPAddress 	[0] IPAddress,

domainName	[1] GraphicString

}

NrCellId		::= UTF8String

-- See 3GPP TS 29.571 [249] for details.

NrLocation	::= SEQUENCE

{

tai								[0] TAI OPTIONAL,

ncgi							[1] Ncgi OPTIONAL,

ageOfLocationInformation		[2] AgeOfLocationInformation OPTIONAL,

ueLocationTimestamp				[3] TimeStamp OPTIONAL,

geographicalInformation			[4] GeographicalInformation	OPTIONAL,

geodeticInformation				[5] GeodeticInformation OPTIONAL,

globalGnbId						[6] GlobalRanNodeId OPTIONAL,

ntnTaiInfo						[7] NtnTaiInfo OPTIONAL

}

NtnTaiInfo	::= SEQUENCE

{

pLMNId			[0] PlmnIdNid,

tacList			[1] SEQUENCE OF TAC,

derivedTac		[2]	TAC OPTIONAL

}

--

#### -- P

--

PDPAddressPrefixLength		::=INTEGER (1..64)

--

-- This is an integer indicating the length of the PDP/PDN IPv6 address prefix

-- and the default value is 64 bits.

--

PDPAddress		::= CHOICE

{

iPAddress				[0] IPAddress

--	eTSIAddress				[1] ETSIAddress

--								has only been used in earlier releases for X.121 format

}

PLMN-Id		::= OCTET STRING (SIZE (3))

-- This is in the same format as octets 2, 3 and 4 of the Routing Area Identity (RAI) IE specified

-- in TS 29.060 [215]

PlmnIdNid	::= SEQUENCE

{

pLMNId		[0] PLMN-Id OPTIONAL,

nid			[1] Nid OPTIONAL

}

PositioningData	::= OCTET STRING (SIZE(1..33))

--

-- See Positioning Data IE (octet 3..n), TS 49.031 [227]

--

PriorityType	::= ENUMERATED

{

low			(0),

normal		(1),

high		(2)

}

PSCellInformation	::= SEQUENCE

{

nRcgi					[0] Ncgi OPTIONAL,

ecgi					[1] Ecgi OPTIONAL

}

--

#### -- R

--

Rac		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

RATType		::= INTEGER

-- This integer is based on the RatType specified in TS 29.571 [249]

-- with 3GPP RAT Type specified in TS 29.061 [216] added for backwards compatibility.

{

-- 0 reserved

uTRAN			(1),

gERAN			(2),

wLAN			(3),

-- 4 reserved for GAN

-- 5 reserved for HSPA Evolution

eUTRAN			(6),

virtual			(7),

-- 8 reserved for nBIoT

-- 9 reserved for lTEM

nR				(51),

nR-U			(52),

eUTRAN-U		(53),

lte-m			(54),

wIRELINE		(55),

wIRELINE-CABLE	(56),

wIRELINE-BBF	(57),

nR-REDCAP	(58),

nR-LEO			(59),

nR-MEO			(60),

nR-GEO			(61),

nR-OTHERSAT		(62),

tRUSTED-N3GA	(65),

tRUSTED-WLAN	(66)

-- 101 reserved for IEEE 802.16e

-- 102 reserved for 3GPP2 eHRPD

-- 103 reserved for 3GPP2 HRPD

-- 104 reserved for 3GPP2 1xRTT

-- 105 reserved for 3GPP2 UMB

}

RANNASCause				::=	OCTET STRING

-- This octet string is a 1:1 copy of the contents (i.e. starting with octet 5)

-- of the "RAN/NAS Cause" information element specified in TS 29.274 [223].

RecordingEntity 		::= AddressString

RecordType 	::= INTEGER

{

--  Record values 0..17 and 87,89  are CS specific.

--  The contents are defined in TS 32.250 [10]

moCallRecord			(0),

mtCallRecord			(1),

roamingRecord			(2),

incGatewayRecord		(3),

outGatewayRecord		(4),

transitCallRecord		(5),

moSMSRecord				(6),

mtSMSRecord				(7),

moSMSIWRecord			(8),

mtSMSGWRecord			(9),

ssActionRecord			(10),

hlrIntRecord			(11),

locUpdateHLRRecord		(12),

locUpdateVLRRecord		(13),

commonEquipRecord		(14),

moTraceRecord			(15),	-- used in earlier releases

mtTraceRecord			(16),	-- used in earlier releases

termCAMELRecord			(17),

--  Record values 18..22 are GPRS specific. The contents are defined in TS 32.251 [11]

--  The contents are defined in TS 32.251 [11]

sgsnPDPRecord			(18),

sgsnMMRecord			(20),

sgsnSMORecord			(21),	-- also MME UE originated SMS record

sgsnSMTRecord			(22),	-- also MME UE terminated SMS record

--  Record values 23..25 are CS-LCS specific. The contents are defined in TS 32.250 [10]

--  The contents are defined in TS 32.250 [10]

mtLCSRecord				(23),

moLCSRecord				(24),

niLCSRecord				(25),

--  Record values 26..28 are GPRS-LCS specific. The contents are defined in TS 32.251 [11]

--  The contents are defined in TS 32.251 [11]

sgsnMTLCSRecord			(26),

sgsnMOLCSRecord			(27),

sgsnNILCSRecord			(28),

-- Record values 30..62 are MMS specific. The contents are defined in TS 32.270 [30]

--  The contents are defined in TS 32.270 [30]

mMO1SRecord				(30),

mMO4FRqRecord			(31),

mMO4FRsRecord			(32),

mMO4DRecord				(33),

mMO1DRecord				(34),

mMO4RRecord				(35),

mMO1RRecord				(36),

mMOMDRecord				(37),

mMR4FRecord				(38),

mMR1NRqRecord			(39),

mMR1NRsRecord			(40),

mMR1RtRecord			(41),

mMR1AFRecord			(42),

mMR4DRqRecord			(43),

mMR4DRsRecord			(44),

mMR1RRRecord			(45),

mMR4RRqRecord			(46),

mMR4RRsRecord			(47),

mMRMDRecord				(48),

mMFRecord				(49),

mMBx1SRecord			(50),

mMBx1VRecord			(51),

mMBx1URecord			(52),

mMBx1DRecord			(53),

mM7SRecord				(54),

mM7DRqRecord			(55),

mM7DRsRecord			(56),

mM7CRecord				(57),

mM7RRecord				(58),

mM7DRRqRecord			(59),

mM7DRRsRecord			(60),

mM7RRqRecord			(61),

mM7RRsRecord			(62),

--  Record values 63..70, 82, 89..91 are IMS specific.

--  The contents are defined in TS 32.260 [20]

sCSCFRecord				(63),

pCSCFRecord				(64),

iCSCFRecord				(65),

mRFCRecord				(66),

mGCFRecord				(67),

bGCFRecord				(68),

aSRecord				(69),

eCSCFRecord				(70),

iBCFRecord				(82),

tRFRecord				(89),

tFRecord				(90),

aTCFRecord				(91),

--  Record values 71..75 are LCS specific. The contents are defined in TS 32.271 [31]

--  The contents are defined in TS 32.271 [31]

lCSGMORecord			(71),

lCSRGMTRecord			(72),

lCSHGMTRecord			(73),

lCSVGMTRecord			(74),

lCSGNIRecord			(75),

--  Record values 76..79,86 are MBMS specific.

--  The contents are defined in TS 32.251 [11] and TS 32.273 [33]

sgsnMBMSRecord			(76),

ggsnMBMSRecord			(77),

gwMBMSRecord			(86),

sUBBMSCRecord			(78),

cONTENTBMSCRecord		(79),

gwMBMSRecord			(86),

--  Record Values 80..81 are PoC specific. The contents are defined in TS 32.272 [32]

--  The contents are defined in TS 32.272 [32]

pPFRecord				(80),

cPFRecord				(81),

--  Record values 84,85 and 92,95,96, 97 are EPC specific.

--  The contents are defined in TS 32.251 [11]

sGWRecord				(84),

pGWRecord				(85),

tDFRecord				(92),

iPERecord				(95),

ePDGRecord				(96),

tWAGRecord				(97),

--  Record Value 83 is MMTel specific. The contents are defined in TS 32.275 [35]

--  The contents are defined in TS 32.275 [35]

mMTelRecord				(83),

--  Record value  87,88 and 89 are CS specific. The contents are defined in TS 32.250 [10]

--  The contents are defined in TS 32.250 [10]

mSCsRVCCRecord			(87),

mMTRFRecord				(88),

iCSRegisterRecord		(99),

--  Record values 93 and 94 are SMS specific. The contents are defined in TS 32.274 [34]

--  The contents are defined in TS 32.274 [34]

sCSMORecord				(93),

sCSMTRecord				(94),

--  Record values 100, 101 and 102 are ProSe specific. The contents are defined in TS 32.277 [36]

--  The contents are defined in TS 32.277 [36]

pFDDRecord				(100),

pFEDRecord				(101),

pFDCRecord				(102),

--  Record values103 and 104 are Monitoring Event specific. The contents are defined in TS

--	32.278 [38]

--  The contents are defined in TS

32.278 [38]

mECORecord				(103),

mERERecord				(104),

--

--  Record values 105 to 106 are CP data transfer specific. The contents are defined in TS

--	32.253 [13]

--  The contents are defined in TS

32.253 [13]

cPDTSCERecord			(105),

cPDTSNNRecord			(106), --

--  Record values 110 to 113 are SMS specific. The contents are defined in TS

--	32.274 [34]

--  The contents are defined in TS

32.274 [34]

sCDVTT4Record			(110),

sCSMOT4Record			(111),

iSMSMORecord			(112),

iSMSMTRecord			(113),

--  Record values 120 are Exposure Function API specific. The contents are defined in TS

--	32.254 [14]

--  The contents are defined in TS

32.254 [14]

eASCERecord				(120),

--  Record values from 200 are specific to Charging Function domain

chargingFunctionRecord		(200)

--

}

RequiredMBMSBearerCapabilities		::= OCTET STRING (SIZE (3..14))

--

-- This octet string is a 1:1 copy of the contents (i.e. starting with octet 5) of the

-- "Quality of service Profile" information element specified in TS 29.060 [75].

--

RoamerInOut	::= ENUMERATED

{

roamerInBound		(0),

roamerOutBound		(1)

}

RoutingAreaCode	::= OCTET STRING (SIZE(1))

-- See TS 24.008 [208]

--

#### -- S

--

Sac		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

SCSASAddress		::= SET

{

sCSAddress		[1] IPAddress,

sCSRealm		[2] DiameterIdentity

}

Session-Id	::= GraphicString

-- rfc3261 [401]: example for SIP CALL-ID: f81d4fae-7dec-11d0-a765-00a0c91e6bf6@foo.bar.com

ServiceAreaId	::= SEQUENCE

{

plmnId			[0] PLMN-Id,

lac				[1] Lac,

sac				[2] Sac

}

ServiceContextID		::= UTF8String

ServiceSpecificInfo  ::=  SEQUENCE

{

serviceSpecificData		[0] GraphicString OPTIONAL, 
	serviceSpecificType		[1] INTEGER OPTIONAL

}

SingleNSSAI	::= SEQUENCE

-- See S-NSSAI subclause 28.4.2 of TS 23.003 [200] for encoding.

{

sST			[0] SliceServiceType,

sD 			[1] SliceDifferentiator OPTIONAL

}

SliceServiceType ::= INTEGER (0..255)

-- See subclause 28.4.2 TS 23.003 [200]

SliceDifferentiator		::= OCTET STRING (SIZE(3))

-- See subclause 28.4.2 TS 23.003 [200]

SMSResult					::= Diagnostics

SmsTpDestinationNumber ::= OCTET STRING

-- This type contains the binary coded decimal representation of

-- the SMS address field the encoding of the octet string is in

-- accordance with the definition of address fields in TS 23.040 [201].

-- This encoding includes type of number and numbering plan indication

-- together with the address value range.

SubscriberEquipmentNumber	::= SET

-- If SubscriberEquipmentType is set to IMEISV and IMEI is received, the number of digits is 15.

{

subscriberEquipmentNumberType	[0]	SubscriberEquipmentType,

subscriberEquipmentNumberData	[1]	OCTET STRING

}

SubscriberEquipmentType	::= ENUMERATED

-- It should be noted that depending on the services, not all equipment types are applicable.

-- For IMS equipment types 0 and 3 are applicable.

-- In 5GS, for PEI defined as:

--        - IMEI or IMEISV, iMEISV type is used and the data is per TS 23.003 [200] format.

--        - MAC address, mAC type is used, and the data is converted from JSON format of the PEI

--          described in TS 29.571 [249].

--        - EUI-64, uEI64 type is used, and the data is converted from JSON format of the PEI

--          described in TS 29.571 [249].

{

iMEISV			(0),

mAC				(1),

eUI64			(2),

modifiedEUI64	(3)

}

SubscriptionID	::= SET

-- See TS 23.003 [200] and TS 29.571 [249]

{

subscriptionIDType	[0]	SubscriptionIDType,

subscriptionIDData	[1]	UTF8String

}

SubscriptionIDType	::= ENUMERATED

-- eND-USER-NAI can be used for externalIdentifier.

-- eND-USER-IMSI can be used for 5G BRG or 5G CRG.

-- eND-USER-NAI can be used for GLI or GCI for wireline access network scenarios

-- NAI format for GCI and GLI is specified in 28.15.5 and 28.15.6 of TS 23.003 [200].

{

eND-USER-E164		(0),

eND-USER-IMSI		(1),

eND-USER-SIP-URI		(2),

eND-USER-NAI			(3),

eND-USER-PRIVATE		(4)

-- eND-USER-NAI can be used for externalIdentifier.

-- eND-USER-IMSI can be used for 5G BRG or 5G CRG.

-- eND-USER-NAI can be used for GLI or GCI for wireline access network scenarios

-- NAI format for GCI and GLI is specified in 28.15.5 and 28.15.6 of TS 23.003 [200].

}

SystemType	::= ENUMERATED

--  "unknown" is not to be used in PS domain.

{

unknown				(0),

iuUTRAN				(1),

gERAN				(2)

}

--

#### -- T

--

ThreeGPPPSDataOffStatus		::= ENUMERATED

{

active		    (0),

inactive		(1)

}

TimeStamp	::= OCTET STRING (SIZE(9))

-- The contents of this field are a compact form of the UTCTime format

-- containing local time plus an offset to universal time. Binary coded

-- decimal encoding is employed for the digits to reduce the storage and

-- transmission overhead

-- e.g. YYMMDDhhmmssShhmm

-- where

-- YY 	= 	Year 00 to 99		BCD encoded

-- MM 	= 	Month 01 to 12 		BCD encoded

-- DD	=	Day 01 to 31		BCD encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

-- ss	=	second 00 to 59		BCD encoded

-- S	=	Sign 0 = "+", "-"	ASCII encoded

-- hh	=	hour 00 to 23		BCD encoded

-- mm	=	minute 00 to 59		BCD encoded

TMGI		::= OCTET STRING

-- This  octet string is a 1:1 copy of the contents (i.e. starting with octet 4)

-- of the "TMGI" information element specified in TS 29.060 [75].

TNAPId		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

TWAPId		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

--

#### -- U

--

UserLocationInformation	::= OCTET STRING

-- UserLocationInformation is an alternative JSON format to UserLocationInformationStructured

-- and is converted from JSON format of the User Location as described in TS 29.571 [249].

UserLocationInformationStructured 	::= SEQUENCE

-- UserLocationInformationStructured is an alternative ASN.1 format to UserLocationInformation

{

eutraLocation				[0] EutraLocation OPTIONAL,

nrLocation					[1] NrLocation OPTIONAL,

n3gaLocation				[2] N3gaLocation OPTIONAL,

utraLocation				[3] UtraLocation OPTIONAL,

geraLocation				 [4] GeraLocation OPTIONAL

}

UtraLocation	::= SEQUENCE

{

cgi							[0] CellGlobalId OPTIONAL,

sai						[1] ServiceAreaId OPTIONAL,

lai							[2] LocationAreaId OPTIONAL,

rai						[3] RoutingAreaId OPTIONAL,

ageOfLocationInformation	[4] AgeOfLocationInformation OPTIONAL,

ueLocationTimestamp			[5] TimeStamp OPTIONAL,

geographicalInformation		[6] GeographicalInformation	OPTIONAL,

geodeticInformation			[7] GeodeticInformation OPTIONAL

}

--

#### -- V

--

VlrNumber	::= UTF8String

-- See 3GPP TS 29.571 [249] for details

.#END

Second change

#### 5.2.5.2	CHF CDRs

This subclause contains the abstract syntax definitions that are specific to the CHF CDR types defined in this document.

.$CHFChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) chfChargingDataTypes (15) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

AMFID,

AccessType,

Bitrate,

CallDuration,

CauseForRecClosing,

ChargingID,

DataVolumeOctets,

DataNetworkNameIdentifier,

Diagnostics,

DynamicAddressFlag,

Ecgi,

EnhancedDiagnostics,

GeographicalLocation,

InternalGroupIdentifier,

DynamicAddressFlag,

InvolvedParty,

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

MessageClass,

MessageReference,

MSCAddress,

MSISDN,

MSTimeZone,

Ncgi,

Nid,

NodeAddress,

PLMN-Id,

PlmnIdNid,

PriorityType,

PSCellInformation,

RANNASCause,

RATType,

RecordType,

RoamerInOut,

ServiceSpecificInfo,

Session-Id,

SingleNSSAI,

SliceServiceType,

SubscriberEquipmentNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp,

TMGI,

UserLocationInformation,

UserLocationInformationStructured

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

AddressString,

IMSI

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18) version21 (21)}

CalleePartyInformation,

ChargingCharacteristics,

ChargingRuleBaseName,

ChChSelectionMode,

EventBasedChargingInformation,

PresenceReportingAreaInfo,

RatingGroupId,

ServiceIdentifier

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

OriginatorInfo,

RecipientInfo,

SMAddressInfo,

SMMessageType,

SMSResult,

SMSStatus

FROM SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  smsChargingDataTypes (10) asn1Module (0) version2 (1)}

APIDirection

FROM ExposureFunctionAPIChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) exposureFunctionAPIChargingDataTypes (14) asn1Module (0) version2 (1)}

SupplService

FROM MMTelChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) mMTelChargingDataTypes (9) asn1Module (0) version2 (1)}

AccessNetworkInfoChange,

AccessTransferInformation,

ApplicationServersInformation,

CalledIdentityChange,

CarrierSelectRouting,

Early-Media-Components-List,

FEIdentifierList,

IMS-Charging-Identifier,

IMSCommunicationServiceIdentifier,

InterOperatorIdentifiers,

ISUPCause,

ListOfInvolvedParties,

ListOfReasonHeader,

MessageBody,

NNI-Information,

NumberPortabilityRouting,

Role-of-Node,

S-CSCF-Information,

SDP-Media-Component,

ServedPartyIPAddress,

Service-Id,

SessionPriority,

SIP-Method,

TADIdentifier,

TransitIOILists,

TransmissionMedium,

TrunkGroupID

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version2 (1)}

AppSpecificData,

ProseFunctionality,

ProSeEventType,

ProSeUERole,

RangeClass,

ProximityAlertIndication,

ChangeOfProSeCondition,

CoverageInfo,

RadioParameterSetInfo,

TransmitterInfo

FROM ProSeChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) proseChargingDataTypes (11) asn1Module (0) version2 (1)}

;

--

#### -- CHF RECORDS

--

CHFRecord	::= CHOICE

--

-- Record values 200..201 are specific

--

{

chargingFunctionRecord			[200] ChargingRecord

}

ChargingRecord 	::= SET

{

recordType									[0] RecordType,

recordingNetworkFunctionID					[1] NetworkFunctionName,

subscriberIdentifier						[2] SubscriptionID OPTIONAL,

nFunctionConsumerInformation				[3] NetworkFunctionInformation,

triggers									[4] SEQUENCE OF Trigger OPTIONAL,

listOfMultipleUnitUsage						[5] SEQUENCE OF MultipleUnitUsage OPTIONAL,

recordOpeningTime							[6] TimeStamp,

duration									[7] CallDuration,

recordSequenceNumber						[8] INTEGER OPTIONAL,

causeForRecClosing							[9] CauseForRecClosing,

diagnostics									[10] Diagnostics OPTIONAL,

localRecordSequenceNumber					[11] LocalSequenceNumber OPTIONAL,

recordExtensions							[12] ManagementExtensions OPTIONAL,

pDUSessionChargingInformation				[13] PDUSessionChargingInformation OPTIONAL,

roamingQBCInformation						[14] RoamingQBCInformation OPTIONAL,

sMSChargingInformation						[15] SMSChargingInformation OPTIONAL,

chargingSessionIdentifier					[16] ChargingSessionIdentifier OPTIONAL,

serviceSpecificationInformation				[17] OCTET STRING OPTIONAL,

exposureFunctionAPIInformation				[18] ExposureFunctionAPIInformation OPTIONAL,

registrationChargingInformation				[19] RegistrationChargingInformation OPTIONAL,

n2ConnectionChargingInformation				[20] N2ConnectionChargingInformation OPTIONAL,

locationReportingChargingInformation		[21] LocationReportingChargingInformation OPTIONAL,

incompleteCDRIndication						[22] IncompleteCDRIndication OPTIONAL,

tenantIdentifier							[23] TenantIdentifier OPTIONAL,

mnSConsumerIdentifier						[24] MnSConsumerIdentifier OPTIONAL,

nSMChargingInformation						[25] NSMChargingInformation OPTIONAL,

nSPAChargingInformation						[26] NSPAChargingInformation OPTIONAL,

chargingID									[27] ChargingID OPTIONAL,

iMSChargingInformation						[28] IMSChargingInformation OPTIONAL,

mMTelChargingInformation					[29] MMTelChargingInformation OPTIONAL,

edgeInfrastructureUsageChargingInformation	[30] EdgeInfrastructureUsageChargingInformation OPTIONAL,

eASDeploymentChargingInformation			 [31] EASDeploymentChargingInformation OPTIONAL,

directEdgeEnablingServiceChargingInformation	[32] ExposureFunctionAPIInformation OPTIONAL,

exposedEdgeEnablingServiceChargingInformation	[33] ExposureFunctionAPIInformation OPTIONAL,

proseChargingInformation 					[34] ProseChargingInformation OPTIONAL,

eASID										[35] UTF8String OPTIONAL,

eDNID							 			[36] UTF8String OPTIONAL,

eASProviderIdentifier						[37] UTF8String OPTIONAL,

mMSChargingInformation						[38] MMSChargingInformation OPTIONAL,

aMFIdentifier								[39] AMFID OPTIONAL,

invocationTimestamp							[40] TimeStamp OPTIONAL,

nSACFChargingInformation					[41] NSACFChargingInformation OPTIONAL,

tSNChargingInformation					[42] TSNChargingInformation OPTIONAL,

mBSSessionChargingInformation				[43] MbsSessionChargingInformation OPTIONAL,

interCHFInformation							[44] InterCHFInformation OPTIONAL,

nSSAAChargingInformation					[45] NSSAAChargingInformation OPTIONAL

}

--

#### -- PDU Session Charging Information

--

PDUSessionChargingInformation 	::= SET

{

pDUSessionChargingID					[0] ChargingID,

userIdentifier							[1] InvolvedParty OPTIONAL,

userEquipmentInfo						[2] SubscriberEquipmentNumber OPTIONAL,

userLocationInformation					[3] UserLocationInformation OPTIONAL,

userRoamerInOut							[4] RoamerInOut OPTIONAL,

presenceReportingAreaInfo				[5]	PresenceReportingAreaInfo OPTIONAL,

pDUSessionId							[6] PDUSessionId,

networkSliceInstanceID					[7] SingleNSSAI OPTIONAL,

pDUType									[8] PDUSessionType OPTIONAL,

sSCMode									[9] SSCMode OPTIONAL,

sUPIPLMNIdentifier						[10] PLMN-Id OPTIONAL,

servingNetworkFunctionID				[11] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

rATType									[12] RATType OPTIONAL,

dataNetworkNameIdentifier				[13] DataNetworkNameIdentifier OPTIONAL,

pDUAddress								[14] PDUAddress OPTIONAL,

authorizedQoSInformation				[15] AuthorizedQoSInformation OPTIONAL,

uETimeZone 								[16] MSTimeZone OPTIONAL,

pDUSessionstartTime						[17] TimeStamp OPTIONAL,

pDUSessionstopTime						[18] TimeStamp OPTIONAL,

diagnostics								[19] Diagnostics OPTIONAL,

chargingCharacteristics					[20] ChargingCharacteristics OPTIONAL,

chChSelectionMode						[21] ChChSelectionMode OPTIONAL,

threeGPPPSDataOffStatus						[22] ThreeGPPPSDataOffStatus OPTIONAL,

rANSecondaryRATUsageReport 				[23] SEQUENCE OF NGRANSecondaryRATUsageReport OPTIONAL,

subscribedQoSInformation 				[24] SubscribedQoSInformation OPTIONAL,

authorizedSessionAMBR 					[25] SessionAMBR OPTIONAL,

subscribedSessionAMBR 					[26] SessionAMBR OPTIONAL,

servingCNPLMNID							[27] PLMN-Id OPTIONAL,

sUPIunauthenticatedFlag 				[28] NULL OPTIONAL,

dnnSelectionMode						[29] DNNSelectionMode OPTIONAL,

homeProvidedChargingID					[30] ChargingID OPTIONAL,

mAPDUNonThreeGPPUserLocationInfo		[31] UserLocationInformation OPTIONAL,

mAPDUNonThreeGPPRATType					[32] RATType OPTIONAL,

mAPDUSessionInformation					[33] MAPDUSessionInformation OPTIONAL,

enhancedDiagnostics						[34] EnhancedDiagnostics5G OPTIONAL,

userLocationInformationASN1				[35] UserLocationInformationStructured OPTIONAL,

mAPDUNonThreeGPPUserLocationInfoASN1	[36] UserLocationInformationStructured OPTIONAL,

userLocationTime						[37] TimeStamp OPTIONAL, -- not to be used

-- user location info time is included under UserLocationInformation

mAPDUNonThreeGPPUserLocationTime		[38] TimeStamp OPTIONAL,

listOfPresenceReportingAreaInformation	[39] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL,

redundantTransmissionType				[40] RedundantTransmissionType OPTIONAL,

pDUSessionPairID						[41] PDUSessionPairID OPTIONAL,

fiveGLANTypeService						[42] FiveGLANTypeService OPTIONAL,

cpCIoTOptimisationIndicator				[43] TimeStamp OPTIONAL,

fiveGSControlPlaneOnlyIndicator			[44] QosMonitoringReport OPTIONAL,

smfChargingID							[45] UTF8String OPTIONAL,

smfHomeProvidedChargingID				[46] UTF8String OPTIONAL,

sNPNInformation							[47] SNPNInformation OPTIONAL,

hPLMNSNSSAI								[48] SingleNSSAI OPTIONAL,

iMSSessionInformation					[49] IMSSessionInformation OPTIONAL,

alternativeSNSSAI						[50] SingleNSSAI OPTIONAL,

fiveGSBridgeInformation					[51] FiveGSBridgeInformation OPTIONAL,

fiveGMulticastService					[52] FiveGMulticastService OPTIONAL,

satelliteAccessIndicator				[53] BOOLEAN OPTIONAL,

satelliteBackhaulInformation			[54] SatelliteBackhaulInformation OPTIONAL

}

--

#### -- Roaming QBC Information

--

RoamingQBCInformation 	::= SET

{

multipleQFIcontainer			[0] SEQUENCE OF MultipleQFIContainer OPTIONAL,

uPFID							[1] NetworkFunctionName OPTIONAL,

-- included for backwards compatibility and

-- can be included based on operators requirement

roamingChargingProfile			[2] RoamingChargingProfile OPTIONAL

}

--

#### -- SMS Charging Information

--

SMSChargingInformation	::= SET

{

originatorInfo				[1] OriginatorInfo OPTIONAL,

recipientInfos				[2] SEQUENCE OF RecipientInfo OPTIONAL,

userEquipmentInfo			[3] SubscriberEquipmentNumber OPTIONAL,

userLocationInformation		[4] UserLocationInformation OPTIONAL,

uETimeZone 					[5] MSTimeZone OPTIONAL,

rATType						[6] RATType OPTIONAL,

sMSCAddress					[7] AddressString OPTIONAL,

eventtimestamp				[8] TimeStamp,

-- 9 to 19 is for future use

sMDataCodingScheme			[20] INTEGER OPTIONAL,

sMMessageType				[21] SMMessageType OPTIONAL,

sMReplyPathRequested			[22] SMReplyPathRequested OPTIONAL,

sMUserDataHeader				[23] OCTET STRING OPTIONAL,

sMSStatus					[24] SMSStatus OPTIONAL,

sMDischargeTime				[25] TimeStamp OPTIONAL,

sMTotalNumber 				[26] INTEGER OPTIONAL,

sMServiceType				[27] SMServiceType OPTIONAL,

sMSequenceNumber 			[28] INTEGER OPTIONAL,

sMSResult					[29] SMSResult OPTIONAL,

submissionTime				[30] TimeStamp OPTIONAL,

sMPriority					[31] PriorityType OPTIONAL,

messageReference				[32] MessageReference OPTIONAL,

messageSize					[33] INTEGER OPTIONAL,

messageClass					[34] MessageClass OPTIONAL,

sMdeliveryReportRequested	[35] SMdeliveryReportRequested OPTIONAL,

messageClassTokenText		[36] UTF8String OPTIONAL,

userRoamerInOut				[37] RoamerInOut OPTIONAL,

userLocationInformationASN1	[38] UserLocationInformationStructured OPTIONAL

}

--

#### -- Exposure Function API Information

-- corresponds to NEF API Charging information

--

ExposureFunctionAPIInformation	::= SET

{

groupIdentifier					[0] UTF8String OPTIONAL,

-- This UTF8String is based on the string specified in TS 29.571 [249]

-- The string may also be based on AddressString.

aPIDirection					[1] APIDirection OPTIONAL,

aPITargetNetworkFunction		[2] NetworkFunctionInformation OPTIONAL,

aPIResultCode					[3] APIResultCode OPTIONAL,

aPIName							[4] IA5String,

aPIReference					[5] IA5String OPTIONAL,

aPIContent						[6] OCTET STRING OPTIONAL,

externalIndividualIdentifier	[7] InvolvedParty OPTIONAL,

externalGroupIdentifier			[8] ExternalGroupIdentifier OPTIONAL,

internalGroupIdentifier			[9] InternalGroupIdentifier OPTIONAL,

internalIndividualIdentifier	[10] SubscriptionID OPTIONAL,

aPIOperation					[11] APIOperation OPTIONAL,

externalIndividualIdList		[12] SEQUENCE OF ExternalGroupIdentifier OPTIONAL,

internalIndividualIdList		[13] SEQUENCE OF SubscriptionID OPTIONAL

}

--

#### -- Registration Charging Information

--

RegistrationChargingInformation 	::= SET

{

registrationMessagetype				[0] RegistrationMessageType,

userIdentifier						[1] InvolvedParty OPTIONAL,

userEquipmentInfo					[2] SubscriberEquipmentNumber OPTIONAL,

sUPIunauthenticatedFlag 				[3] NULL OPTIONAL,

userRoamerInOut						[4] RoamerInOut OPTIONAL,

userLocationInformation				[5] UserLocationInformation OPTIONAL,

userLocationInfoTime					[6] TimeStamp OPTIONAL, -- This field is not used

-- user location info time is included under UserLocationInformation

uETimeZone 							[7] MSTimeZone OPTIONAL,

rATType								[8] RATType OPTIONAL,

mICOModeIndication					[9] MICOModeIndication OPTIONAL,

smsIndication						[10] SmsIndication OPTIONAL,

taiList								[11] SEQUENCE OF TAI OPTIONAL,

serviceAreaRestriction				[12] ServiceAreaRestriction OPTIONAL,

requestedNSSAI						[13] SEQUENCE OF SingleNSSAI OPTIONAL,

allowedNSSAI							[14] SEQUENCE OF SingleNSSAI OPTIONAL,

rejectedNSSAI						[15] SEQUENCE OF SingleNSSAI OPTIONAL,

pSCellInformation					[16] PSCellInformation OPTIONAL,

fiveGMMCapability					[17] FiveGMMCapability OPTIONAL,

nSSAIMapList							[18] SEQUENCE OF NSSAIMap OPTIONAL,

amfUeNgapId							[19] AmfUeNgapId OPTIONAL,

ranUeNgapId							[20] RanUeNgapId OPTIONAL,

ranNodeId							[21] GlobalRanNodeId OPTIONAL,

userLocationInformationASN1			[22] UserLocationInformationStructured OPTIONAL,

sNPNID								[23] PlmnIdNid OPTIONAL,

aMFIdentifier						[24] AMFID OPTIONAL,

cAGIDList							[25] SEQUENCE OF CagId OPTIONAL,

alternativeNSSAIMap					[26] SEQUENCE OF AlternativeNSSAIMap OPTIONAL,

satelliteAccessIndicator			[27] BOOLEAN OPTIONAL

}

--

#### -- N2 connection charging Information

--

N2ConnectionChargingInformation 	::= SET

{

n2ConnectionMessageType				[0] N2ConnectionMessageType,

userIdentifier						[1] InvolvedParty OPTIONAL,

userEquipmentInfo					[2] SubscriberEquipmentNumber OPTIONAL,

sUPIunauthenticatedFlag 				[3] NULL OPTIONAL,

userRoamerInOut						[4] RoamerInOut OPTIONAL,

userLocationInformation				[5] UserLocationInformation OPTIONAL,

userLocationInfoTime					[6] TimeStamp OPTIONAL, -- This field is not used

-- user location info time is included under UserLocationInformation

uETimeZone 							[7] MSTimeZone OPTIONAL,

rATType								[8] RATType OPTIONAL,

ranUeNgapId							[9] RanUeNgapId OPTIONAL,

ranNodeId							[10] GlobalRanNodeId OPTIONAL,

restrictedRatList					[11] SEQUENCE OF RATType OPTIONAL,

forbiddenAreaList					[12] SEQUENCE OF Area OPTIONAL,

serviceAreaRestriction				[13] ServiceAreaRestriction OPTIONAL,

restrictedCnList						[14] SEQUENCE OF CoreNetworkType OPTIONAL,

allowedNSSAI							[15] SEQUENCE OF SingleNSSAI OPTIONAL,

rrcEstablishmentCause				[16] RrcEstablishmentCause OPTIONAL,

pSCellInformation					[17] PSCellInformation OPTIONAL,

amfUeNgapId							[18] AmfUeNgapId OPTIONAL,

userLocationInformationASN1			[19] UserLocationInformationStructured OPTIONAL,

nSSAIMapList						[20] SEQUENCE OF NSSAIMap OPTIONAL,

aMFIdentifier						[21] AMFID OPTIONAL,

satelliteAccessIndicator			[22] BOOLEAN OPTIONAL

}

--

#### -- Location reporting charging Information

--

LocationReportingChargingInformation 	::= SET

{

locationReportingMessagetype			[0] LocationReportingMessageType,

userIdentifier						[1] InvolvedParty OPTIONAL,

userEquipmentInfo					[2] SubscriberEquipmentNumber OPTIONAL,

sUPIunauthenticatedFlag 				[3] NULL OPTIONAL,

userRoamerInOut						[4] RoamerInOut OPTIONAL,

userLocationInformation				[5] UserLocationInformation OPTIONAL,

userLocationInfoTime					[6] TimeStamp OPTIONAL, -- This field is not used

-- user location info time is included under UserLocationInformation

uETimeZone 							[7] MSTimeZone OPTIONAL,

presenceReportingAreaInfo			[8]	PresenceReportingAreaInfo OPTIONAL,

rATType								[9] RATType OPTIONAL,

pSCellInformation					[10] PSCellInformation OPTIONAL,

userLocationInformationASN1			[11] UserLocationInformationStructured OPTIONAL,

listOfPresenceReportingAreaInformation	[12] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL,

aMFIdentifier						[13] AMFID OPTIONAL,

satelliteAccessIndicator			[14] BOOLEAN OPTIONAL

}

--

#### -- Network Slice Performance and Analytics charging Information

--

NSPAChargingInformation			::= SET

{

singelNSSAI					[0] SingleNSSAI

}

--

#### -- NSM charging Information

--

--

-- See TS 28.541 [254] for more information

--

NSMChargingInformation 	::= SET

{

managementOperation						[0] ManagementOperation OPTIONAL,

iDnetworkSliceInstance					[1] OCTET STRING OPTIONAL,

listOfserviceProfileChargingInformation	[2] SEQUENCE OF ServiceProfileChargingInformation OPTIONAL,

managementOperationStatus				[3]	ManagementOperationStatus OPTIONAL,

operationalState							[4]	OperationalState OPTIONAL,

administrativeState						[5]	AdministrativeState OPTIONAL

}

--

#### -- MMTel charging Information

-- See TS 32.275 [35] for more information

--

MMTelChargingInformation	::= SET

{

supplementaryServices			[0] SEQUENCE OF SupplService OPTIONAL

}

--

#### -- IMS charging Information

-- See TS 32.260 [20] for more information

--

IMSChargingInformation	::= SET

{

eventType								[0] SIPEventType OPTIONAL,

iMSNodeFunctionality					[1] IMSNodeFunctionality OPTIONAL,

roleOfNode								[2] Role-of-Node OPTIONAL,

userIdentifier							[3] InvolvedParty OPTIONAL,

userEquipmentInfo						[4] SubscriberEquipmentNumber OPTIONAL,

userLocationInfo						[5] UserLocationInformation OPTIONAL,

ueTimeZone								[6] MSTimeZone OPTIONAL,

threeGPPPSDataOffStatus					[7] ThreeGPPPSDataOffStatus OPTIONAL,

iSUPCause								[8] ISUPCause OPTIONAL,

controlPlaneAddress						[9] NodeAddress OPTIONAL,

vlrNumber								[10] MSCAddress OPTIONAL,

mscAddress								[11] MSCAddress OPTIONAL,

userSessionID							[12] Session-Id OPTIONAL,

outgoingSessionID						[13] Session-Id OPTIONAL,

sessionPriority							[14] SessionPriority OPTIONAL,

callingPartyAddresses					[15] ListOfInvolvedParties OPTIONAL,

calledPartyAddress						[16] InvolvedParty OPTIONAL,

numberPortabilityRouting				[17] NumberPortabilityRouting OPTIONAL,

carrierSelectRoutingInformation			[18] CarrierSelectRouting OPTIONAL,

alternateChargedPartyAddress			[19] UTF8String OPTIONAL,

requestedPartyAddresses					[20] ListOfInvolvedParties OPTIONAL,

calledAssertedIdentities				[21] ListOfInvolvedParties OPTIONAL,

calledIdentityChanges					[22] SEQUENCE OF CalledIdentityChange OPTIONAL,

associatedURIs							[23] ListOfInvolvedParties OPTIONAL,

timeStamps								[24] TimeStamp OPTIONAL,

applicationServerInformation			[25] SEQUENCE OF ApplicationServersInformation OPTIONAL,

interOperatorIdentifiers				[26] SEQUENCE OF InterOperatorIdentifiers OPTIONAL,

imsChargingIdentifier					[27] IMS-Charging-Identifier OPTIONAL,

relatedICID								[28] IMS-Charging-Identifier OPTIONAL,

relatedICIDGenerationNode				[29] NodeAddress OPTIONAL,

transitIOIList							[30] TransitIOILists OPTIONAL,

earlyMediaDescription					[31] SEQUENCE OF Early-Media-Components-List OPTIONAL,

sdpSessionDescription					[32] SEQUENCE OF UTF8String OPTIONAL,

sdpMediaComponent						[33] SEQUENCE OF SDP-Media-Component OPTIONAL,

servedPartyIPAddress					[34] ServedPartyIPAddress OPTIONAL,

serverCapabilities						[35] S-CSCF-Information OPTIONAL,

trunkGroupID							[36] TrunkGroupID OPTIONAL,

bearerService							[37] TransmissionMedium OPTIONAL,

imsServiceId							[38] Service-Id OPTIONAL,

messageBodies							[39] SEQUENCE OF MessageBody OPTIONAL,

accessNetworkInformation				[40] SEQUENCE OF UTF8String OPTIONAL,

additionalAccessNetworkInformation		[41] UTF8String OPTIONAL,

cellularNetworkInformation				[42] UTF8String OPTIONAL,

accessTransferInformation				[43] SEQUENCE OF AccessTransferInformation OPTIONAL,

accessNetworkInfoChange					[44] SEQUENCE OF AccessNetworkInfoChange OPTIONAL,

imsCommunicationServiceID				[45] IMSCommunicationServiceIdentifier OPTIONAL,

imsApplicationReferenceID				[46] UTF8String OPTIONAL,

causeCode								[47] INTEGER OPTIONAL,

reasonHeaders							[48] ListOfReasonHeader OPTIONAL,

initialIMSChargingIdentifier			[49] IMS-Charging-Identifier OPTIONAL,

nniInformation							[50] SEQUENCE OF NNI-Information OPTIONAL,

fromAddress								[51] UTF8String OPTIONAL,

imsEmergencyIndicator					[52] NULL OPTIONAL,

imsVisitedNetworkIdentifier				[53] UTF8String OPTIONAL,

sipRouteHeaderReceived					[54] UTF8String OPTIONAL,

sipRouteHeaderTransmitted				[55] UTF8String OPTIONAL,

tadIdentifier							[56] TADIdentifier OPTIONAL,

feIdentifierList						[57] FEIdentifierList OPTIONAL

}

--

#### -- Edge Enabling Infrastructure Resource Usage Charging Information

--

EdgeInfrastructureUsageChargingInformation	::= SET

{

meanVirtualCPUUsage				[0] REAL OPTIONAL,

meanVirtualMemoryUsage			[1] REAL OPTIONAL,

meanVirtualDiskUsage			[2] REAL OPTIONAL,

durationStartTime				[3] TimeStamp OPTIONAL,

durationEndTime					[4] TimeStamp OPTIONAL,

measuredInBytes					[5]	INTEGER OPTIONAL,

measuredOutBytes				[6]	INTEGER OPTIONAL

}

--

#### -- EAS Deployment Charging Information

--

EASDeploymentChargingInformation	::= SET

{

eASDeploymentRequirements			[0] EASDeploymentRequirements OPTIONAL,

lCMStartTime						[1] TimeStamp,

lCMEndTime							[2] TimeStamp,

lCMEventType						[3]	ManagementOperation OPTIONAL,

satelliteBackhaulInformation		[4] SatelliteBackhaulInformation OPTIONAL

}

--

#### -- Prose Charging Information

-- See TS 32.277 [34] for more information

-- See clause 5.2.4.7 for ProSe CDR types definition

--

ProseChargingInformation		::= SET

{

announcingPlmnID						[0] PLMN-Id OPTIONAL,

announcingUeHplmnIdentifier				[1] PLMN-Id OPTIONAL,

announcingUeVplmnIdentifier				[2] PLMN-Id OPTIONAL,

monitoringUeHplmnIdentifier				[3] PLMN-Id OPTIONAL,

monitoringUeVplmnIdentifier				[4] PLMN-Id OPTIONAL,

discovererUeHplmnIdentifier				[5] PLMN-Id OPTIONAL,

discovererUeVplmnIdentifier				[6] PLMN-Id OPTIONAL,

discovereeUeHplmnIdentifier				[8] PLMN-Id OPTIONAL,

discovereeUeVplmnIdentifier				[9] PLMN-Id OPTIONAL,

monitoredPlmnIdentifier					[10] PLMN-Id OPTIONAL,

proseApplicationID						[11] UTF8String OPTIONAL,

applicationID							[12] UTF8String OPTIONAL,

applicationSpecificDataList				[13] SEQUENCE OF AppSpecificData,

proseFunctionality						[14] ProseFunctionality OPTIONAL,

proseEventType							[15] ProSeEventType OPTIONAL,

directDiscoveryModel					[16] UTF8String OPTIONAL,

validityPeriod							[17] INTEGER OPTIONAL,

roleOfUE								[18] ProSeUERole OPTIONAL,

proseRequestTimestamp					[19] TimeStamp OPTIONAL,

pC3ProtocolCause						[20] INTEGER OPTIONAL,

monitoringUEIdentifier					[21] SubscriptionID OPTIONAL,

requestedPLMNIdentifier					[22] PLMN-Id OPTIONAL,

timeWindow								[23] INTEGER OPTIONAL,

rangeClass								[24] RangeClass OPTIONAL,

proximityAlertIndication				[25] ProximityAlertIndication OPTIONAL,

proximityAlertTimestamp					[26] TimeStamp OPTIONAL,

proximityCancellationTimestamp			[27] TimeStamp OPTIONAL,

relayIPAddress							[28] IPAddress OPTIONAL,

proseUEToNetworkRelayUEID				[29] OCTET STRING OPTIONAL,

proseDestinationLayer2ID				[30] OCTET STRING OPTIONAL,

pFIContainerInformation					[31] SEQUENCE OF PFIContainerInformation OPTIONAL,

transmissionDataContainer				[32] SEQUENCE OF ChangeOfProSeCondition OPTIONAL,

receptionDataContainer					[33] SEQUENCE OF ChangeOfProSeCondition OPTIONAL

}

--

#### -- MMS Charging Information

--

MMSChargingInformation	::= SET

{

mMOriginatorInfo			[1] MMOriginatorInfo OPTIONAL,

mMRecipientInfoList		[2] SEQUENCE OF MMRecipientInfo OPTIONAL,

userLocationInformation		[3] UserLocationInformation OPTIONAL,

uETimeZone 					[4] MSTimeZone OPTIONAL,

rATType						[5] RATType OPTIONAL,

correlationInformation		[6] UTF8String OPTIONAL,

submissionTime				[7] TimeStamp OPTIONAL,

mMContentType				[8] MMContentType OPTIONAL,

mMPriority					[9] PriorityType OPTIONAL,

messageID					[10] UTF8String OPTIONAL,

messageType					[11] UTF8String OPTIONAL,

messageSize					[12] INTEGER OPTIONAL,

messageClass				[13] UTF8String OPTIONAL,

deliveryReportRequested		[14] BOOLEAN OPTIONAL,

readReplyReportRequested	[15] BOOLEAN OPTIONAL,

applicID					[16] UTF8String OPTIONAL,

replyApplicID				[17] UTF8String OPTIONAL,

auxApplicInfo				[18] UTF8String OPTIONAL,

contentClass				[19] UTF8String OPTIONAL,

dRMContent					[20] BOOLEAN OPTIONAL,

adaptations					[21] BOOLEAN OPTIONAL,

vasID						[22] UTF8String OPTIONAL,

vaspID						[23] UTF8String OPTIONAL

}

--

#### -- NSACF Charging Information

--

NSACFChargingInformation 	::= SET

{

nSACFChargingIndicator							[1] BOOLEAN OPTIONAL

}

--

#### -- TSN charging Information

-- See TS 32.282 [43] for more information

--

TSNChargingInformation	::= SET

{

dNN										[0] DataNetworkNameIdentifier OPTIONAL,

sNSSAI									[1] SingleNSSAI OPTIONAL,

internalGroupIdentifier					[2] InternalGroupIdentifier OPTIONAL,

externalIndividualIdList				[3] SEQUENCE OF InvolvedParty OPTIONAL,

fiveGSBridgeInformation					[4] FiveGSBridgeInformation OPTIONAL,

tSNQoSInformation						[5] TSNQoSInformation OPTIONAL,

tSCAssistanceInformation				[6] TSCAssistanceInformation OPTIONAL,

timeSynchronizationInformation			[7] TimeSynchronizationInformation OPTIONAL

}

--

#### -- MBS Session charging Information

--

MbsSessionChargingInformation ::= SET

{

mBSSessionID 				[1] MbsSessionId,

mBSServiceType				[2] MbsServiceType,

serviceArea					[3] ServiceArea OPTIONAL,

mBSStartTime 				[4] TimeStamp OPTIONAL,

mBSStopTime					[5] TimeStamp OPTIONAL,

servingNetworkFunctionID	[6] SEQUENCE OF ServingNetworkFunctionID OPTIONAL

}

--

#### -- Inter-CHF Information

--

--

-- See TS 32.255 [15] and TS 32.256 [16] for more information

--

InterCHFInformation	::= SET

{

remoteCHFResource		[0] UTF8String OPTIONAL,

originalNFConsumerId	[1] NetworkFunctionInformation OPTIONAL,

mBSSessionActivityStatus	[7] MbsSessionActivityStatus

}

--

#### -- NSSAA Charging Information

--

NSSAAChargingInformation 	::= SET

{

nSSAAMessageType				[0] NSSAAMessageType,

userIdentification				[1] InvolvedParty OPTIONAL,

aAAPAddress 					[2] NodeAddress OPTIONAL,

aAASAddress 					[3] NodeAddress OPTIONAL,

eAPIDResponse 					[4] EAPIDResponse OPTIONAL,

eAPAuthStatus 					[5] EAPAuthStatus OPTIONAL,

aMFIdentifier					[6] AMFID OPTIONAL

}

--

#### -- CHF CHARGING TYPES

--

--

#### -- A

--

AFChargingID	::= UTF8String

--

-- See 3GPP TS 29.571 [249] for details.

--

AffinityAntiAffinity	::= SEQUENCE

{

affinityEAS				[0] SEQUENCE OF UTF8String OPTIONAL,

antiAffinityEAS			[1] SEQUENCE OF UTF8String OPTIONAL

}

AdministrativeState 	::= ENUMERATED

{

lOCKED		 (0),

uNLOCKED 	 (1),

sHUTTINGDOWN (2)

}

AllocatedUnit 	::= SEQUENCE

{

quotaManagementIndicator			[0] BOOLEAN OPTIONAL,

triggers							[1] SEQUENCE OF Trigger OPTIONAL,

triggerTimeStamp					[2] TimeStamp OPTIONAL,

localSequenceNumber					[3] LocalSequenceNumber OPTIONAL,

nSACFContainerInformation			[4] NSACFContainerInformation OPTIONAL

}

AllocationRetentionPriority	::= SEQUENCE

{

priorityLevel 			[1] INTEGER,

preemptionCapability	[2] PreemptionCapability,

preemptionVulnerability	[3] PreemptionVulnerability

}

AlternativeNSSAIMap		::= SEQUENCE

{

snssai						[0] SingleNSSAI,

alternativeSnssai			[1] SingleNSSAI

}

AmfUeNgapId	::= INTEGER

APIOperation	::= SEQUENCE

{

name			[1] UTF8String,

description		[2] UTF8String

}

APIResultCode	::= INTEGER

-- See specific API for more information

Area	::= SEQUENCE

{

tacs 		[0] SEQUENCE OF TAC OPTIONAL,

areaCode	[1] OCTET STRING OPTIONAL

}

ATSSSCapability	::= ENUMERATED

{

aTSSS-LL					(0),

mPTCP-ATSS-LL				(1),

mPTCP-ATSS-LL-ASModeUL		(2),

mPTCP-ATSS-LL-ExSDModeUL	(3),

mPTCP-ATSS-LL-ASModeDLUL	(4)

}

AuthorizedQoSInformation	::= SEQUENCE

-- See TS 32.291 [58] for more information

{

fiveQi				[1] INTEGER OPTIONAL,

aRP					[2] AllocationRetentionPriority OPTIONAL,

priorityLevel 		[3] INTEGER OPTIONAL,

averWindow			[4] INTEGER OPTIONAL,

maxDataBurstVol		[5] INTEGER OPTIONAL

}

--

#### -- B

--

--

#### -- C

--

CagId		::= OCTET STRING

--

-- See 3GPP TS 29.571 [249] for details

--

ChargingSessionIdentifier	::= OCTET STRING

-- See 3GPP TS 32.290 [57] for details.

ClockQuality					::= SEQUENCE

-- See 3GPP TS 29.571 [249] for details

{

traceabilityToGnss						[1] BOOLEAN OPTIONAL,

traceabilityToUtc						[2] BOOLEAN OPTIONAL,

frequencyStability						[3] INTEGER OPTIONAL,

clockAccuracy							[4] OCTET STRING (SIZE(2)) OPTIONAL

}

CoreNetworkType 		::= ENUMERATED

{

fiveGC 		(0),

ePC			(1)

}

--

#### -- D

--

DelayToleranceIndicator   ::= ENUMERATED

{

dTSupported 			(0),

dTNotSupported			(1)

}

DNNSelectionMode	::= ENUMERATED

-- See Information Elements TS 29.502 [250] for more information

{

uEorNetworkProvidedSubscriptionVerified				(0),

uEProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

--

#### -- E

--

EAPAuthStatus		::= ENUMERATED

{

eAPSuccess		(0),

eAPFailure		(1),

pending			(2)

}

EAPIDResponse		::= OCTET STRING

-- See 3GPP TS 28.538 [256] for details

EASDeploymentRequirements	::= SEQUENCE

{

requiredEASservingLocation			[0] ServingLocation OPTIONAL,

softwareImageInfo					[1] SoftwareImageInfo OPTIONAL,

affinityAntiAffinity				[2] AffinityAntiAffinity OPTIONAL,

serviceContinuity					[3] BOOLEAN OPTIONAL,

virtualResource						[4] VirtualResource OPTIONAL

}

ExternalGroupIdentifier		::= UTF8String

-- See 3GPP TS 29.571 [249] for details

EstablishedConnectionInfo ::= SEQUENCE

{

uPFIDs			[0] SEQUENCE OF NetworkFunctionName OPTIONAL,

ranNodeIDs		[1] SEQUENCE OF GlobalRanNodeId OPTIONAL

}

EnhancedDiagnostics5G					::= SEQUENCE

{

rANNASRelCause						[0] SEQUENCE OF RANNASRelCause

}

--

#### -- F

--

FiveGLANTypeService			::= SEQUENCE

{

internalGroupIdentifier		[1] UTF8String

}

FiveGMMCapability	::= OCTET STRING

-- See 3GPP TS 29.571 [249] for details

FiveGMmCause	::= INTEGER

-- See 3GPP TS 29.571 [249] for details

FiveGMulticastService	::= SEQUENCE

{

mBSSessionIDList		[0] SEQUENCE OF MbsSessionId

}

FiveGQoSInformation	::= SEQUENCE

-- See TS 32.291 [58] for more information

{

fiveQi					[1] INTEGER OPTIONAL,

aRP						[2] AllocationRetentionPriority OPTIONAL,

qoSNotificationControl	[3] BOOLEAN OPTIONAL,

reflectiveQos			[4] BOOLEAN OPTIONAL,

maxbitrateUL				[5] Bitrate OPTIONAL,

maxbitrateDL				[6] Bitrate OPTIONAL,

guaranteedbitrateUL		[7] Bitrate OPTIONAL,

guaranteedbitrateDL		[8] Bitrate OPTIONAL,

priorityLevel 			[9] INTEGER OPTIONAL,

averWindow				[10] INTEGER OPTIONAL,

maxDataBurstVol			[11] INTEGER OPTIONAL,

maxPacketLossRateDL 		[12] INTEGER OPTIONAL,

maxPacketLossRateUL 		[13] INTEGER OPTIONAL

}

FiveGSBridgeInformation	::= SEQUENCE

{

bridgeId							[1] INTEGER,

nWTTPortNumber						[2] INTEGER OPTIONAL,

dSTTPortNumber						[3] INTEGER OPTIONAL

}

FiveGSmCause	::= INTEGER

-- See 3GPP TS 29.571 [249] for details

--

#### -- G

--

--

#### -- H

--

HFCNodeId		::= UTF8String

--

-- See 3GPP TS 29.571 [249] for details

--

--

#### -- I

--

IMSNodeFunctionality	::= ENUMERATED

{

iMS-GWF				(0),

aS					(1),

mRFC				(2)

}

IMSSessionInformation ::= SEQUENCE

{

callerInformation	[0] SEQUENCE OF InvolvedParty OPTIONAL,

calleeInformation	[1] CalleePartyInformation OPTIONAL

}

IMSTrigger			::= INTEGER

{

--Initial

sIPInvite									(1),

--Change of charging conditions

sIPReInviteOrUpdate							(2),

sIP2xxAcknowledging							(3),

sIP1xxProvisionalResponse					(4),

sIP4xx5xxOr6xxResponse						(5),

otherSipMessage								(6),

--CHF Limit

expiryOfTimeLimit							(7),

expiryOfLimitOfNumOfChConditionChanges		(8),

--Quota management

timeThresholdReached						(9),

timeQuotaExhausted							(10),

unitQuotaExhausted							(11),

expiryOfQuotaValidityTime					(12),

expiryOfQuotaHoldingTime					(13),

reAuthorizationReqByChf						(14),

--Other

managementIntervention						(15),

--Termination

sIPByeMessage

sIP2xxAcknowledgingASipBye					(16),

abortingASipSessionSetup					(17),

sIP3xxFinalOrRedirectionResponse			(18),

sIP4xx5xxOr6xxFinalResponse					(19)

}

IncompleteCDRIndication	::= SEQUENCE

-- The values are TRUE if the corresponding message was lost, FALSE if it is not lost

-- and not included if the status is unknown

{

initialLost		[0] BOOLEAN OPTIONAL,	-- Initial was lost

updateLost		[1] BOOLEAN OPTIONAL,	-- An Update was lost,

terminationLost	[2] BOOLEAN OPTIONAL	-- Termination was lost

}

--

#### -- J

--

--

#### -- K

--

--

#### -- L

--

LocationReportingMessageType		::= INTEGER

--

#### -- M

--

ManagementOperation 	::= ENUMERATED

{

createMOI 			(0),

modifyMOIAttributes	(1),

deleteMOI			(2),

notifyMOICreation	(3),

notifyMOIAttrChange	(4),

notifyMOIDeletion	(5)

}

ManagementOperationStatus 	::= ENUMERATED

{

oPERATION-SUCCEEDED	(0),

oPERATION-FAILED	(1)

}

MbsContainerInformation ::= SEQUENCE

{

timeOfFirstUsage			[0] TimeStamp OPTIONAL,

timeOfLastUsage				[1] TimeStamp OPTIONAL,

qoSInformation				[2] FiveGQoSInformation OPTIONAL,

establishedConnectionInfo	[3] EstablishedConnectionInfo OPTIONAL

}

MBSMFTrigger	::= INTEGER

{

startOfMBSSession							(1),

-- Change of Charging conditions

connectionEstablishedWithNGRAN				(100),

connectionReleasedWithNGRAN					(101),

connectionEstablishedWithUPF				(102),

tariffTimeChange							(103),

connectionReleasedWithUPF					(104),

mBSSessionContextUpdate						(105),

mBSSessionActivityStatusChangetoActive		(106),

mBSSessionActivityStatusChangetoInactive	(107),

-- Limit per MBS session

mBSSessionExpiryDataTimeLimit				(200),

mBSSessionExpiryDataVolumeLimit				(201),

mBSSessionExpiryChargingConditionChanges	(202),

-- Quota management

timeThresholdReached						(400),

timeQuotaExhausted							(401),

-- Others

endOfMBSSession								(500)

}

MbsServiceArea ::= SEQUENCE

-- See 3GPP TS 29.571 [249] for details

{

ncgiList	[0] SEQUENCE OF NcgiTai OPTIONAL,

taiList		[1] SEQUENCE OF TAI OPTIONAL

}

MbsServiceType ::= ENUMERATED

-- See 3GPP TS 29.571 [249] for details

{

multicast (0),

broadcast (1)

}

MbsSessionActivityStatus := ENUMERATED

-- See 3GPP TS 29.571 [249] for details

{

active 			(0),

inactive		(1)

}

MbsSessionId	::= SEQUENCE

-- See 3GPP TS 29.571 [249] for details.

{

tMGI			[0] TMGI OPTIONAL,

ssm				[1] Ssm OPTIONAL,

nid				[2] Nid OPTIONAL

}

MbsDeliveryMethod	::= ENUMERATED

{

shared 			(0),

individual		(1)

}

MnSConsumerIdentifier		::= OCTET STRING

MAPDUSessionIndicator	::= ENUMERATED

{

mAPDURequest 					(0),

mAPDUNetworkUpgradeAllowed		(1)

}

MAPDUSessionInformation	::= SEQUENCE

{

mAPDUSessionIndicator			[0] MAPDUSessionIndicator OPTIONAL,

aTSSSCapability					[1] ATSSSCapability OPTIONAL

}

MAPDUSteeringFunctionality	::= ENUMERATED

{

mPTCP 		(0),

aTSSSLL		(1)

}

MAPDUSteeringMode	::= SEQUENCE

{

steerModeValue			[0] SteerModeValue OPTIONAL,

active					[1] AccessType OPTIONAL,

standby					[2] AccessType OPTIONAL,

threegLoad				[3] INTEGER OPTIONAL,

prioAcc					[4] AccessType OPTIONAL

}

MICOModeIndication 		::= ENUMERATED

{

mICOMode 			(0),

noMICOMode			(1)

}

MMAddContentInfo	::= SEQUENCE

{

typeNumber			[0] UTF8String OPTIONAL,

addtypeInfo			[1] UTF8String OPTIONAL,

contentSize			[2] INTEGER OPTIONAL

}

MMContentType	::= SEQUENCE

{

typeNumber			[0] UTF8String OPTIONAL,

addtypeInfo			[1] UTF8String OPTIONAL,

contentSize			[2] INTEGER OPTIONAL,

mmAddContentInfo	[3] SEQUENCE OF MMAddContentInfo OPTIONAL

}

MMOriginatorInfo	::= SEQUENCE

{

originatorIMSI				[0] IMSI OPTIONAL,

originatorMSISDN			[1] MSISDN OPTIONAL,

originatorOtherAddresses	[2] SEQUENCE OF SMAddressInfo OPTIONAL

}

MMRecipientInfo	::= SEQUENCE

{

recipientIMSI				[0] IMSI OPTIONAL,

recipientMSISDN				[1] MSISDN OPTIONAL,

recipientOtherAddresses		[2] SEQUENCE OF SMAddressInfo OPTIONAL

}

MobilityLevel	::= ENUMERATED

{

stationary			(0),

nomadic				(1),

restrictedMobility	(2),

fullyMobility		(3)

}

MultipleUnitUsage 		::= SEQUENCE

{

ratingGroup							[0] RatingGroupId,

usedUnitContainers					[1] SEQUENCE OF UsedUnitContainer OPTIONAL,

uPFID								[2] NetworkFunctionName OPTIONAL,

multihomedPDUAddress				[3] PDUAddress OPTIONAL,

allocatedUnit			 			[4] AllocatedUnit OPTIONAL,

mBUPFID								[5] NetworkFunctionName OPTIONAL

}

MultipleQFIContainer 		::= SEQUENCE

{

qosFlowId								[0] QoSFlowId OPTIONAL,

triggers								[1] SEQUENCE OF Trigger OPTIONAL,

triggerTimeStamp						[2] TimeStamp OPTIONAL,

dataTotalVolume							[3] DataVolumeOctets OPTIONAL,

dataVolumeUplink						[4] DataVolumeOctets OPTIONAL,

dataVolumeDownlink						[5] DataVolumeOctets OPTIONAL,

localSequenceNumber						[6] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage						[8] TimeStamp OPTIONAL,

timeOfLastUsage							[9] TimeStamp OPTIONAL,

qoSInformation							[10] FiveGQoSInformation OPTIONAL,

userLocationInformation					[11] UserLocationInformation OPTIONAL,

uETimeZone	 							[12] MSTimeZone OPTIONAL,

presenceReportingAreaInfo				[13] PresenceReportingAreaInfo OPTIONAL,

rATType									[14] RATType OPTIONAL,

reportTime								[15] TimeStamp,

servingNetworkFunctionID				[16] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

threeGPPPSDataOffStatus					[17] ThreeGPPPSDataOffStatus OPTIONAL,

threeGPPChargingID						[18] ChargingID OPTIONAL,

diagnostics						[19] Diagnostics OPTIONAL,

extensionDiagnostics					[20] EnhancedDiagnostics OPTIONAL,

qoSCharacteristics						[21] QoSCharacteristics OPTIONAL,

time									[22] CallDuration OPTIONAL,

userLocationInformationASN1				[23] UserLocationInformationStructured OPTIONAL,

listOfPresenceReportingAreaInformation	[39] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL

}

--

#### -- N

--

N2ConnectionMessageType		::= INTEGER

NcgiTai ::= SEQUENCE

-- See 3GPP TS 29.571 [249] for details

{

tai 		[0] TAI,

cellList 	[1] SEQUENCE OF Ncgi

}

NSACFContainerInformation 		::= SEQUENCE

{

numberOfUEs 					[0] INTEGER OPTIONAL,

numberOfPDUs					[1] INTEGER OPTIONAL

}

NSACFTrigger			::= INTEGER

{

--Initial

nSACThresholdInitial						(1),

--Change of charging conditions

nSACThresholdUpwardsReached					(2),

nSACThresholdUpwardsCrossed					(3),

nSACThresholdDownwardsCrossed				(4),

--Quota management

nSACQuotaThreshold							(5),

nSACQuotaExhausted							(6),

nSACValidityTime							(7),

nSACQHT										(8),

nSACThresholdTermination					(9),

--Termination

nSTermination								(10)

}

NSSAAMessageType		::= ENUMERATED

{

authenticate					(0),

reAuthenticationNotification	(1),

revocationNotification			(2)

}

NetworkAreaInfo	::= SEQUENCE

{

ecgis				[0] SEQUENCE OF Ecgi OPTIONAL,

ncgis				[1] SEQUENCE OF Ncgi OPTIONAL,

gRanNodeIds			[2] SEQUENCE OF GlobalRanNodeId OPTIONAL,

tais				[3] SEQUENCE OF TAI OPTIONAL

}

NetworkFunctionInformation	::= SEQUENCE

{

networkFunctionality					[0] NetworkFunctionality,

networkFunctionName					[1] NetworkFunctionName OPTIONAL,

networkFunctionIPv4Address			[2] IPAddress OPTIONAL,

networkFunctionPLMNIdentifier		[3] PLMN-Id OPTIONAL,

networkFunctionIPv6Address			[4] IPAddress OPTIONAL,

networkFunctionFQDN					[5] NodeAddress OPTIONAL

}

NetworkFunctionName	::= IA5String (SIZE(1..36))

-- Shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [410]

NetworkFunctionality	::= ENUMERATED

{

cHF				(0),

-- CHF is applicable in two scenarios: inter-CHF communication and failure cases

sMF				(1),

-- SMF is applicable in two scenarios: as NF consumer of CHF services, and as API Target NF

-- in NEF charging

aMF				(2),

-- AMF is applicable in two scenario: as NF consumer of CHF services, and as API Target NF

-- in NEF charging

sMSF			(3),

sGW				(4),

-- SGW is only applicable for interworking with EPC scenario

-- when UE is connected to P-GW+SMF via EPC

iSMF			(5),

ePDG			(6),

-- ePDG is only applicable for interworking with EPC scenario

-- when UE is connected to P-GW+SMF via EPC/ePDG

cEF				(7),

nEF			(8),

pGWCSMF			(9),

mnS-Producer 	(10),

sGSN			(11),

-- SGSN is only applicable when UE is connected to SMF+PGW-C via GERAN/UTRAN

fiveGDDNMF		(12),

vSMF			(13),

-- vSMF may be used instead of sMF in roaming scenarios}

iMS-Node		(14),

eES				(15),

mMS-Node		(16),

pCF				(17),

-- PCF is applicable only as API Target NF in NEF charging

uDM				(18),

-- UDM is applicable only as API Target NF in NEF charging

uPF				(19),

-- UPF is applicable only as API Target NF in NEF charging

tSN-AF			(20),

tSNTSF			(21),

mB-SMF			(22)

}

NgApCause	::= SEQUENCE

-- See 3GPP TS 29.571 [249] for details.

{

group			[0] INTEGER,

value			[1] INTEGER

}

NGRANSecondaryRATType	::= OCTET STRING

-- "NR" or "EUTRA"

NGRANSecondaryRATUsageReport	::= SEQUENCE

{

nGRANSecondaryRATType			[0] NGRANSecondaryRATType OPTIONAL,

qosFlowsUsageReports			[1] SEQUENCE OF QosFlowsUsageReport OPTIONAL

}

NsiLoadLevelInfo		::= SEQUENCE

-- See 3GPP TS 29.520 [233] for details

{

loadLevelInformation				[0] INTEGER OPTIONAL,

snssai								[1] SingleNSSAI OPTIONAL,

nsiId								[2] OCTET STRING OPTIONAL

}

NSPAContainerInformation		::= SEQUENCE

{

--	latency								[0] INTEGER OPTIONAL,

--	throughput							[1] Throughput OPTIONAL,

--	maximumPacketLossRate				[3] UTF8String OPTIONAL,

serviceExperienceStatisticsData		[4] ServiceExperienceInfo OPTIONAL,

numberOfPDUSessions					[5] INTEGER OPTIONAL,

numberOfRegisteredSubscribers		[6] INTEGER OPTIONAL,

loadLevel							[7] NsiLoadLevelInfo OPTIONAL,

uplinkLatency						[8] INTEGER OPTIONAL,

downlinkLatency						[9] INTEGER OPTIONAL,

uplinkThroughput						[10] Throughput OPTIONAL,

downlinkThroughput					[11] Throughput OPTIONAL,

maximumPacketLossRateUL				[12] INTEGER OPTIONAL,

maximumPacketLossRateDL				[13] INTEGER OPTIONAL

}

NSSAIMap		::= SEQUENCE

{

servingSnssai						[0] SingleNSSAI,

homeSnssai							[1] SingleNSSAI

}

--

#### -- O

--

OperationalState 	::= ENUMERATED

{

eNABLED	(0),

dISABLED(1)

}

--

#### -- P

--

PartialRecordMethod	::= ENUMERATED

{

default			(0),

individual		(1)

}

PDUAddress 	::= SEQUENCE

{

pDUIPv4Address				[0] IPAddress OPTIONAL,

pDUIPv6AddresswithPrefix		[1] IPAddress OPTIONAL,

iPV4dynamicAddressFlag		[2] DynamicAddressFlag OPTIONAL,

iPV6dynamicPrefixFlag		[3] DynamicAddressFlag OPTIONAL,

additionalPDUIPv6Prefixes	[4]	SEQUENCE OF IPAddress OPTIONAL

}

PDUContainerInformation 		::= SEQUENCE

{

chargingRuleBaseName					[0] ChargingRuleBaseName OPTIONAL,

-- aFCorrelationInformation [1] is replaced by afChargingIdentifier [14]

timeOfFirstUsage						[2] TimeStamp OPTIONAL,

timeOfLastUsage							[3] TimeStamp OPTIONAL,

qoSInformation							[4] FiveGQoSInformation OPTIONAL,

userLocationInformation					[5] UserLocationInformation OPTIONAL,

presenceReportingAreaInfo				[6] PresenceReportingAreaInfo OPTIONAL,

rATType									[7] RATType OPTIONAL,

sponsorIdentity							[8] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity		[9] OCTET STRING OPTIONAL,

servingNetworkFunctionID				[10] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

uETimeZone 								[11] MSTimeZone OPTIONAL,

threeGPPPSDataOffStatus					[12] ThreeGPPPSDataOffStatus OPTIONAL,

qoSCharacteristics						[13] QoSCharacteristics OPTIONAL,

afChargingIdentifier					[14] ChargingID OPTIONAL,

afChargingIdString						[15] AFChargingID OPTIONAL,

mAPDUSteeringFunctionality				[16] MAPDUSteeringFunctionality OPTIONAL,

mAPDUSteeringMode						[17] MAPDUSteeringMode OPTIONAL,

userLocationInformationASN1				[18] UserLocationInformationStructured OPTIONAL,

listOfPresenceReportingAreaInformation	[19] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL,

trafficForwardingWay					[20] TrafficForwardingWay OPTIONAL,

qosMonitoringReport						[21] QosMonitoringReport OPTIONAL,

mBSSessionID							[22] MbsSessionId OPTIONAL,

mBSDeliveryMethod						[23] MbsDeliveryMethod OPTIONAL

}

PDUSessionPairID	::= INTEGER

PDUSessionId 		::= INTEGER (0..255)

-- See 3GPP TS 29.571 [249] for details

PDUSessionType		::= ENUMERATED

-- See 3GPP TS 29.571 [249] for details.

{

iPv4v6			(0),

iPv4			(1),

iPv6			(2),

unstructured	(3),

ethernet		(4)

}

-- See 3GPP TS 29.571 [249] for details.

PFIContainerInformation 		::= SEQUENCE

{

pC5qosFlowId							[0] QoSFlowId OPTIONAL,

timeOfFirstUsage						[1] TimeStamp OPTIONAL,

timeOfLastUsage							[2] TimeStamp OPTIONAL,

qoSInformation							[3] FiveGQoSInformation OPTIONAL,

userLocationInformation					[4] UserLocationInformation OPTIONAL,

uETimeZone	 							[5] MSTimeZone OPTIONAL,

presenceReportingAreaInfo				[6] PresenceReportingAreaInfo OPTIONAL,

reportTime								[7] TimeStamp,

qoSCharacteristics						[8] QoSCharacteristics OPTIONAL

}

PreemptionCapability		::= ENUMERATED

{

nOT-PREEMPT			(0),

mAY-PREEMPT			(1)

}

PreemptionVulnerability		::= ENUMERATED

{

nOT-PREEMPTABLE		(0),

pREEMPTABLE			(1)

}

PC5ContainerInformation		::= SET

{

coverageInfoList				[0] SEQUENCE OF CoverageInfo OPTIONAL,

radioParameterSetInfoList		[1] SEQUENCE OF RadioParameterSetInfo OPTIONAL,

transmitterInfoList				[2] SEQUENCE OF TransmitterInfo OPTIONAL,

timeOfFirstTransmission			[3] TimeStamp OPTIONAL,

timeOfFirstReception			[4] TimeStamp OPTIONAL

}

--

#### -- Q

--

QoSCharacteristics	::= OCTET STRING

-- This data is converted from JSON format of the QoSCharacteristics

-- as described in TS 29.512 [251].

QoSFlowId		::= INTEGER

QosFlowsUsageReport		::= SEQUENCE

{

qosFlowId						[0] QoSFlowId OPTIONAL,

startTime						[1] TimeStamp,

endTime							[2] TimeStamp,

dataVolumeDownlink				[3] DataVolumeOctets,

dataVolumeUplink					[4] DataVolumeOctets

}

QuotaManagementIndicator	::= ENUMERATED

{

onlineCharging				(0),

offlineCharging				(1),

quotaManagementSuspended	(2)

}

QosMonitoringReport		::= SEQUENCE-- The maximum number of elements in the SEQUENCE of ulDelays,dlDelays and rtDelays is 2.

{

ulDelays						 [0] SEQUENCE OF INTEGER OPTIONAL,

dlDelays						 [1] SEQUENCE OF INTEGER OPTIONAL,

rtDelays						 [2] SEQUENCE OF INTEGER OPTIONAL

}

--

#### -- R

--

RanUeNgapId	::= INTEGER

RANNASRelCause 		::= SEQUENCE

-- Mode details are described in TS 29.512[251].

{

ngApCause		[0] NgApCause OPTIONAL,

fivegMmCause	[1] FiveGMmCause OPTIONAL,

fivegSmCause	[2] FiveGSmCause OPTIONAL,

epsCause		[3] RANNASCause OPTIONAL

}

RatingIndicator	::= BOOLEAN

-- Included if the units have been rated.

RegistrationMessageType		::= ENUMERATED

{

initial			(0),

mobility		(1),

periodic		(2),

emergency		(3),

deregistration	(4)

}

RestrictionType		::= ENUMERATED

{

allowedAreas	(0),

notAllowedAreas	(1)

}

RoamingChargingProfile 		::= SEQUENCE

{

roamingTriggers			[0] SEQUENCE OF RoamingTrigger OPTIONAL,

partialRecordMethod		[1] PartialRecordMethod OPTIONAL

}

RoamingTrigger 		::= SEQUENCE

{

trigger					[0] SMFTrigger OPTIONAL,

triggerCategory			[1] TriggerCategory	 OPTIONAL,

timeLimit				[2] CallDuration OPTIONAL,

volumeLimit				[3] DataVolumeOctets OPTIONAL,

maxNbChargingConditions	[4] INTEGER OPTIONAL

}

RrcEstablishmentCause	::= OCTET STRING

RedundantTransmissionType		::= ENUMERATED

{

nonTransmission				 (0),

endToEndUserPlanePaths		 (1),

n3N9						 (2),

transportLayer				 (3)

}

--

#### -- S

--

SatelliteBackhaulInformation 	::= SEQUENCE

-- See 3GPP TS 29.571 [249] for details.

{

satelliteBackhaulCategory			[0] SatelliteBackhaulCategory OPTIONAL,

gEOSatelliteID					[1] UTF8String OPTIONAL

}

SatelliteBackhaulCategory ::= ENUMERATED

{

gEO   				(0),

mEO 				(1),

lEO					(2),

oTHERSAT 			(3),

dYNAMICGEO 			(4),

dYNAMICMEO			(5),

dYNAMICLEO 			(6),

dYNAMICOTHERSAT 	(7),

nONSATELLITE		(8)

}

ServiceArea ::= SEQUENCE

{

mBSServiceArea	[0] MbsServiceArea OPTIONAL,

uPFIDs			[1] SEQUENCE OF NetworkFunctionName OPTIONAL,

ranNodeIDs		[2] SEQUENCE OF GlobalRanNodeId OPTIONAL

}

ServiceAreaRestriction	::= SEQUENCE

-- See 3GPP TS 29.571 [249] for details.

{

restrictionType					[0] RestrictionType OPTIONAL,

areas							[1] SEQUENCE OF Area OPTIONAL,

maxNumOfTAs						[2] INTEGER OPTIONAL,

maxNumOfTAsForNotAllowedAreas	[3] INTEGER OPTIONAL

}

-- See 3GPP TS 29.571 [249] for details.

ServiceExperienceInfo	::= SEQUENCE

-- See 3GPP TS 29.520 [233] for details

{

svcExprc						[0] SvcExperience OPTIONAL,

svcExprcVariance				[1] INTEGER OPTIONAL,

snssai						[2] SingleNSSAI OPTIONAL,

appId						[3] OCTET STRING OPTIONAL,

confidence					[4] INTEGER OPTIONAL,

dnn							[5] DataNetworkNameIdentifier OPTIONAL,

networkArea					[6] NetworkAreaInfo OPTIONAL,

nsiId						[7] OCTET STRING OPTIONAL,

ratio						[8] INTEGER OPTIONAL

}

ServiceProfileChargingInformation 	::= SET

-- attributes of the service profile: see TS 28.541 [254]

{

serviceProfileIdentifier				[0] OCTET STRING OPTIONAL,

sNSSAIList							[1] SEQUENCE OF SingleNSSAI OPTIONAL,

sST			 						[2] SliceServiceType OPTIONAL,

latency								[3] INTEGER OPTIONAL,

availability							[4]	INTEGER OPTIONAL,

resourceSharingLevel					[5] SharingLevel OPTIONAL,

jitter								[6]	INTEGER OPTIONAL,

reliability							[7] OCTET STRING OPTIONAL,

maxNumberofUEs 						[8] INTEGER OPTIONAL,

coverageArea 						[9] OCTET STRING OPTIONAL,

uEMobilityLevel						[10] MobilityLevel OPTIONAL,

delayToleranceIndicator 				[11] DelayToleranceIndicator OPTIONAL,

dLThroughtputPerSlice				[12] Throughput OPTIONAL,

dLThroughtputPerUE					[13] Throughput OPTIONAL,

uLThroughtputPerSlice				[14] Throughput OPTIONAL,

uLThroughtputPerUE					[15] Throughput OPTIONAL,

maxNumberofPDUsessions 				[16] INTEGER OPTIONAL,

kPIsMonitoringList 					[17] OCTET STRING OPTIONAL,

supportedAccessTechnology			[18] INTEGER OPTIONAL,

v2XCommunicationMode 				[19] V2XCommunicationModeIndicator OPTIONAL,

addServiceProfileChargingInfo		[100] OCTET STRING OPTIONAL

}

ServingLocation	::= SEQUENCE

{

geographicalLocation				[0] SEQUENCE OF GeographicalLocation OPTIONAL,

topologicalLocation					[1] TopologicalLocation OPTIONAL

}

ServingNetworkFunctionID	::= SEQUENCE

{

servingNetworkFunctionInformation	[0] NetworkFunctionInformation,

aMFIdentifier						[1] AMFID OPTIONAL

}

SessionAMBR	::= SEQUENCE

{

ambrUL				[1] Bitrate,

ambrDL				[2] Bitrate

}

SharingLevel	::= ENUMERATED

{

sHARED			(0),

nON-SHARED		(1)

}

SIPEventType	::= SEQUENCE

{

sIPMethod              		[0] SIP-Method OPTIONAL,

eventHeader					[1] INTEGER OPTIONAL,

expiresHeader			[2] UTF8String OPTIONAL

}

SMdeliveryReportRequested ::= ENUMERATED

{

yes		(0),

no		(1)

}

SMFTrigger				::= INTEGER

-- See TS 32.255 [15] for details.

{

startOfPDUSession							(1),

startOfServiceDataFlowNoSession				(2),

-- Change of Charging conditions

qoSChange									(100),

userLocationChange							(101),

servingNodeChange							(102),

presenceReportingAreaChange					(103),

threeGPPPSDataOffStatusChange				(104),

tariffTimeChange							(105),

uETimeZoneChange							(106),

pLMNChange									(107),

rATTypeChange								(108),

sessionAMBRChange							(109),

additionOfUPF								(110),

removalOfUPF 								(111),

insertionOfISMF								(112),

removalOfISMF								(113),

changeOfISMF								(114),

gFBRGuaranteedStatusChange					(115),

additionOfAccess							(116),

removalOfAccess 							(117),

redundantTransmissionChange					(118),

vSMFChange									(119),

sNSSAIReplacement							(120),

joinMulticastMBSSession						(121),

mBSDeliveryMethodChange						(122),

leaveMulticastMBSSession					(123),

satelliteBackhaulCategoryChange				(124),

satelliteBackhaulQoSChange					(125),

gEOSatelliteIDCchange						(126),

-- Limit per PDU session

pDUSessionExpiryDataTimeLimit				(200),

pDUSessionExpiryDataVolumeLimit				(201),

pDUSessionExpiryDataEventLimit				(202),

pDUSessionExpiryChargingConditionChanges	(203),

-- Limit per Rating group

ratingGroupDataTimeLimit					(300),

ratingGroupDataVolumeLimit					(301),

ratingGroupDataEventLimit					(302),

-- Quota management

timeThresholdReached						(400),

volumeThresholdReached						(401),

unitThresholdReached						(402),

timeQuotaExhausted							(403),

volumeQuotaExhausted						(404),

unitQuotaExhausted							(405),

expiryOfQuotaValidityTime					(406),

reAuthorizationRequest						(407),

startOfServiceDataFlowNoValidQuota			(408),

otherQuotaType								(409),

expiryOfQuotaHoldingTime					(410),

startOfSDFAdditionalAccessNoValidQuota		(411),

-- Others

terminationOfServiceDataFlow				(500),

managementIntervention						(501),

unitCountInactivityTime						(502),

endOfPDUSession								(503),

cHFResponseWithSessionTermination			(504),

cHFAbortRequest								(505),

abnormalRelease								(506),

notProvidedBySMF							(507), -- used if not provided by SMF

-- Limit per QoS Flow

qoSFlowExpiryDataTimeLimit					(600),

qoSFlowExpiryDataVolumeLimit				(601),

-- interworking with EPC

eCGIChange									(700),

tAIChange									(701),

handoverCancel								(702),

handoverStart								(703),

handoverComplete							(704),

-- GERAN/UTRAN access

cGI-SAIChange								(705),

rAIChange									(706)

}

-- See TS 32.255 [15] for details.

SMReplyPathRequested	::= ENUMERATED

{

noReplyPathSet 			(0),

replyPathSet			(1)

}

SMServiceType 	::= INTEGER

{

-- 0 to 10 VAS4SMS Short Message, see TS 22.142 [105] for details

contentProcessing					(0),

forwarding							(1),

forwardingMultipleSubscriptions		(2),

filtering 							(3),

receipt								(4),

networkStorage						(5),

toMultipleDestinations				(6),

virtualPrivateNetwork				(7),

autoreply							(8),

personalSignature					(9),

deferredDelivery					(10)

-- 11 to 99	Reserved for 3GPP defined SM services

-- 100 to 199 Vendor specific SM services

}

SmsIndication   ::= ENUMERATED

SmsIndication	::= ENUMERATED

{

sMSSupported 			(0),

sMSNotSupported			(1)

}

SNPNInformation   ::= SET

SNPNInformation		::= SET

{

sNPNID				[0] PlmnIdNid,

accessType			[1] AccessType OPTIONAL,

n3IWFFQDN			[2] NodeAddress OPTIONAL

}

SoftwareImageInfo	::= SEQUENCE

{

minimumDisk				[0] INTEGER OPTIONAL,

minimumRAM				[1] INTEGER OPTIONAL,

swImageRef				[2] UTF8String OPTIONAL,

diskFormat				[3] UTF8String OPTIONAL,

operatingSystem			[4] UTF8String OPTIONAL

}

SSCMode	::= INTEGER

SSCMode		::= INTEGER

-- See 3GPP TS 23.501 [247] for details.

{

sSCMode1				(1),

sSCMode2				(2),

sSCMode3				(3)

}

-- See 3GPP TS 23.501 [247] for details.

Ssm ::= SEQUENCE

Ssm	::= SEQUENCE

-- See 3GPP TS 29.571 [249] for details.

{

sourceIpAddr	[0] IPAddress,

destIpAddr		[1] IPAddress

}

SteerModeValue	::= ENUMERATED

{

activeStandby 		(0),

loadBalancing		(1),

smallestDelay 		(2),

priorityBased 		(3)

}

SubscribedQoSInformation	::= SEQUENCE

-- See TS 32.291 [58] for more information

{

fiveQi				[1] INTEGER OPTIONAL,

aRP					[2] AllocationRetentionPriority OPTIONAL,

priorityLevel 		[3] INTEGER OPTIONAL

}

SvcExperience 	::= SEQUENCE

{

mos					[0] INTEGER OPTIONAL,

upperRange			[1] INTEGER OPTIONAL,

lowerRange			[2] INTEGER OPTIONAL

}

SynchronizationState			::= ENUMERATED

{

locked						(0),

holdover					(1),

freerun						(2)

}

SatelliteBackhaulInformation 	::= SEQUENCE

{

satelliteBackhaulCategory			[0] SatelliteBackhaulCategory OPTIONAL,

gEOSatelliteID					[1] UTF8String OPTIONAL

}

-- See 3GPP TS 29.571 [249] for details.

SatelliteBackhaulCategory ::= ENUMERATED

{

gEO   				(0),

mEO 				(1),

lEO					(2),

oTHERSAT 			(3),

dYNAMICGEO 			(4),

dYNAMICMEO			(5),

dYNAMICLEO 			(6),

dYNAMICOTHERSAT 	(7),

nONSATELLITE		(8)

}

--

#### -- T

--

TenantIdentifier			::= OCTET STRING

Throughput	::= SEQUENCE

{

guaranteedThpt			[0] Bitrate,

maximumThpt				[1] Bitrate

}

TimeDistributionMethod			::= ENUMERATED

{

gPTP				(0),

aSTI				(1)

}

TimeSource			::= ENUMERATED

-- See 3GPP TS 29.571 [249] for details

{

pTP						(0),

gNSS					(1),

atomicClock				(2),

terrestrialRadio		(3),

serialTimeCode			(4),

nTP						(5),

handSet					(6),

other					(7)

}

TimeSynchronizationInformation	::= SEQUENCE

{

distributionMethod							[1] TimeDistributionMethod OPTIONAL,

tSNtimeDomainNumber							[2] INTEGER OPTIONAL,

temporalValidityInformation					[3] CallDuration OPTIONAL,

spatialValidityInformation					[4] SEQUENCE OF TAI OPTIONAL,

timeSynchronizationErrorBudget				[5] INTEGER OPTIONAL,

synchronizationState						[6] SynchronizationState OPTIONAL,

clockQuality								[7] ClockQuality OPTIONAL,

parentTimeSource							[8] TimeSource OPTIONAL

}

TopologicalLocation		::= SEQUENCE

-- See 3GPP TS 29.571 [249] for details

{

cellIdList						[0] SEQUENCE OF Ncgi OPTIONAL,

trackingAreaIdList				[1] SEQUENCE OF TAI OPTIONAL,

servingPLMN						[2] SEQUENCE OF PLMN-Id

}

TrafficForwardingWay	::= ENUMERATED

{

n6				(0),

n19				(1),

localSwitch		(2)

}

Trigger	::= CHOICE

{

sMFTrigger		[0] SMFTrigger,

mBSMFTrigger	[1] MBSMFTrigger,

nSACFTrigger	[2] NSACFTrigger,

iMSTrigger	[3] IMSTrigger }

TriggerCategory	::= ENUMERATED

{

immediateReport		(0),

deferredReport		(1)

}

TSCAssistanceInformation	::= SEQUENCE

{

flowDirection							[1] TSCFlowDirection OPTIONAL,

periodicity								[2] INTEGER OPTIONAL

}

TSCFlowDirection			::= ENUMERATED

{

uplink				(0),

downlink			(1)

}

TSNQoSInformation	::= SEQUENCE

{

priority								[1] INTEGER OPTIONAL,

bridgeDelay								[2] SEQUENCE OF INTEGER OPTIONAL

}

--

#### -- U

--

UsedUnitContainer 		::= SEQUENCE

{

serviceIdentifier					[0] ServiceIdentifier OPTIONAL,

time								[1] CallDuration OPTIONAL,

triggers							[2] SEQUENCE OF Trigger OPTIONAL,

triggerTimeStamp					[3] TimeStamp OPTIONAL,

dataTotalVolume						[4] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[5] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[6] DataVolumeOctets OPTIONAL,

serviceSpecificUnits				[7] INTEGER OPTIONAL,

eventTimeStamp						[8] TimeStamp OPTIONAL,

localSequenceNumber					[9] LocalSequenceNumber OPTIONAL,

ratingIndicator						[10] RatingIndicator OPTIONAL,

pDUContainerInformation				[11] PDUContainerInformation OPTIONAL,

quotaManagementIndicator			[12] BOOLEAN OPTIONAL,

quotaManagementIndicatorExt			[13] QuotaManagementIndicator OPTIONAL,

nSPAContainerInformation			[14] NSPAContainerInformation OPTIONAL,

eventTimeStampExt					[15] SEQUENCE OF TimeStamp OPTIONAL,

pC5ContainerInformation				[16] PC5ContainerInformation OPTIONAL,

mBSContainerInformation				[17] MbsContainerInformation OPTIONAL

}

--

#### -- V

--

VirtualResource	::= SEQUENCE

{

virtualMemory				[0] INTEGER OPTIONAL,

virtualDisk					[1] INTEGER OPTIONAL,

virtualResource				[2] OCTET STRING OPTIONAL

}

V2XCommunicationModeIndicator   ::= ENUMERATED

{

v2XComSupported 			(0),

v2XComNotSupported			(1)

}

--

#### -- W

--

--

#### -- X

--

--

#### -- Y

--

--

#### -- Z

--

.#END

End of changes