3GPP TSG-SA5 Meeting #156 	S5-244171

Maastricht, Netherlands, 18 Aug - 23 Aug 2024

# Source:	Samsung

# Title:	Conflict Management Solutions.

# Document for:	Approval

Agenda Item:	6.19.4

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

None

# 3	Rationale

This provides the solution for existing use case of CCL.

# 4	Detailed proposal

First Change

## 5.6 	Use case 6: CCL conflicts management

### 5.6.1	Description

Multiple CCLs could co-exist and concurrently act within the same environment. The CCLs can affect one another, in the worst cases leading to conflicts. The different kinds of conflicts are summarized by Table 5.6.1-1.

5.6.1-1: Types of potential conflicts among CCL instances for goals g1, g2 and g3

| Conflict Type | Description | CCL-A | CCL-B | Comments |
| --- | --- | --- | --- | --- |
| Target Conflict | For CCLs C1 and C2, when same at least 1 target of a goal is present in both CCL asking for different outcomes on that target on same controlled entity (ME1). | Control Scope: ME1 Goal targets: - Load > 90% (to maximize resource utilization) - latency < 10ms | Control Scope: ME1 Goal target: - Load < 90% (to avoid congestion) | Conflict among the targets within the goals - due to different required target outcomes |
| Action Conflict | For CCLs C1 and C2, when both C1 and C2 is trying to configure the same characteristics of same target entity (gNB-g1) in contradiction. | Example 1 |  | Conflict due to configuration actions at execution step because both CCL want different contradicting value for a particular characteristic of gNB-g1. Effect: even when executed at different times, the value may ping-pong continuously. |
|  |  | Goals target: - Throughput > 10gbps Actions: - Target Entity: gNB-g1 - Target Change: scale-out virtual resource | Goals target: - EC is < 10KVA Actions: - Target Entity: gNB-g1 - Target Change: scale-in virtual resource |  |
|  |  | Example 2 |  |  |
|  |  | Goal target: - HO failure is < 2% Actions: - Target Entity: gNB-g1 - Target Change: set CIO to a small positive value{to guarantee HOs with low chances of HO failure} | Goal target: - Load < 80% Actions: - Target Entity: gNB-g1 - Target Change: set CIO to a small negative value [to advance HOs and move load to other cells] |  |
| Indirect target conflict | For CCLs C1 and C2, when C1 [optimize handover] and C2 [minimize interference] have different goals but the actions of C1 affect the goals of C2 | Goal target: - HO failure is < 2% Actions: - Target Entity: gNB-g1 - Target Change: reduce CIO {to reduce chances of HO failure} | Goal target: - SINR > 10dB Actions: - Target Entity: gNB-g1 - Target Change: lower antenna tilt | By reducing antenna tilt to minimize interference C2 affect the HO goal target of C1 |
| Action Execution Time Conflict | For CCLs C1 and C2, when both C1 and C2 are trying to configure the same characteristics of same target entity (gNB-g1) in contradiction. | Goals: - Throughput > 10gbps Actions: - Target Entity: gNB-g1 - Target Change: scale-out - Target Time: 04:00 | Goals: - EC is < 10KVA Actions: - Target Entity: gNB-g1 - Target Change: scale-in - Target Time: 04:00 | Conflict due to the time of executing the configuration actions at the execution step |
| Scope conflict | For CCLs C1 and C2, C1 and C2 have different goals and actions but their scopes are overlapping – e.g. C1’s control scope (i.e. the controlled entities in the network) is part of C2’s measurement scope (i.e. the measured entities in the network) | Measurement scope: cells g1 Control Scope: g1 Goal targets: - EC/bit is < 1WA Actions: - Target Entity: gNB-g2 - Target Change: switch off g2 | Measurement scope: cells g1, g2, g3, g4 Control Scope: g2 Goals: - Load < 80% Actions: - Target Entity: gNB-g2 - Target Change: change CIO | By switching off g2, C1 affects the scope which C2 reads for its load distribution measurements |

The CCL may detect or observe events that identify the possibility of any one of the above conflicts. The conflict can be avoided using some information or the policies (e.g., priority) provided by the consumer. If the conflict actually occurs, the CCL MnS producer should support services to inform MnS consumers the confirmed detected conflicts. This may also include informing MnS consumer about the potential conflict.

### 5.6.2		Potential Requirements

REQ-CCL-CONFLICT-1: The MnS Producer for CCL management should support a capability to detect a potential or actual conflict.

Note: A potential conflict is where some events are observed that indicate that there may be a conflict, but the CCL MnS Producer cannot conclude that it is a conflict. So, the CCL can indicate this so that some other entity e.g. the MnS consumer takes responsibility to confirm the conflict.

REQ-CCL-CONFLICT-2: The MnS Producer for CCL management should support a capability to inform an authorized MnS consumer about a potential conflict that has been detected.

REQ-CCL-CONFLICT-3: The MnS Producer for CCL management should support a capability to confirm a detected potential goal, action, indirect target, action execution time, scope conflict.

REQ-CCL-CONFLICT-4: The MnS Producer for CCL management should support a capability to resolve a goal, action, indirect target, action execution time, scope conflict that has been detected.

REQ-CCL-CONFLICT-5: The MnS Producer for CCL management should enable authorized MnS consumers to provide information that can be used to avoid the conflict.

REQ-CCL-CONFLICT-6: The MnS Producer for CCL management should enable authorized MnS consumers to provide information that can be used to resolve the conflict.

5.6.3			Potential Solution

The solution involves introducing an IOC to contain conflict related information and mechanism to mitigate any conflict between CCL that may arise during instantiation of a new CCL or between two existing CCL.

- Target Conflict Information: This provide information about the conflict between an existing CCL and a requested CCL.

  - Existing and new CCL identification

  - Conflict Information (conflicting goals/target)

  - Conflict Resolution information

    - Priority: This provides the priority of the CCL. This will be the numerical value between 1 to 10, with 1 being the least priority.

    - OverridingCapable: Whether the CCL can override other CCL

    - OverrideProtect: Whether CCL can be overridden.

  - Target CCL: The identification of the CCL that need to be deleted. This will be decided as per the conflict resolution information.

- Action Conflict Information: This provide information about the conflict between two existing CCL.

  - Conflicting CCLs identification

  - Conflict Information

    - Conflicting Goal

    - Conflicting Execute actions: This provides the set of actions that have been taken by the CCL as part of the Execute step.

  - Conflict Resolution information

    - Priority: This provides the priority of the CCL. This will be the numerical value between 1 to 10, with 1 being the least priority.

    - GoalBreachPercentage: It defines the breach percentage per goal in terms of how bad the goal(s) is breached. For example, if the goal of guaranteed throughput is 200mbps and the actual throughput is coming to be 100mbps then the breach percentage would be 50%. The CCL that have higher percentage of breach will be prioritized.

  - Target CCL: The identification of the CCL that need to be deleted. This will be decided as per the conflict resolution information.

Procedure flow

- The CCL(s) are deployed and running.

- Producer checks for the conflict. This will include checking for both implicit and explicit conflicts. Producer will decide which CCL is to be deleted, among the conflicting CCL, as per the conflict resolution logic provided below.

- The producer send a notification to the consumer providing details on the conflict. This notification will also identify the CCL which need to be deleted in order to mitigate the conflict.

- Based on the recommendation the consumer may delete a CCL. Consumer sends a deleteMOI request for the same.

- Producer sends a reply.

- Producer also request MNO to release all the resources being used by the CCL

- MANO sends response.

- Alternative to 4 above, consumer may decide to modify the CCL instead of deleting it. It sends modifyMOIattribute request.

- Producer sends a response.

Conflict Resolution logic

The target CCL i.e the CCL that need to be deleted to mitigate the conflict will be decided as per the below logic

Last Change