# 3GPP TSG-SA5 Meeting #155	S5-244185

# 19 - 23 Aug 2024, Maastricht, The Netherlands

# Source:	Ericsson

# Title:	Add Cost Function use case

# Document for:	Approval

# Agenda Item:	6.19.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.866 “Study on Management Data Analytics (MDA) – Phase 3”.

# 3	Rationale

This pCR introduces a new use case for the consumer of MDA to provide a cost function when requesting for analytics. There is a current assumption for time urgency when requesting analytics that assist in completing a task. However, when we remove the need for time urgency it allows for further analytics exploration and the possibility to provide multiple sets of recommended actions that take the cost function into consideration, allowing the consumer to choose which of the actions to take based on their individual needs.

# 4	Detailed proposal

Start of modification

## 5.x MDA Cost Function

### 5.x.1 Description

Operators and vendors alike are constantly driving to reduce their energy consumption. This can be achieved in many ways. MDA currently, can provide to a consumer, predictions and/or recommended actions to take to fulfill a certain task e.g., energy saving in a network. Management Data Analytics currently does not address costs, in particular current energy analytics does not cover any aspects of energy cost or Greenhouse Gas Emissions of recommended actions.

It is proposed to enhance the current MDA solution so that the consumer could introduce some further information when making a request for analytics. This extra information could be for e.g., the availability of various energy sources at different times, or an indication of how urgent it is to complete the task. This could be called a "cost function” and could apply to many MDATypes e.g., Energy Analytics, Software Upgrade etc. The cost function would essentially be a mechanism by which the consumer can provide extra information to the producer to assist in providing and categorizing the recommended actions. The analytics producer could then provide several recommended actions, that take into consideration, the extra information provided by the consumer.

Example use case:

Assume that we have a request for prediction that will affect the decision to schedule a task that we need to assign to be executed on various equipment. Tasks and equipment may be very diverse - tasks may range from short to very long-time scales (transmissions of software update), equipment may be a single antenna or entire sites. The current assumption of urgency eliminates the opportunity to find other solutions that are more energy or resource-efficient but less time-efficient. This proposal would provide several sets of recommended actions for the consumer to choose between.

Recommended Action set 1 - most time efficient

Recommended Action set 2 - most energy efficient

Recommended Action set 3 - most resource efficient

It may be that the consumer of analytics is not under any strict time constraints to complete the task at hand so may take the recommendation for the most energy efficient way forward. This could be as simple as executing the task at nighttime when energy costs are lower.

### 5.x.2 Potential Requirements

REQ-COST-FUNC-1 - The producer of analytics should be able to allow an authorized consumer to provide a cost function which provides extra information about the task to be completed e.g., time versus energy, resource versus time.

REQ-COST-FUNC-2 - The producer of analytics should be able to provide different sets of recommended actions based on different criteria provided by the consumer e.g., time efficiency, energy efficiency, resource efficiency.

### 5.x.3 Possible Solutions

It is proposed to enhance the current MDAS solution as specified in TS 28.104 [2]:

- Add a cost function attribute to the existing MDARequest to allow the consumer to add supplementary information on the task for which the analytics is being requested.

- Extend the MDAS reporting to give several sets of recommended actions for the consumer to choose from.

### 5.x.4 Evaluation of Solutions

The solution in 5.x.3 fulfills the proposed requirements.

End of modifications