3GPP TSG-SA5 Meeting #156 	S5-244205

Maastricht, Netherland, 19 Aug - 23 Aug 2024

# Source:	Samsung

# Title:	MnS authorization for external consumer solutions

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

Discuss and agree.

# 2	References

None

# 3	Rationale

This provides the solution for the existing use case of Authorization of the external MnS consumer to access the management service API

# 4	Detailed proposal

First Change

### 5.1.4 Use case #4: Authorization of the external MnS consumer to access the management service API

#### 5.1.4.1	Description

After the external MnS consumer has discovered the available management services (now service APIs) at the CCF, the external MnS consumer proceeds to request the CCF for the authorization information to access the service API (see clause 8.11 of TS 23.222 [5] and clause 6.5.2.3 of TS 33.122 [14]) via the CAPIF-1e interface. This authorization information contains the actions (e.g., ALLOW, DENY) the external MnS consumer can perform on the discovered MnS producer. These actions might differ from what the external MnS consumer is authorized to discover from the CCF (depending on the configured discovery policy). However, currently, the CCF does not have this authorization information in order to generate the access token to grant external MnS consumers accessto the discovered service APIs (i.e., the management services).

Furthermore, after receiving the authorization information from the CCF, the external MnS consumer will proceed to access the management service at the MnS producer (the MnS producer is the CAPIF API exposing function (AEF)) via the CAPIF-2e interface. This implies that the MnS producer should be able to support the CAPIF-2e interface; however, it is safe to assume that this is already supported.

#### 5.1.4.2	Potential requirements

PREQ-FS_MExpo-Auth-01: The 3GPP management system shall provide the CCF with the authorization information for an external MnS consumer.

#### 5.1.4.3	Potential solutions

#### 5.1.4.3.x	Potential solution #1: ExposureGovernanceControl

###### 5.1.4.3.x.1	Introduction

The solution requires the creation of a new NRM fragment to control the exposure governance.

###### 5.1.4.3.x.2	Description

In this solution an ExposureGovernanceControl IOC is proposed which will control the exposure of the available management capabilities towards an external consumer. An external consumer may not be provided with the full set of management capabilities. Instead, they will only be authorized to consume a part of management capabilities as per the contract with the PLMN operator and local polices in place with in operator domain.	 The following information may be included as part of ExposureGovernanceControl IOC:

- External consumer identification: The identification of the consumer.

- Consumer Type: This defines the type of consumer.

  - NOP-External: This refers to the entities outside 3GPP e.g GSMA

  - OAM-External: This refers to the entities defined in 3GPP by other groups e.g NSCE Server (SA6), NWDAF (SA2)

- MnS Information: This refer to the MnSInfo as defined in [3]

- Access Authorization: This refers to the AccessRule [29]

Figure 5.1.4.3.x.2-1: Exposure Governance Control Fragment

#### 5.1.4.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

Next Change