3GPP TSG-SA5 Meeting #156 	S5-244285

Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	Ericsson, Deutsche Telekom

# Title:	pCR TR 28.867 Add clause on CCL as enabler for intent

# Document for:	Approval

Agenda Item:	6.19.4

# 1	Decision/action requested

The group agrees the text in the detailed proposal.

# 2	References

[1]	3GPP TS 28.536: "3GPP TS 28.536: “Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3”

[2]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements"

[3]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

# 3	Rationale

The study item description for the study on closed control loop management includes a work task to “Study the relation between closed loops and other management features (e.g. SON functions, MDA, Intent) with the objective to harmonize, as appropriate”.

This contribution explores the capabilities of closed control loop, see reference [1] and [2] versus the capabilities of intent, see reference [3], their commonalities their differences and how they complement each other.

It is proposed to add the descriptions in clause 4 under the title “Closed control loop as enabler for intent” and a conclusion and recommendation in clause 6.

# 4	Detailed proposal

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements"

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[y]	3GPP TS 28.536: "3GPP TS 28.536: “Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3”

[z]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

2nd Change

## 4.x	Closed control loop as enabler for Intent handling

A closed control loop can be described as an autonomous “entity” (after it has been configured in the network and associated with the controlled entity) with goals and descriptions what shall be achieved continuously going through the phases of monitoring, analysis, decide and execute. The producer reports after an observation window if the goal is fulfilled or not fulfilled, see for further details TS 28.536 [y].

An intent handler function can be described as an autonomous “entity” handling an intent containing a list of expectations, see TS 28.312 [z]. An expectation includes knowledge about the desired outcomes. An intent handler function is producer of intent MnS. The producer of the intent MnS, reports on the fulfilment of an expectation according to the reporting requirements from the MnS consumer.

In contrast to closed control loop an intent has no information on how the producer realizes the intent expectations. Intent handling includes initial fulfilment and continuous assurance of the intent requirements. In many but not all cases closed control loops can be used to implement intent handling. The relationship between Intents and CCLs is shown in Figure 4.x.1.

Producers of  CCLs

Intent handlers

CCL is used to implement intent handling.

Non-intent driven CCL.

Intent not using CCL.

Figure 4.x.1: Venn diagram showing relationship between set of Intent and set of CCLs.

The main difference between intent handling and closed control loop concepts are demonstrated in table 4.x.1

Table 4.x. 1: Comparison between intent handling and closed control loop concepts.

|  | Intent handling | Closed control loops | Comments |
| --- | --- | --- | --- |
| Objective | Focuses on how to comply to service and domain requirements as well as knowledge about desired outcomes as communicated by intent. | Focuses on maintaining optimal network performance through continuous feedback and adjustments i.e., implementing the means to achieve desired outcomes. | An intent can be used to specify what a closed control loop considers to be optimal. |
| Level of abstraction | Operates at an appropriate level, providing abstract requirements, focusing on requirements and communicate knowledge about desired outcomes | Implementing the means to achieve and assure desired outcomes. | Intent handler for business, service or resource might be different |
| Automation focus | Multi-stage translation of higher-level requirements to lower-level requirements. Monitoring of compliance to desired outcomes and translation into policies, actions, and configurations to reach compliance. | Automates the continuous monitoring and immediate adjustment processes. |  |
| Integration | Intent-based systems can utilize closed control loops to ensure that intents are continuously assured by dynamically adjusting network parameters based on immediate feedback. | Closed Control loops can participate in intent handling if they get the task to monitor and assure intent based requirements and detailed objectives derived from intent via an intent handler |  |
| Functional scope | Includes negotiation capabilities see Annex B in TS 28.312 [2]. | Closed Control loop management doesn’t provide negotiation capabilities. |  |
|  | Deployment and assurance of networks and services (see for example clause 5.1.8 of TS 28.312 [2]) | Assurance of communication services (see TS 28.535 [2] and TS 28.536 [y]) |  |

3rd Change

# 6. 			Conclusions and Recommendations

## 6.x	Closed control loop and intent

### 6.x.1	Conclusion

Intent and closed control loops are different, but complementary concepts as discussed in clause 4.x in Table 4.x.1. An intent manager may utilize one or several closed control loops for implementing the assurance of intent requirements. This means intent managers can become consumers of closed control loops.

A closed control loop that provides (MnS producer) an intent API is configured using intent. This closed control loop therefore meets the definition of being an intent manager in the role of intent handler. If the closed control loop acts by sending intent, it implements the intent owner role. This means, that closed control loop can either be utilized as integral part of intent management or directly implement intent management. However, the functional scope of intent management exceeds the proposed scope of closed control loop. Intent management considers, for example, the negotiation of requirements and intent handlers are already driving deployment decisions and processes. Furthermore, the reporting on achievements and results to the source of intent is an integral part of intent management and realized through intent reports.

### 6.x.2	Recommendation

The normative work should follow the principles in the conclusion.

End of Changes