3GPP TSG-SA5 Meeting #156 	S5-244311

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.869 Add background info on cloud-native design principles and their relevance to 3GPP OAM

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.869, " Study on cloud aspects for management and orchestration."

# 3	Rationale

Currently, TR 28.869[1] refers to the term cloud-native network functions throughout the document. The cloud-native network functions have been linked to cloud-native design principles without a proper definition of what these cloud-native design principles are. In addition, it remains unclear what is the relevance of the cloud-native design principles to 3GPP network functions.

# 4	Detailed proposal

This pCR proposes to add background information on cloud-native design principles and their relevance to 3GPP OAM to clause 4 of TR 28.869 [1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                        ETSI GS NFV-IFA 049: “Network Functions Virtualisation (NFV) Release 5; Architectural Framework; VNF generic OAM functions specification.

[3]                        ETSI GR NFV-EVE 019: "Network Functions Virtualisation (NFV) ;Architectural Framework;Report on VNF generic OAM functions".

[4]                        3GPP TR 28.834: "Study on management of cloud-native Virtualized Network Functions (VNF)".

[5]                        SP-230764 New WID on Management of cloud-native Virtualized Network Functions.

[6]                        3GPP TS 28.526: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[7]                       3GPP TS 28.531: “Management and orchestration; Provisioning”.

[8]	ETSI GS NFV-IFA 013 (V4.5.1) (2023-09): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[9]	ETSI GS NFV-IFA 008 (V4.3.1) (2022-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[10]                     3GPP TR 28.532: “Management and orchestration; Generic management services”.

[11]                     ETSI GR NFV 003 (V1.8.3): "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[12]                      3GPP TS 28.555: "Management and orchestration; Network policy management for 5G mobile networks; Stage 1".

[13]                       ETSI GR NFV-IFA 023 (V3.1.1): "Network Functions Virtualisation (NFV); Management and Orchestration; Report on Policy Management in MANO; Release 3".

[14]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements ".

[15]                       3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPIs)".

- 3GPP TS 28.533: “Management and orchestration; Architecture framework”.

[17]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[18]	ETSI GS NFV 006: "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Architectural Framework Specification".

[19] 	ETSI GS NFV-IFA 007: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Or-Vnfm reference point - Interface and Information Model Specification".

[20] 	ETSI GS NFV-IFA 008: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[21] 	ETSI GS NFV-IFA 010: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Functional requirements specification".

[22] 	ETSI GS NFV-IFA 011: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; VNF Descriptor and Packaging Specification".

[23] 	ETSI GS NFV-IFA 013: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[24] 	ETSI GS NFV-IFA 014: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Network Service Templates Specification".

[25] 	ETSI GR NFV-IFA 029: "Network Functions Virtualisation (NFV) Release 3; Architecture;Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS".

[26] 	ETSI GS NFV-IFA 036: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Requirements for service interfaces and object model for container cluster management and orchestration specification".

[27] 	ETSI GR NFV-IFA 038: "Network Functions Virtualisation (NFV) Release 4; Architectural Framework; Report on network connectivity for container-based VNF".

[28] 	ETSI GS NFV-IFA 040: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Requirements for service interfaces and object model for OS container management and orchestration specification".

[29] 	ETSI GR NFV-IFA 043: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; Report on enhanced container networking".

[30] 	ETSI GS NFV-SOL 001: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; NFV descriptors based on TOSCA specification".

[31] 	ETSI GS NFV-SOL 002: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; RESTful protocols specification for the Ve-Vnfm Reference Point".

[32] 	ETSI GS NFV-SOL 003: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; RESTful protocols specification for the Or-Vnfm Reference Point".

[33] 	ETSI GS NFV-SOL 004: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; VNF Package and PNFD Archive specification".

[34] 	ETSI GS NFV-SOL 005: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; RESTful protocols specification for the Os-Ma-nfvo Reference Point".

[35]	ETSI GS NFV-SOL 016: "Network Functions Virtualisation (NFV) Release 4; Protocols and Data Models; NFV-MANO procedures specification".

[36] 	ETSI GS NFV-SOL 018: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models; Profiling specification of protocol and data model solutions for OS Container management and orchestration".

[37] 	ETSI GS NFV-SOL 020: "Network Functions Virtualisation (NFV) Release 5; Protocols and Data Models Specification of protocols and data models for Container Infrastructure Service Cluster Management".

- ETSI GS NFV-IFA 027: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Performance Measurements Specification".

- ETSI GR NFV-EVE 021: "Network Functions Virtualisation (NFV) Release 5; Evolution and Ecosystem;Report on energy efficiency aspects for NFV".

- ETSI GS NFV-IFA 053: "Network Functions Virtualisation (NFV) Release 5; Management and Orchestration; Requirements and interface specification for Physical Infrastructure Management".

[X]                      CNCF Cloud Native Definition v1.1

https://github.com/cncf/toc/blob/main/DEFINITION.md

Second Change

## 4. X	Cloud-native design principles and their relevance to 3GPP OAM

### 4.X.1 Background on cloud-native design principles

Editor's note: In this clause, we would like to list and refer to the industry-agreed/recognized cloud-native principles. Further revision and references may be needed.

The industry-recognized cloud-native design principles[X] that are potentially relevant to the present study are:

Micro-services design:  involves breaking down a cloud-native application into smaller components called micro-services. Each micro-service focuses on solving a smaller and specific problem.

Loosely coupled: implies that each micro-service composing a cloud-native application is a small independent deployable unit. This principle enables each micro-service to evolve independently of the other micro-services.

Containerization: Micro-services and their dependencies are packaged into containers that can run independently across all environments and host operating systems.

Repeatable deployment process: Since cloud-native applications are based on containers, the deployment process of cloud-native applications is reproducible, consistent, and thus repeatable (i.e., can be automated).

Immutable infrastructure: implies that the infrastructure hosting the cloud-native applications cannot be modified after deploying the applications. Modifications are only possible by replacing the old infrastructure with new infrastructure and moving the cloud-native applications to the new infrastructure.

Declarative API: cloud-native applications use declarative APIs to define what to do (i.e., the desired state) instead of how to do it.

Observability: monitoring cloud-native applications through access to metrics, logs, and traces should be possible.

Resiliency: cloud-native applications should be able to tolerate failures by leveraging self-healing mechanisms (e.g., system restarts).

Dynamic scalability: cloud-native applications should be able to scale up or down dynamically depending on the load conditions.

### 4.X.2 Relevance of cloud-native design principles to 3GPP OAM

The 3GPP network functions could follow zero, one or more of the cloud-native design principles listed in clause 4.X.1. Which cloud-native design principles the 3GPP network functions adhere to is implementation-specific. It is not up to the 3GPP management system to mandate that network functions adhere to cloud-native principles in their design.

Editor's note: In this clause, we would like to evaluate each cloud-native design principle listed in clause 4.X.1 from its relevance to 3GPP OAM and document our findings.

Micro-services design: whether a network function is a monolith, or a set of well-defined functional components is not an OAM decision. For example, 3GPP NRM follows the functional architecture defined by 3GPP SA2 for 5GC and 3GPP RAN3 for NR. Conclusion: this principle is not relevant to 3GPP OAM.

Loosely coupled: Conclusion: this principle is not relevant to 3GPP OAM.

Containerization:  Whether network function implementations are virtualized via hypervisor-based vs non-hypervisor-based methods is irrelevant to 3GPP OAM. Conclusion: It may be relevant whether the reference point between the 3GPP management system and the virtualization management system (responsible for management and orchestration) may have a dependency on the type of virtualization (containers vs. VMs).

Repeatable deployment process: Conclusion: this principle is not relevant to 3GPP OAM.

Immutable infrastructure: Conclusion: this principle is out-of-scope for 3GPP and, therefore, irrelevant to 3GPP OAM.

Declarative API: Conclusion: this principle is not relevant to 3GPP OAM.

Observability: From the 3GPP OAM perspective, all 3GPP network functions are observable. Their observability level will be reflected in the corresponding NRM. Conclusion: this principle is not relevant to 3GPP OAM.

Resiliency: If something fails, the 3GPP management system will have an alarm. And if something self-heals, the said alarm will be cleared. In addition, 3GPP can implement resiliency at the core network level through the network repository function. Conclusion: There is nothing special to be done for cloud-native resiliency or not.

Dynamic scalability: Conclusion: The 3GPP management system supports network function scalability, i.e., there is nothing special to support cloud-native dynamic scalability.

End of Changes