3GPP TSG-SA5 Meeting #156	S5-244406

Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | 0142 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Annex with example of a hierarchical discovery of MnS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MADCOL_ph2, MADCOL_ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 28.842 concludes and recommends to do normative work for the possible solution of a hierarchical discovery of management data (clause 4.2.3.3). Due to the nature of SBMA as defined in TS 28.533 a hierarchical discovery of MnS producers is possible already today, although this fact is not necessarily obvious and caused discussions during SA5 154. Since it is important to understand such general implications of SBMA, it is proposed to add a corresponding annex to 28.533 as explanation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The non-normative annex describes how the existing definition of SBMA (TS 28.533) allows for a hierarchical discovery of management services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This very useful consequence of SBMA might not be taken into consideration when desinging management systems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# A.x	An example of hierarchical discovery of MnS producers

This annex shows an example how the service based management architecture (SBMA) allows for a hierarchical discovery of MnS producers by interested MnS consumers.

SBMA allows any management function (MnF) to implement any MnS producer and consumer. As consequence, an implementation of a 3GPP management system might be composed of several management functions as depicted in Fig. A.x-1:

- A MnF “Central Registry” that offers an interface with operations according to a generic provisioning MnS [9] and exposing only the IOC SubNetwork, MnsRegistry, and MnsInfo according to [32].

- A MnF “PM-Reader” with the primary task to act as MnS consumer to collect PM data. In order to discover the required MnS producers, the MnF “PM-Reader” implements a provisioning MnS consumer that is able to request the operation getMOIAttributes with certain filters on the attributes of MnsRegistry and MnsInfo.

- Two MnF (PM-Provider-1, PM-Provider-2) that each are acting as MnS producer for PM data of the network element BTS-1, and that each are acting as MnS producers for generic provisioning [9] and are exposing (omong other IOC) the IOC SubNetwork, MnsRegistry, and MnsInfo according to [32]. PM-Provider-1 offers an interface according to 3GPP release 17, while PM-Provider-2 offers an interface according to 3GPP release 18.

Fig. A.x-1: A simple hierarchical registry according to the current standards, where one Central Registry holds a subset of information to describe MnS, while the local MnsRegistries of the PM-Providers 1 and 2 are describing more details.

For a hierarchical discovery of MnS producers the MnF “PM-Reader” might apply the following strategy (Fig. A.x-1):

- PM-Reader sends a request getMOIAttributes to the MnF “Central Repository” to request all objects of type MnsInfo, that relate to mnsScope=BTS-1.

The MnF “Central Repository” will reply with all instances of MnsInfo that relate to BTS-1. In the example these are the PM MnS producers and provisioning MnS producers of both, PM-Provider-1 and PM-Provider-2.

- Now PM-Reader sends a request getMOIAttributes to the already discovered provisioning MnS producers of PM-Provider-1 and PM-Provider-2. In these requests the PM-Reader filters on additional details, in this example the mnsType=FileDataReporting and mnsVersion=Rel-18, since the PM-Reader requires these for interoperability.

PM-Provider-1 of Rel-17 sends an empty result, since the filter does not result in any match, since Rel-18 is requested.

PM-Provider-2 sends an instance of MnsInfo that matches the filter criteria as requested by MnF PM-Reader.

End of change