3GPP TSG-SA5 Meeting #156	S5-244451

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0205 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.554 End to end downlink and end to end uplink reliability KPIs of URLLC Network Slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Shenzhen |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | End to end Reliability is a unidirectional integral performance characteristic of a URLLC network slice. A CSP/NOP may want to assess end to end DL or end to end UL reliability of a URLLC network slice. Communication services such as Cloud/Edge/Split Rendering, Gaming or Interactive Data Exchanging, wireless road-side infrastructure backhaul etc have high reliability requirements. A CSP/NOP providing such crucial services via a URLLC network slice, would like to check its URLLC slice’s end to end DL and/or end to end UL reliability. Hence, the solution is proposed to determine end to end DL and end to end UL reliability of URLLC network slice. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two new KPIs for calculating end to end Downlink reliability KPI and end to end Uplink reliability KPI of a URLLC Network Slice |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | End to end downlink and end to end uplink reliability KPIs of URLLC network slice would not be present in specification as KPI formula. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause no. 6.8.1. New clauses 6.8.1.X and 6.8.1.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.8.1.X	End to end Downlink reliability KPI of URLLC Network Slice

a)	PURLLC,ReliabilityDL,PSR

b)	A packet success rate percentage (PSR%) based reliability of a network slice is a uni-directional performance characteristic of the slice. This KPI describes the end to end reliability of a URLLC network slice in downlink direction based on PSR%. It is obtained by multiplying PSR% based downlink reliability of Uu interface and N3 interface which are defined in clause 6.8.1.1 and clause 6.8.1.3 respectively. It is a percentage value (%).

c)  Below is the equation for end to end reliability of a URLLC network slice in downlink direction

PURLLC,ReliabilityDL,PSR  = PSRDL,Uu × PSRDL,N3

Where,

PSRDL,Uu  is equal to DLRelPSR_Uu.SNSSAI which is as defined in clause 6.8.1.1.

PSRDL,N3  is equal to DLRelPSR_N3.SNSSAI which is as defined in clause 6.8.1.3.

d) NetworkSlice

Second change

#### 6.8.1.Y	End to end Uplink reliability KPI of URLLC Network Slice

a)	PURLLC,ReliabilityUL,PSR

b)	A packet success rate percentage (PSR%) based reliability of a network slice is a uni-directional performance characteristic of the slice. This KPI describes the end to end reliability of a URLLC network slice in uplink direction based on PSR%. It is obtained by multiplying PSR% based uplink reliability of Uu interface and N3 interface which are defined in clause 6.8.1.2 and clause 6.8.1.4 respectively. It is a percentage value (%).

c)  Below is the equation for end to end reliability of a URLLC network slice in uplink direction

PURLLC,ReliabilityUL,PSR  = PSRUL,Uu × PSRUL,N3

Where,

PSRUL,Uu  is equal to ULRelPSR_Uu.SNSSAI which is as defined in clause 6.8.1.2.

PSRUL,N3  is equal to ULRelPSR_N3.SNSSAI which is as defined in clause 6.8.1.4.

d) NetworkSlice

End of changes