3GPP TSG-SA5 Meeting #156 	S5-244727

Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	Ericsson

# Title:	pCR TR 28.871 Alarm definition

# Document for:	Approval

Agenda Item:	6.19.8

# 1	Decision/action requested

Approve the proposal.

# 2	References

[1]	3GPP TR 28.871 Study on Service Based Management Architecture enhancement phase 3 (Release 19) v0.2.0

[2]	S5-238120	Discussion paper on Alarm definition

# 3	Rationale

The Study on Service Based Management Architecture enhancement phase 3 [1] includes the work task:

WT-3.1	Study whether a new 3GPP alarm type in Alarm definition is needed.

This document specifies the use-cases and potential requirements and solution for this topic. The main goals include:

- Making it possible to list alarming conditions supported by the producer

- Allowing alarm filtering based on the 3 parameters defining the alarming-condition

# 4	Detailed proposal

First change

## 5.X	Use case #<X>: List and handle Alarming-Conditions

### 5.X.1	Description

Observation 1: Alarm capability

Currently, there is no standardized mechanism defined that can be used to retrieve the set of all alarming-conditions that a MnS producer could notify about.

Observation 2: Alarm related notification filtering is difficult

The most basic requirement on alarm related notification filtering is to filter based on the alarming-condition (defined by the alarmType, probableCause, specificProblem). As some of these parameters are not carried in some alarm related notifications this filtering task is not possible e.g. for notifyCorrelatedNotificationChanged.

### 5.X.2	Potential requirements

| Requirement label | Description | Related use case(s)/Motivation |
| --- | --- | --- |
| REQ-MS-FMAL-1 | A producer should be able to provide a list of all the alarming-conditions it might raise an alarm for. |  |
| REQ-MS- FMAL-2 | Every alarming-condition should have a set of associated data that describes the alarming-condition. The information set shall at least include: alarmType, probableCause, specificProblem | The information helps the user correctly handle an alarm. Additional information may be included for each alarming-condition |
| REQ-MS- FMAL-3 | It should be possible to filter alarm related notifications based on the alarming-condition. | Motivation: Management systems may not want to handle all alarming-conditions. |

### 5.X.3	Potential solutions

#### 5.X.3.1	Potential solution #part1: List of alarming-conditions

FFS

#### 5.X.3.2S	Potential solution #part2: Filtering on alarming condition

FFS

### 5.X.4	Evaluation of potential solutions

FFS

End of change