3GPP TSG-SA5 Meeting #156 	S5-244747

Maastricht, The Netherlands, 19 - 23 August 2024						Revision of S5-244421

# Source:	Ericsson España S.A., Deutsche Telekom, AT&T

# Title:	pCR TR 28.879 Registration use case – update agreed content and add evaluation of solution 1

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

[2] S5-243250: “pCR TR 28.879 Registration of MnS producer into CAPIF”

# 3	Rationale

Clause 5.1.1 in TR 28.879 [1] focuses on registration use case, which describes the ability of the MnS producer to register into the CAPIF. Once registered, the MnS producer is able to communicate with the CCF for the subsequent procedures needed to configure accessibility to MnS(s) from external MnS consumers. These procedures include publication, discovery and access control, among others.

Status: At SA5#155, a first draft of publishing use case was agreed in S5-243250 [2]. This first draft contains:

- Description (clause 5.1.2.1)

- Potential requirements (clause 5.1.2.2)

- Potential solutions (clause 5.1.2.3). Specifically, one potential solution was agreed: solution #1. This solution proposes how to capture MnS producer information (SA5 definition) into APIProviderEnrolmentDetails data type (SA6 definition).

Gaps: the agreed content needs re-shaping, in order to:

- provide further clarity on the scope and relationship with other use cases

- for the solution #1 description, align style of mapping table with the other agreed use cases.

- Decouple solution description (clause 5.1.2.3) from solution evaluation (clause 5.1.2.4); in the agreed content they are mixed in clause 5.1.2.3

# 4	Detailed proposal

The present pCR aims to bridge this gap, by making the following changes to the latest version of TR 28.879 [1].

Begin Change

## 5.1	Exposure of management services

### 5.1.1	Use case #1:  MnS producer registration into CAPIF

#### 5.1.1.1	Description

For a MnS to be made available for consumption through an exposure framework, it is needed that this MnS producer registers into that exposure framework. Once registered, the MnS producer is able to communicate with the CCF for the subsequent procedures needed to configure accessibility to MnS from external MnS consumers. These procedures include publication, discovery and access control, among others.

In the CAPIF, the scope of registration is limited to API provider domain functions, which include API Exposing Function (AEF), API Publishing Function (APF) and API Management Function (AMF). The registration is a procedure whereby these functions become recognized users of the CCF. This procedure is described in clause 8.28 of TS 23.222 [5], with stage 3 solution set detailed in clause 8.9 of TS 29.222 [13].

To register (and/or update the registration information of) an API provider domain on the CCF, the AMF from this API provider domain communicates with the CCF over CAPIF-5 interface. This means that:

-	This API provider domain shall have an AMF.

-	The AMF of this API provider domain shall be an authorized user of CCF.

-	The AMF security information for CCF to validate the registration request is provisioned by the CAPIF administrator.

To register a MnS producer on the CCF, it is needed for the MnS producer to support some API provider domain functionality, as detailed below:

-	AEF. If supported, this means that the MnS producer will need to support:

-	CAPIF-2/2e interface, so that the API invokers acting as external MnS consumers can access service APIs, when required. The API operations that will be implemented in this interface (see TS 29.222 [13], clause 9.1) can be configured later, depending on the intended usage of MnS producer.

NOTE: The service APIs can carry different MnS information for different API invokers. This issue is discussed in use case #3 (clause 5.1.3).

-	CAPIF-3 interface, so that the MnS producer can communicate with CCF to exercise access and policy related control for service API invocations initiated by the API invoker. The API operations that will be implemented in this interface (see TS 29.222 [13], clauses 8.3, 8.5, 8.7, 8.6) can be configured later, depending on the intended usage of MnS producer.

-	APF. If supported, this means that the MnS producer will need to support CAPIF-4 interface, so that it can communicate with CCF to publish (and manage the published) MnS information. The API operations that will be implemented in this interface (see TS 29.222 [13], clause 8.2) can be configured later, depending on the intended usage of MnS producer.

-	AMF. If supported, this means that MnS producer will need to support CAPIF-5 interface. The API operations that will be implemented in this interface (see TS 29.222 [13], clause 8.3 and 8.9) can be configured later, depending on the intended usage of MnS producer.

The MnS producer shall support one or more API provider domain functions, with at least one of them being AEF; otherwise, the MnS producer is not eligible for integration into the CAPIF.

The AMF communicates the MnS producer details (including supported API provider domain functions) to the CCF using CAPIF_API_Provider_Management_API (see TS 29.222 [13], clause 8.9). Upon reception of these details, the CCF registers the MnS producer with the supported API provider domain function(s).

2nd Change

#### 5.1.1.3	Potential solutions

##### 5.1.1.3.1	Potential solution #1: Capturing MnS Producer registration information with APIProviderEnrolmentDetails

###### 5.1.1.3.1.1	Introduction

This solution focuses on meeting the requirements PREQ-FS_MExpo-Reg-01,  PREQ-FS_MExpo-Reg-02 and PREQ-FS_MExpo-Reg-03.

In the CAPIF, the registration procedure is executed with the CAPIF_API_Provider_Management API (see TS 29.222 [13], clause 8.3), initiated by the AMF over the CAPIF-5 interface. The AMF sends a HTTP POST message to the CCF with a request body containing the following dataType: “APIProviderEnrolmentDetails” (see TS 29.222 [13], clause 8.9.4.2.2).

The solution proposes to capture MnS producer information with “APIProviderEnrolmentDetails” and reuse the CAPIF_API_Provider_Management API to register, de-register and update registration information related toMnS producer.

###### 5.1.1.3.1.2	Description

How the MnS producer information relevant for registration can be captured with “APIProviderEnrolmentDetails” is described in tables 5.1.1.3.1.2-1, 5.1.1.3.1.2-2, 5.1.1.3.1.2-3, and 5.1.1.3.1.2-4.

Table 5.1.1.3.1.2-1: Representing MnS producer registration information with APIProviderEnrolmentDetails attributes (extract Table 8.9.4.2.2-1 in TS 29.222 [13]).

| Attribute name | Data type | P | Cardinality | Description | Comments |
| --- | --- | --- | --- | --- | --- |
| apiProvDomId | string | O | 0..1 | API provider domain ID assigned by the CAPIF core function to the API management function while registering the API provider domain. Shall not be present in the HTTP POST request from the API management function to the CAPIF core function, to on-board itself. Shall be present in all other HTTP requests and responses. |  |
| regSec | string | M | 1 | Security information necessary for the CAPIF core function to validate the registration of the API provider domain. Shall be present in HTTP request from API management function to CAPIF core function for API provider domain registration. | It can be used to store the credentials of the MnS producer. |
| apiProvFuncs | array(APIProviderFunctionDetails) | O | 1..N | A list of individual API provider domain functions details. When included by the API management function in the HTTP request message, it lists the API provider domain functions that the API management function intends to register/update in registration or update registration procedure. When included by the CAPIF core function in the HTTP response message, it lists the API domain functions details that are registered or updated successfully. | See Table 5.1.1.3.1.2-2 |
| apiProvDomInfo | string | O | 0..1 | Generic information related to the API provider domain such as details of the API provider applications. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the supported optional features of the API as described in clause 7.8. This attribute shall be provided in the HTTP POST request and in the response of successful resource creation. | Only applicable when API provider is SCEF/NEF, and thus not applicable when API provider is MnS producer. |
| failReason | string | C | 0..1 | Registration or update specific failure information of failed API provider domain function registrations. Shall be present in the HTTP response body if atleast one of the API provider domain function registration or update registration fails. |  |
| apiProvName | string | O | 0..1 | Represents the API provider name. | RNAA |

Table 5.1.1.3.1.2-2: Representing MnS producer registration information with APIProvFuncs attributes (extract Table 8.9.4.2.3-1 in TS 29.222 [13]).

| Attribute name | Data type | P | Cardinality | Description | Comments |
| --- | --- | --- | --- | --- | --- |
| apiProvFuncId | string | C | 0..1 | API provider domain functionID assigned by the CAPIF core function to the API provider domain function while registering/updating the API provider domain. Shall not be present in the HTTP POST request from the API management function to the CAPIF core function, to register itself. Shall be present in all other HTTP requests and responses. |  |
| regInfo | RegistrationInformation | M | 1 | Information necessary for the CAPIF core function to register the API provider domain function. This information shall be present in HTTP POST/PUT request from API management function to CAPIF core function for API provider domain registration. In the HTTP response message from CAPIF core function, shall include the updated registration information for API provider domain function. | See Table 5.1.1.3.1.2-3 |
| apiProvFuncRole | ApiProviderFuncRole | M | 1 | Role of API provider domain function. The role shall be present in the HTTP POST/PUT request that the API management function intends to register/update the API provider domain function as. CAPIF core function shall register the role of API provider domain function as per the request. | See Table 5.1.1.3.1.2-4. This data type serves to specify, for the MnS producer to be registered, which CAPIF API provider domain function(s) will be supported. |
| apiProvFuncInfo | string | O | 0..1 | Generic information related to the API provider domain function such as details of the API provider applications. |  |

Table 5.1.1.3.1.2-3: Representing MnS producer registration information with RegistrationInformation attributes (extract Table 8.9.4.2.4-1 in TS 29.222 [13]).

| Attribute name | Data type | P | Cardinality | Description | Comments |
| --- | --- | --- | --- | --- | --- |
| apiProvPubKey | string | M | 1 | Public Key of API Provider domain function. | It can be used to store the public key of the MnS producer. |
| apiProvCert | string | O | 0..1 | API provider domain function's generic client certificate | It can be used to store the client certificate of the MnS producer, if existing. |

Table 5.1.1.3.1.2-4: Representing MnS producer registration information with ApiProviderFuncRole attributes (extract Table 8.9.4.3.3-1 in TS 29.222 [13]).

| Enumeration value | Description | Comments |
| --- | --- | --- |
| AEF | API provider function is API Exposing Function. |  |
| APF | API provider function is API Publishing Function. |  |
| AMF | API provider function is API Management Function. |  |

#### 5.1.1.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

##### 5.1.1.4.x	Evaluation of potential solution #1: Capturing MnS Producer registration information with APIProviderEnrolmentDetails

To manage the registration of an MnS producer on the CCF using CAPIF_API_Provider_Management API, it is needed to populate the “APIProviderEnrolmentDetails” datatype with the registration information of the MnS producer. Detailed in clause 5.1.1.3.1.2, the mapping solution is summarized below:

- The credentials of the MnS producer can be mapped to “APIProviderEnrolmentDetails /regSec”.

- The different roles played for the MnS producer can be mapped to “APIProviderEnrolmentDetails /apiProvFuncs”. Note that the MnS producer will always play AEF, with the option to also play APF and/or AMF, if needed.

- The public key of the MnS producer can be mapped to “APIProviderEnrolmentDetails/ apiProvFuncs/regInfo/apiProvPubKey”.

The solution is feasible.

End Change