3GPP TSG-SA5 Meeting #156 	S5-244750

Maastricht, The Netherlands, 19 - 23 August 2024						Revision of S5-244424

# Source:	Ericsson España S.A., Deutsche Telekom, AT&T

# Title:	pCR TR 28.879 Publishing of MnS into the CCF solution evaluation

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

[2] S5-243252: “Rel-19 pCR TR 28.879 Add use case for publishing management services to the CCF”

# 3	Rationale

Clause 5.1.2 in TR 28.879 [1] focuses on publishing use case, which describes the ability of the MnS producer to publish MnS information into the CCF, so this information can later be used by the external MnS consumers to discover and subsequently invoke the MnS.

Status: At SA5#155, a first draft of publishing use case was agreed in S5-243252 [2]. This first draft contains:

- Description (clause 5.2.2.1)

- Potential requirements (clause 5.2.2.2)

- Potential solutions (clause 5.2.2.3). Specifically, one potential solution was agreed: solution #1. This solution proposes how the MnS information can be published into the CCF, by mapping MnsInfo IOC (SA5 definition) Into the ServiceAPIDescription data type (SA6 definition).

Gap: the evaluation of potential solutions (clause 5.2.2.4) is empty; no evaluation has been performed so far to assess the merits/drawbacks of the proposed solutions.

Ambition: The present pCR aims to provide an evaluation of solution #1. This evaluation will serve as a basis to later report the conclusions and recommendations for this study.

# 4	Detailed proposal

It is proposed to make the following changes in the latest version of TR 28.879 [1].

Begin Change

#### 5.1.2.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

An MnS can be published into or more service APIs each characterized by ServiceAPIDescription data type.

The solution #1 shows that it is feasible to publish MnS information (captured in MnSInfo IOC ) using ServiceAPIDescription attributes.  The solution fulfils PREQ-FS_MExpo-Pub-01 and PREQ-FS_MExpo-Pub-02.

The published MnS information will be used by the external MnS consumers to discover and subsequently invoke the service API(s) . For discovery purposes, the external MnS consumer can read the following ServiceAPIDescription attributes:

- “resources” (mandatory). It identifies which resources can be accessed using the service API, and for each resource, the supported operations. This attribute allows external MnS consumers to know the scope of a published service API. The discovery here is similar to SA5 discovery mechanism, with the difference that external MnS consumer reads URIs (identifiers of resources) instead of DNs (mnsScope attribute of MnSInfo IOC).

- “description” (optional). It can be used to provide additional information for a specific service API, e.g. optional MnSInfo IOC attributes based on the MnS type. If present, this attribute complements “resources” attributes.

End Change