3GPP TSG-SA5 Meeting #156 	S5-244789

# Maastricht, Netherlands 19 - 23 August 2024                             revision of S5-243971

# Source:	Huawei

# Title:	pCR TR 28.874 Solution for NTN neighbour cell management

# Document for:	Approval

Agenda Item:	6.19.15

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TR 28.874: " Study on management aspects of NTN – Phase 2"

[2]		3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

3	Rationale

It’s proposed to add solution for use case NTN neighbour cell management as described in clause 5.2.1 of [1]. There is an existing IOC representing the neighbour cell relation from a source cell to a target cell - NRCellRelation defined in TS 28.541 [2] clause 4.3.32.1, as shown below:

“

4.3.32	NRCellRelation

4.3.32.1	Definition

This IOC represents a neighbour cell relation from a source cell to a target cell, where the target cell is an NRCellCU or ExternalNRCellCU instance.

The source cell can be a NRCellCU instance. This is the case for an Intra-NR neighbour cell relation.

The source cell can be a EUtranGenericCell instance. This is the case for Inter-LTE-NR neighbour cell relation, from E-UTRAN to NR. See 3GPP TS 28.658 [19].

Neighbour cell relations are unidirectional.

4.3.32.2	Attributes

The NRCellRelation IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nRTCI | O | T | T | F | T |
| cellIndividualOffset | M | T | T | F | T |
| isRemoveAllowed | CM | T | T | F | T |
| isHOAllowed | CM | T | T | F | T |
| isESCoveredBy | CM | T | T | F | T |
| isENDCAllowed | CM | T | T | F | T |
| isMLBAllowed | CM | T | T | F | T |
| attribute related to role |  |  |  |  |  |
| nRFreqRelationRef | M | T | T | F | T |
| adjacentNRCellRef | M | T | T | F | T |

”

It is proposed to enhance NRCellRelation IOC to enable NTN neighbour cell valid duration configuration.

4	Detailed proposal

This document proposes the following changes in TR 28.874.

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.423: "Technical Specification Group Radio Access Network; NG-RAN; Xn application protocol (XnAP) "

[3]	3GPP TS 38.300: "Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description; Stage 2"

[4]	3GPP TR 38.821: "Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) "

[5]	3GPP TR 22.865: “Study on satellite access Phase 3”

[6]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”

[7]	3GPP TS 23.401: “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”

[8]	3GPP TS 23.682: “Architecture enhancements to facilitate communications with packet data networks and applications”

[9]	3GPP TS 28.530: “Management and orchestration; Concepts, use cases and requirements”

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[11]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[12]		3GPP TR 23.700-29: " Study on integration of satellite components in the 5G architecture; Stage 3".

[x]		3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

Next Change

5.2.1.3	Potential solutions

5.2.1.3.1	Potential solution #<i>: Add valid duration for NRCellRelation

The valid neighbour cells of an NTN cell keep changing as the satellite is moving. To provide better mobility coordination and service continuity, the valid duration of the NTN neighbour cell can be configured. This solution proposes to add a new attribute for NRCellRelation IOC (defined in TS 28.541 clause 4.3.32) which indicates the valid duration of neighbour cell relation between a source cell (i.e., an NTN cell) and a target cell (i.e., a neighbour NTN cell of this NTN cell). The valid duration is a time window which represents the start time and end time when the target cell is a valid neighbour cell towards the source cell.

Figure 5. 2.1.3.1-1: procedure of NTN neighbour cells relation configuration

- According to the satellite ephemeris information, such as the orbital trajectory information or coordinates for the NTN vehicles, MnS consumer (e.g., operators) can obtain a neighbour NTN cell list of an NTN cell, which contains one or more neighbour NTN cells. For each neighbor cell in the list, there is a corresponding valid duration.

- When MnS consumer requests to create NRCellRelation instance for each neighbour cell relation between the NTN cell and its neighbour cell, the valid duration is added as one attribute of this NRCellRelation instance. Since the valid duration is only useful for NTN scenario, one attribute constraint for it is that the source and target cell are NTN cells.

- Provisioning MnS producer sends NRCellRelation instance creation result to MnS consumer, which contains the neighbour NTN cell and its valid duration for the source NTN cell.

NOTE:  This solution is most useful for earth-moving scenario, and not applicable for earth-fix scenario.

End of change