3GPP TSG-SA5 Meeting #156 	S5-245052

# Maastricht, Netherlands 19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0198 | rev | 1 | Current version: | 16.16.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS 28.554 update the use of NR option 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | ‘NR option 3’ is one of the 5G architecture options used in TR 38.801 for study, which shows the architecture where the LTE eNB is connected to the EPC with Non-standalone NR. When it turns into normative work, this architecture is renamed as EN-DC (E-UTRA-NR Dual Connectivity) architecture as defined in TS 37.340 clause 4.1.2. Therefore, it is proposed to replace ‘NR option 3’ with ‘EN-DC’ to keep alignment with TS 37.340. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace ‘NR option 3’ with ‘EN-DC’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of ‘NR option 3’ is not normatively correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 3; 6.3.1; 6.3.6; A.7; A.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	ITU-T Recommendation E.800: "Definitions of terms related to quality of service".

[4]	3GPP TS 24.501: " Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[6]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[7]	3GPP TS 23.501: " System Architecture for the 5G System; Stage 2".

[8]	ETSI ES 203 228 V1.2.1 (2017-04): "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[9]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[x]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Overall Description; Stage 2"

Next Change

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EE	Energy Efficiency

kbit	kilobit (1000 bits)

RTT	Round Trip Time

EN-DC	E-UTRA-NR Dual Connectivity

Next Change

6.3.1	Latency and delay of 5G networks

6.3.1.0	Void

6.3.1.1	Downlink latency in gNB-DU

a)	DLLat_gNBDU.

b)	This KPI describes the gNB-DU part of the packet transmission latency experienced by an end-user. It is used to evaluate the gNB latency contribution to the total packet latency. It is the average (arithmetic mean) of the time from reception of IP packet to gNB-DU until transmission of first part of that packet over the air interface, for a packet arriving when there is no previous data in queue for transmission to the UE. It is a time interval (0.1 mS). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	DLLat_gNBDU = DRB.RlcSduLatencyDl

or optionally DLLat_gNBDU.QoS = DRB.RlcSduLatencyDl.QoS where QOS identifies the target QoS quality of service class.

or optionally DLLat_gNBDU.SNSSAI = DRB.RlcSduLatencyDl.SNSSAI where SNSSAI identifies the S-NSSAI.

d)	NRCellDU

6.3.1.2	Integrated downlink delay in RAN

6.3.1.2.1	Downlink delay in NG-RAN for a sub-network

a)	DLDelay_NR_SNw.

b)	This KPI describes the average packet transmission delay through the RAN part to the UE. It is used to evaluate delay performance of NG-RAN in downlink for a sub-network. It is the weighted average packets delay from reception of IP packet in gNB-CU-UP until the last part of an RLC SDU packet was received by the UE according to received HARQ feedback information for UM mode or until the last part of an RLC SDU packet was received by the UE according to received RLC ACK for AM mode. It is a time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below are the equations for average “Integrated downlink delay in RAN” for this KPI on SubNetwork level. The “Integrated downlink delay in RAN” is the sum of average DL delay in gNB-CU-UP of the sub-network (DLDelay_gNBCUUP_SNw) and the average DL delay in gNB-DU of the sub-network (DLDelay_gNBDU_SNw):

DLDelay_NR_SNw = DLDelay_gNBCUUP_SNw + DLDelay_gNBDU_SNw

or optionally DLDelay_ NR_SNw.QOS = DLDelay_gNBCUUP_SNw.QOS + DLDelay_gNBDU_SNw.QOS where QOS identifies the target quality of service class.

or optionally DLDelay_NR_SNw.SNSSAI = DLDelay_gNBCUUP_SNw.SNSSAI + DLDelay_gNBDU_SNw.SNSSAI where SNSSAI identifies the S-NSSAI.

d)	SubNetwork

6.3.1.2.2	Downlink delay in NG-RAN for a network slice subnet

a)	DLDelay_NR_Nss.

b)	This KPI describes the average packet transmission delay through the RAN part to the UE. It is used to evaluate delay performance of NG-RAN in downlink for a network slice subnet. It is the weighted average packets delay from reception of IP packet in gNB-CU-UP until the last part of an RLC SDU packet was received by the UE according to received HARQ feedback information for UM mode or until the last part of an RLC SDU packet was received by the UE according to received RLC ACK for AM mode. It is a time interval (0.1 ms). The KPI type is MEAN.

c)	Below is the equation for average "Integrated downlink delay in RAN" for this KPI on NetworkSliceSubnet level. The "Integrated downlink delay in RAN" for network slice subnet is the sum of average DL delay in gNB-CU-UP of the network slice subnet (DLDelay_gNBCUUP_Nss) and the average DL delay in gNB-DU of the network slice subnet (DLDelay_gNBDU_Nss):

DLDelay_NR_Nss.SNSSAI = DLDelay_gNBCUUP_Nss.SNSSAI + DLDelay_gNBDU_Nss.SNSSAI where SNSSAI identifies the S-NSSAI that the network slice subnet supports.

d)	NetworkSliceSubnet

6.3.1.3	Downlink delay in gNB-DU

6.3.1.3.1	Downlink delay in gNB-DU for a NRCellDU

a)	DLDelay_gNBDU_Cell.

b)	This KPI describes the average packet transmission delay through the gNB-DU part to the UE. It is used to evaluate delay performance of gNB-DU in downlink. It is the average packets delay time from arrival of an RLC SDU at the RLC ingress F1-U termination until the last part of an RLC SDU packet was received by the UE according to received HARQ feedback information for UM mode or until the last part of an RLC SDU packet was received by the UE according to received RLC ACK for AM mode. It is a Time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average DL delay in gNB-DU for a NRCellDU:

DLDelay_gNBDU_Cell = DRB.RlcSduDelayDl + DRB.AirIfDelayDl.

and optionally: DLDelay_gNBDU.QOS = DRB.RlcSduDelayDl.QOS + DRB.AirIfDelayDl.QOS where QOS identifies the target quality of service class.

and optionally: DLDelay_gNB.SNSSAI = DRB.RlcSduDelayDl.SNSSAI + DRB.AirIfDelayDl.SNSSAI where SNSSAI identifies the S-NSSAI

d)	NRCellDU

6.3.1.3.2	Downlink delay in gNB-DU for a sub-network

a)	DLDelay_gNBDU_SNw.

b)	This KPI describes the average packet transmission delay through the gNB-DU part to the UE. It is used to evaluate delay performance of gNB-DU in downlink for a sub-network. It is the weighted average packets delay time from arrival of an RLC SDU at the RLC ingress F1-U termination until the last part of an RLC SDU packet was received by the UE according to received HARQ feedback information for UM mode or until the last part of an RLC SDU packet was received by the UE according to received RLC ACK for AM mode. It is a Time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average DL delay in gNB-DU for a sub-network, where

-	W is the measurement for the weighted average, one of the following:

-	the DL data volume of the NR cell;

-	the number of UL user data packets of the NR cell;

-	any other types of weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s in the SubNetwork.

DLDelay_gNBDU_SNw= 1#NRCellDUDRB.RlcSduDelayDl + DRB.AirIfDelayDl*W1#NRCellDUW

and optionally KPI on SubNetwork level per QoS and per S-NSSAI:

DLDelay_gNBDU_SNw.QoS= 1#NRCellDUDRB.RlcSduDelayDl.QoS + DRB.AirIfDelayDl.QoS*W.QoS1#NRCellDUW.QoS

DLDelay_gNBDU_SNw.SNSSAI= 1#NRCellDUDRB.RlcSduDelayDl.SNSSAI + DRB.AirIfDelayDl.SNSSAI*W.SNSSAI1#NRCellDUW.SNSSAI

d)	SubNetwork

6.3.1.3.3	Downlink delay in gNB-DU for a network slice subnet

a)	DLDelay_gNBDU_Nss.

b)	This KPI describes the average packet transmission delay through the gNB-DU part to the UE. It is used to evaluate delay performance of gNB-DU in downlink for a network slice subnet. It is the weighted average packets delay time from arrival of an RLC SDU at the RLC ingress F1-U termination until the last part of an RLC SDU packet was received by the UE according to received HARQ feedback information for UM mode or until the last part of an RLC SDU packet was received by the UE according to received RLC ACK for AM mode. It is a Time interval (0.1 ms). The KPI type is MEAN.

c)	Below is the equation for average DL delay in gNB-DU for a network slice subnet, where

-	W is the measurement for the weighted average, one of the following:

-	the DL data volume of the NR cell;

-	the number of DL user data packets of the NR cell;

-	any other types of weight requested by the consumer of KPI;

-	the #NRCellDU is the number of NRCellDU’s associated with the NetworkSliceSubnet.

DLDelay_gNBDU_Nss.SNSSAI= 1#NRCellDUDRB.RlcSduDelayDl.SNSSAI + DRB.AirIfDelayDl.SNSSAI*W.SNSSAI1#NRCellDUW.SNSSAI

d)	NetworkSliceSubnet

6.3.1.4	Downlink delay in gNB-CU-UP

6.3.1.4.1	Downlink delay in gNB-CU-UP

a)	DLDelay_gNBCUUP.

b)	This KPI describes the average packet transmission delay through the gNB-CU-UP to the gNB-DU. It is used to evaluate the delay performance of gNB-CU-UP in downlink. It is the average packets delay from reception of IP packet in gNB-CU-UP until the time of arrival, at the gNB-DU, of the RLC SDU at the RLC ingress F1-U termination. It is a Time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below the equation for average DL delay in a gNB-CU-CP:

DLDelay_gNBCUUP = DRB. PdcpSduDelayDl + DRB.PdcpF1Delay

and optionally:  DLDelay_ gNBCUUP.QOS = DRB.PdcpSduDelayDl.QOS + DRB.PdcpF1Delay.QOS where QOS identifies the target quality of service class.

and optionally:  DLDelay_gNBCUUP.SNSSAI = DRB.PdcpSduDelayDl.SNSSAI + DRB.PdcpF1Delay.SNSSAI where SNSSAI identifies the S-NSSAI.

d)	GNBCUUPFunction

e)	In non-split gNB scenario, the value of DRB.PdcpF1Delay (optionally DRB.PdcpF1Delay.QOS, and optionally DRB.PdcpF1Delay.SNSSAI) is set to zero because there are no F1-interfaces in this scenario.

6.3.1.4.2	Downlink delay in gNB-CU-UP for a sub-network

a)	DLDelay_gNBCUUP_SNw.

b)	This KPI describes the average packet transmission delay through the gNB-CU-UP to the gNB-DU. It is used to evaluate the delay performance of gNB-CU-UP in downlink for a sub-network. It is the weighted average packets delay from reception of IP packet in gNB-CU-UP until the time of arrival, at the gNB-DU, of the RLC SDU at the RLC ingress F1-U termination. It is a Time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average UL delay in gNB-CU-UP for a sub-network, where

-	W is the measurement for the weighted average, one of the following:

-	the DL data volume in gNB-CU-UP;

-	the number of DL user data packets in gNB-CU-UP;

-	any other types of weight requested by the consumer of KPI;

-	the # GNBCUUPFunctions is the number of GNBCUUPFunctions’s in the SubNetwork.

DLDelay_gNBCUUP_SNw= 1#GNBCUUPFunctionDRB.PdcpSduDelayDl + DRB.PdcpF1Delay*W1#GNBCUUPFunctionW

and optionally KPI on SubNetwork level per QoS and per S-NSSAI:

DLDelay_gNBCUUP_SNw.QoS= 1#GNBCUUPFunctionDRB.PdcpSduDelayDl.QoS + DRB.PdcpF1Delay.QoS*W.QoS1#GNBCUUPFunctionW.QoS

DLDelay_gNBCUUP_SNw.SNSSAI= 1#GNBCUUPFunctionDRB.PdcpSduDelayDl.SNSSAI + DRB.PdcpF1Delay.SNSSAI*W.SNSSAI1#GNBCUUPFunctionW.SNSSAI

d)	SubNetwork

e)	In non-split gNB scenario, the value of DRB.PdcpF1Delay (optionally DRB.PdcpF1Delay.QOS, and optionally DRB.PdcpF1Delay.SNSSAI) is set to zero because there are no F1-interfaces in this scenario.

6.3.1.4.3	Downlink delay in gNB-CU-UP for a network slice subnet

a)	DLDelay_gNBCUUP_Nss.

b)	This KPI describes the average packet transmission delay through the gNB-CU-UP to gNB-DU. It is used to evaluate the delay performance of gNB-CU-UP in downlink for a network slice subnet. It is the weighted average packets delay from reception of IP packet in gNB-CU-UP until the time of arrival, at the gNB-DU, of the RLC SDU at the RLC ingress F1-U termination. It is a Time interval (0.1 ms). The KPI type is MEAN.

c)	Below is the equation for average UL delay in gNB-CU-UP for a network slice subnet, where

-	W is the measurement for the weighted average, one of the following:

-	the DL data volume in gNB-CU-UP;

-	the number of DL user data packets in gNB-CU-UP;

-	any other types of weight requested by the consumer of KPI;

-	the # GNBCUUPFunctions is the number of GNBCUUPFunctions’s associated with the NetworkSliceSubnet.

DLDelay_gNBCUUP_Nss.SNSSAI= 1#GNBCUUPFunctionDRB.PdcpSduDelayDl.SNSSAI + DRB.PdcpF1Delay.SNSSAI*W.SNSSAI1#GNBCUUPFunctionW.SNSSAI

d)	NetworkSliceSubnet

e)	In non-split gNB scenario, the value of DRB.PdcpF1Delay.SNSSAI is set to zero because there are no F1-interfaces in this scenario.

6.3.1.5	Uplink delay in gNB-DU

6.3.1.5.1	Uplink delay in gNB-DU for a NR cell

a)	ULDelay_gNBDU_Cell.

b)	This KPI describes the average packet transmission delay through the gNB-DU part from the UE in a NR cell. It is used to evaluate delay performance of gNB-DU in uplink. It is the average packet delay from when an UL RLC SDU was scheduled, as per the scheduling grant provided, until time when the RLC SDU is sent to PDCP or CU for split gNB. It is a time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average UL delay in gNB-DU for a NRCellDU:

ULDelay_gNBDU_Cell = DRB.RlcDelayUI + DRB.AirIfDelayUI

and optionally: ULDelay_gNBDU.QoS=DRB.RlcDelayUl.QOS + DRB.AirIfDelayUl.QOS  where QOS identifies the target quality of service class.

and optionally: ULDelay_gNBDU.SNSSAI=DRB.RlcDelayUl.SNSSAI + DRB.AirIfDelayUl.SNSSAI  where SNSSAI identifies the S-NSSAI.

d)	NRCellDU

6.3.1.5.2	Uplink delay in gNB-DU for a sub-network

a)	ULDelay_gNBDU_SNw.

b)	This KPI describes the average packet transmission delay through the gNB-DU part from the UE for a sub-network. It is used to evaluate delay performance of gNB-DU in uplink for a sub-network. It is the weighted average packet delay from when an UL RLC SDU was scheduled, as per the scheduling grant provided, until time when the RLC SDU is sent to PDCP or CU for split gNB. It is a time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average UL delay in gNB-DU for a sub-network, where

-	W is the measurement for the weighted average, one of the following:

-	the UL data volume of the NR cell;

-	the number of UL user data packets of the NR cell;

-	any other types of weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s in the SubNetwork.

ULDelay_gNBDU_SNw= 1#NRCellDUDRB.RlcDelayUl + DRB.AirIfDelayUl*W1#NRCellDUW

and optionally KPI on SubNetwork level per QoS and per S-NSSAI:

ULDelay_gNBDU_SNw.QoS= 1#NRCellDUDRB.RlcDelayUl.QoS + DRB.AirIfDelayUl.QoS*W.QoS1#NRCellDUW.QoS

ULDelay_gNBDU_SNw.SNSSAI= 1#NRCellDUDRB.RlcDelayUl.SNSSAI + DRB.AirIfDelayUl.SNSSAI*W.SNSSAI1#NRCellDUW.SNSSAI

d)	SubNetwork

6.3.1.5.3	Uplink delay in gNB-DU for a network slice subnet

a)	ULDelay_gNBDU_Nss.

b)	This KPI describes the average packet transmission delay through the gNB-DU part from the UE for a network slice subnet. It is used to evaluate delay performance of gNB-DU in uplink for a network slice subnet. It is the weighted average packet delay from when an UL RLC SDU was scheduled, as per the scheduling grant provided, until time when the RLC SDU is sent to PDCP or CU for split gNB. It is a time interval (0.1 ms). The KPI type is MEAN.

c)	Below is the equation for average UL delay in gNB-DU for a network slice subnet, where

-	W is the measurement for the weighted average, one of the following:

-	the UL data volume of the NR cell;

-	the number of UL user data packets of the NR cell;

-	any other types of weight requested by the consumer of KPI;

-	the #NRCellDU is the number of NRCellDU’s associated with the NetworkSliceSubnet.

ULDelay_gNBDU_Nss.SNSSAI= 1#NRCellDUDRB.RlcDelayUl.SNSSAI + DRB.AirIfDelayUl.SNSSAI*W.SNSSAI1#NRCellDUW.SNSSAI

d)	NetworkSliceSubnet

6.3.1.6	Uplink delay in gNB-CU-UP

6.3.1.6.1	Uplink delay in gNB-CU-UP

a)	ULDelay_gNBCUUP.

b)	This KPI describes the average packet transmission delay through the gNB-CU-UP from gNB-DU. It is used to evaluate delay performance of gNB-CU-UP in uplink. It is the average packet delay from when the RLC SDU is sent to PDCP or CU for split gNB, until time when the corresponding PDCP SDU was sent to the core network from gNB-CU-UP. It is a time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below the equation for average UL delay in a gNB-CU-CP:

ULDelay_gNBCUUP=DRB.PdcpReordDelayUl + DRB.PdcpF1Delay

and optionally: ULDelay_gNBCUUP.QoS=DRB.PdcpReordDelayUl.QoS + DRB.PdcpF1Delay.QoS  where QOS identifies the target quality of service class.

and optionally: ULDelay_gNBCUUP.SNSSAI=DRB.PdcpReordDelayUl.SNSSAI + DRB.PdcpF1Delay.SNSSAI  where SNSSAI identifies the S-NSSAI.

d)	GNBCUUPFunction

e)	It is assumed that the F1 uplink delay is the same as the F1 downlink delay. In non-split gNB scenario, the value of DRB.PdcpF1Delay (optionally DRB.PdcpF1Delay.QOS, and optionally DRB.PdcpF1Delay.SNSSAI) is set to zero because there are no F1-interfaces in this scenario.

6.3.1.6.2	Uplink delay in gNB-CU-UP for a sub-network

a)	ULDelay_gNBCUUP_SNw.

b)	This KPI describes the average packet transmission delay through the gNB-CU-UP part from the gNB-DU for a sub-network. It is used to evaluate delay performance of gNB-CU-UP in uplink for a sub-network. It is the weighted average packet delay from when the RLC SDU is sent to PDCP or CU for split gNB, until time when the corresponding PDCP SDU was sent to the core network from gNB-CU-UP. It is a time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average UL delay in gNB-CU-UP for a sub-network, where

-	W is the measurement for the weighted average, one of the following:

-	the UL data volume in gNB-CU-UP ;

-	the number of UL user data packets in gNB-CU-UP ;

-	any other types of weight requested by the consumer of KPI;

-	the # GNBCUUPFunctions is the number of GNBCUUPFunctions’s in the SubNetwork.

ULDelay_gNBCUUP_SNw= 1#GNBCUUPFunctionDRB.PdcpReordDelayUl + DRB.PdcpF1Delay*W1#GNBCUUPFunctionW

and optionally KPI on SubNetwork level per QoS and per S-NSSAI:

ULDelay_gNBCUUP_SNw.QoS= 1#GNBCUUPFunctionDRB.PdcpReordDelayUl.QoS + DRB.PdcpF1Delay.QoS*W.QoS1#GNBCUUPFunctionW.QoS

ULDelay_gNBCUUP_SNw.SNSSAI= 1#GNBCUUPFunctionDRB.PdcpReordDelayUl.SNSSAI + DRB.PdcpF1Delay.SNSSAI*W.SNSSAI1#GNBCUUPFunctionW.SNSSAI

d)	SubNetwork

e)	It is assumed that the F1 uplink delay is the same as the F1 downlink delay. In non-split gNB scenario, the value of DRB.PdcpF1Delay (optionally DRB.PdcpF1Delay.QOS, and optionally DRB.PdcpF1Delay.SNSSAI) is set to zero because there are no F1-interfaces in this scenario.

6.3.1.6.3	Uplink delay in gNB-CU-UP for a network slice subnet

a)	ULDelay_gNBCUUP_Nss.

b)	This KPI describes the average packet transmission delay through the gNB-CU-UP part from the gNB-DU for a network slice subnet. It is used to evaluate delay performance of gNB-CU-UP in uplink for a network slice subnet. It is the weighted average packet delay from when the RLC SDU is sent to PDCP or CU for split gNB, until time when the corresponding PDCP SDU was sent to the core network from gNB-CU-UP. It is a time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average UL delay in gNB-CU-UP for a network slice subnet, where

-	W is the measurement for the weighted average, one of the following:

-	the UL data volume in gNB-CU-UP;

-	the number of UL user data packets in gNB-CU-UP;

-	any other types of weight requested by the consumer of KPI;

-	the # GNBCUUPFunctions is the number of GNBCUUPFunctions’s associated with the NetworkSliceSubnet.

ULDelay_gNBCUUP_Nss.SNSSAI= 1#GNBCUUPFunctionDRB.PdcpReordDelayUl.SNSSAI + DRB.PdcpF1Delay.SNSSAI*W.SNSSAI1#GNBCUUPFunctionW.SNSSAI

d)	NetworkSliceSubnet

e)	It is assumed that the F1 uplink delay is the same as the F1 downlink delay. In non-split gNB scenario, the value of DRB.PdcpF1Delay.SNSSAI is set to zero because there are no F1-interfaces in this scenario.

6.3.1.7	Integrated uplink delay in RAN

6.3.1.7.1	Uplink delay in NG-RAN for a sub-network

a)	ULDelay_NR_SNw.

b)	This KPI describes the average packet transmission delay through the RAN part from the UE for a sub-network. It is used to evaluate delay performance of NG-RAN in uplink. It is the weighted average packet delay from when an UL RLC SDU was scheduled, as per the scheduling grant provided, until time when the corresponding PDCP SDU was sent to the core network from gNB-CU-UP. It is a time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below are the equations for average "Integrated uplink delay in RAN" for this KPI on SubNetwork level. The "Integrated uplink delay in RAN" is the sum of average UL delay in gNB-CU-UP of the sub-network (ULDelay_gNBCUUP_SNw) and the average UL delay in gNB-DU of the sub-network (ULDelay_gNBDU_SNw):

ULDelay_NR_SNw = ULDelay_gNBCUUP_SNw + ULDelay_gNBDU_SNw

or optionally ULDelay_ NR_SNw.QOS = ULDelay_gNBCUUP_SNw.QOS + ULDelay_gNBDU_SNw.QOS where QOS identifies the target quality of service class.

or optionally ULDelay_NR_SNw.SNSSAI = ULDelay_gNBCUUP_SNw.SNSSAI + ULDelay_gNBDU_SNw.SNSSAI where SNSSAI identifies the S-NSSAI.

d)	SubNetwork

6.3.1.7.2	Uplink delay in NG-RAN for a network slice subnet

a)	ULDelay_NR_Nss.

b)	This KPI describes the average packet transmission delay through the RAN part from the UE for a network slice subnet. It is used to evaluate delay performance of NG-RAN in uplink. It is the weighted average packet delay from when an UL RLC SDU was scheduled, as per the scheduling grant provided, until time when the corresponding PDCP SDU was sent to the core network from gNB-CU-UP. It is a time interval (0.1 ms). The KPI type is MEAN. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average “Integrated uplink delay in RAN” for this KPI on NetworkSliceSubNet level. The “Integrated uplink delay in RAN” for network slice subnet is the sum of average UL delay in gNB-CU-UP of the network slice subnet (ULDelay_gNBCUUP_Nss) and the average UL delay in gNB-DU of the network slice subnet (ULDelay_gNBDU_Nss):

ULDelay_NR_Nss.SNSSAI = ULDelay_gNBCUUP_Nss.SNSSAI + ULDelay_gNBDU_Nss.SNSSAI where SNSSAI identifies the S-NSSAI that the network slice subnet supports.

d)	NetworkSliceSubnet

6.3.1.8	E2E delay for network slice

6.3.1.8.1	Average e2e uplink delay for a network slice

a)	DelayE2EUlNs.

b)	This KPI describes the average e2e UL packet delay between the PSA UPF and the UE for a network slice. It is the weighted average packet delay from the time when an UL RLC SDU was scheduled at the UE until the time when the corresponding GTP PDU was received by the PSA UPF. The KPI type is MEAN in unit of 0.1 ms.

c)	This KPI is the weighted average of UL packet delay between PSA UPF and UE, for all N3 interfaces (modelled by EP_N3 MOIs) and N9 interfaces (modelled by EP_N9 MOIs) of all PSA UPFs supporting the network slice (modelled by NetworkSlice MOI) identified by the S-NSSAI.

This KPI is calculated in the equation below, where Wn3 and Wn9 are the measurements for the weighted average, Wn3 is one of the following:

-	the data volume of UL GTP PDUs received by PSA UPF on the N3 interface;

-	the number of UL GTP PDUs received by PSA UPF on the N3 interface;

-	any other types of weight defined by the consumer of KPI.

And Wn9 is one of the following:

-	the data volume of UL GTP PDUs received by PSA UPF on the N9 interface;

-	the number of UL GTP PDUs received by PSA UPF on the N9 interface;

-	any other types of weight defined by the consumer of KPI.

DelayE2EUlNs = EP_N3GTP.DelayUlPsaUpfUeMean.SNSSAI*Wn3.SNSSAI + EP_N9(GTP.DelayUlPsaUpfUeMean.SNSSAI*Wn9.SNSSAI)  EP_N3Wn3.SNSSAI+ EP_N9Wn9.SNSSAI

Where the SNSSAI identifies the S-NSSAI.

d)	NetworkSlice.

6.3.1.8.2	Average e2e downlink delay for a network slice

a)	DelayE2EDlNs.

b)	This KPI describes the average e2e DL packet delay between the PSA UPF and the UE for a network slice. It is the weighted average packet delay from the time when an GTP PDU has been sent by the PSA UPF until time when the corresponding RLC SDU was received by the UE. The KPI type is MEAN in unit of 0.1 ms.

c)	This KPI is the weighted average of DL packet delay between PSA UPF and UE, for all N3 interfaces (modelled by EP_N3 MOIs) and N9 interfaces (modelled by EP_N9 MOIs) of all PSA UPFs supporting the network slice (modelled by NetworkSlice MOI) identified by the S-NSSAI.

This KPI is calculated in the equation below, where Wn3 and Wn9 are the measurements for the weighted average, Wn3 is one of the following:

-	the data volume of DL GTP PDUs transmitted by PSA UPF on the N3 interface;

-	the number of DL GTP PDUs transmitted by PSA UPF on the N3 interface;

-	any other types of weight defined by the consumer of KPI.

And Wn9 is one of the following:

-	the data volume of DL GTP PDUs transmitted by PSA UPF on the N9 interface;

-	the number of DL GTP PDUs transmitted by PSA UPF on the N9 interface;

-	any other types of weight defined by the consumer of KPI.

DelayE2EDlNs =  EP_N3GTP.DelayDlPsaUpfUeMean.SNSSAI*GTP.Wn3.SNSSAI + EP_N9(GTP.DelayDlPsaUpfUeMean.SNSSAI*Wn9.SNSSAI)  EP_N3Wn3.SNSSAI+ EP_N9Wn9.SNSSAI

Where the SNSSAI identifies the S-NSSAI.

d)	NetworkSlice.

Next Change

6.3.6	RAN UE Throughput

6.3.6.1	Void

6.3.6.2	RAN UE Throughput definition

To achieve a Throughput measurement (below examples are given for DL) that is independent of file size and gives a relevant result it is important to remove the volume and time when the resource on the radio interface is not fully utilized. (Successful transmission, buffer empty in figure 1).

Time (slots)

Data arrives to

empty DL buffer

First data is

transmitted to the UE

The send buffer is

again empty

calulations since it

can be impacted

by packet size of

User Plane (UP) packets.

ThpTimeDl

Failed transmission (

”

Block

error

”

)

Successful transmission,

buffer not empty

Successful transmission,

buffer empty

ThpVolDl =

∑

Total DL transferred volume =

∑

(kbits)

+

(kbits)

UE Throughput in DL =

ThpVolDl / ThpTimeDl (kbits/s)

No transmission, buffer not

empty (e.g. due to contention)

The last slot shall always be removed from

Figure 1

To achieve a throughput measurement that is independent of bursty traffic pattern, it is important to make sure that idle gaps between incoming data is not included in the measurements. That shall be done as considering each burst of data as one sample.

6.3.6.3	DL RAN UE throughput

6.3.6.3.1	DL RAN UE throughput for a NRCellDU

a)	DlUeThroughput _Cell.

b)	This KPI describes the average DL RAN UE throughput for a NRCellDU. The KPI type is MEAN in kbit per second. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average DL RAN UE throughput for a NRCellDU:

DlUeThroughput _Cell = DRB.UEThpDl;

and optionally: DlUeThroughput _Cell.QOS = DRB.UEThpDl.QOS, where QOS identifies the target quality of service class;

and optionally: DlUeThroughput _Cell.SNSSAI = DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

d)	NRCellDU

6.3.6.3.2	DL RAN UE throughput for a sub-network

a)	DlUeThroughput _SNw.

b)	This KPI describes the average DL RAN UE throughput for a sub-network. The KPI type is MEAN in kbit per second. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average DL RAN UE throughput for a sub-network, where

-	W is the measurement for the weighted average, it is one of the following:

-	the DL data volume of the NR cell;

-	a weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s in the SubNetwork.

DlUeThroughput _SNw= 1#NRCellDUW1#NRCellDUWDRB.UEThpDl

and optionally KPI on SubNetwork level per QoS and per S-NSSAI:

DlUeThroughput _SNw.QoS= 1#NRCellDUW.QoS1#NRCellDUW.QoSDRB.UEThpDl.QoS

DlUeThroughput _SNw.SNSSAI= 1#NRCellDUW.SNSSAI1#NRCellDUW.SNSSAIDRB.UEThpDl.SNSSAI

d)	SubNetwork

6.3.6.3.3	DL RAN UE throughput for a network slice subnet

a)	DlUeThroughput _Nss.

b)	This KPI describes the average DL RAN UE throughput for a network slice subnet. The KPI type is MEAN in kbit per second.

c)	Below is the equation for average DL RAN UE throughput for a network slice subnet, where

-	W is the measurement for the weighted average, it is one of the following:

-	the DL data volume of the NR cell;

-	a weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s associated with the NetworkSliceSubnet.

DlUeThroughput_Nss.SNSSAI= 1#NRCellDUW.SNSSAI1#NRCellDUW.SNSSAIDRB.UEThpDl.SNSSAI, where the SNSSAI identifies the S-NSSAI that the NetworkSliceSubnet supports.

d)	NetworkSliceSubnet

6.3.6.4	UL RAN UE throughput

6.3.6.4.1	UL RAN UE throughput for a NRCellDU

a)	UlUeThroughput_Cell.

b)	This KPI describes the average UL RAN UE throughput for a NRCellDU. The KPI type is MEAN in kbit per second. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average UL RAN UE throughput for a NRCellDU:

UlUeThroughput _Cell = DRB.UEThpUl;

and optionally: UlUeThroughput_Cell.QOS = DRB.UEThpUl.QOS, where QOS identifies the target quality of service class;

and optionally: UlUeThroughput_Cell.SNSSAI = DRB.UEThpUl.SNSSAI, where SNSSAI identifies the S-NSSAI.

d)	NRCellDU

6.3.6.4.2	UL RAN UE throughput for a sub-network

a)	UlUeThroughput_SNw.

b)	This KPI describes the average UL RAN UE throughput for a sub-network. The KPI type is MEAN in kbit per second. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in EN-DC architecture) and per S-NSSAI.

c)	Below is the equation for average UL RAN UE throughput for a sub-network, where

-	W is the measurement for the weighted average, it is one of the following:

-	the UL data volume of the NR cell;

-	a weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s in the SubNetwork.

UlUeThroughput_SNw= 1#NRCellDUW1#NRCellDUWDRB.UEThpUl

and optionally KPI on SubNetwork level per QoS and per S-NSSAI:

UlUeThroughput_SNw.QoS= 1#NRCellDUW.QoS1#NRCellDUW.QoSDRB.UEThpUl.QoS

UlUeThroughput_SNw.SNSSAI= 1#NRCellDUW.SNSSAI1#NRCellDUW.SNSSAIDRB.UEThpUl.SNSSAI

d)	SubNetwork

6.3.6.4.3	UL RAN UE throughput for a network slice subnet

a)	UlUeThroughput _Nss.

b)	This KPI describes the average UL RAN UE throughput for a network slice subnet. The KPI type is MEAN in kbit per second.

c)	Below is the equation for average UL RAN UE throughput for a network slice subnet, where

-	W is the measurement for the weighted average, it is one of the following:

-	the UL data volume of the NR cell;

-	a weight defined by the consumer of KPI

-	the #NRCellDU is the number of NRCellDU’s associated with the NetworkSliceSubnet.

UlUeThroughput_Nss.SNSSAI= 1#NRCellDUW.SNSSAI1#NRCellDUW.SNSSAIDRB.UEThpUl.SNSSAI, where the SNSSAI identifies the S-NSSAI that the NetworkSliceSubnet supports.

d)	NetworkSliceSubnet

Next Change

A.7	Use case for RAN UE throughput related KPI

The UE perceived throughput in NG-RAN is an important performance parameter for operating 5G network. If the UE throughput of the NR cell cannot meet the performance requirement, some actions need to be performed to the network, e.g. reconfiguration or capacity increase. So it is necessary to define UE throughput KPI to evaluate whether the end-users are satisfied. The KPI covers volume large enough to make the throughput measurement relevant, i.e. excluding data volume of the last or only slot.

The UE throughput KPI covers also E-UTRA-NR Dual Connectivity (EN-DC) [x]  scenarios. Then the gNB is "connected" towards the EPC, and not towards 5GC.

It is proposed to allow UE throughput KPI split into KPIs per QoS level based on mapped 5QI (or QCI in case of EN-DC architecture).

When network slicing is supported by the NG-RAN, multiple NSIs may be supported. The UL and DL UE throughput for each NSI is then of importance to the operator to pinpoint a specific performance problem.

Next Change

A.11	Use case for DRB retainability related KPI

DRB is the key and limited resource for 5GS to deliver services. Once a QoS flow reaches a gNB it will trigger setup of a new DRB or it will be mapped to an existing DRB. The decision on how to map QoS flows into new or existing DRBs is taken at the CU-CP. CU-CP also defines one set of QoS parameters (one 5QI) for the DRB. If a QoS flow is mapped to an existing DRB, the packets belonging to that QoS flow are not treated with the 5QI of the QoS flow, but they are treated with the mapped 5QI of the DRB.

The release of the DRB needs to be monitored, so that abnormal releases while the UE is considered in an active transfer shall be logged. DRB retainability is a key performance indicator of how often an end-user abnormally loses a DRB during the time the DRB is actively used. This key performance indicator is of great important to estimate the end users’ experiences. DRBs with bursty flow are considered active if any data (UL or DL) has been transferred during the last 100 ms. DRBs with continuous flow are seen as active DRBs in the context of this measurement as long as the UE is in RRC connected state. A particular DRB is defined to be of type continuous flow if the mapped 5QI is any of {1, 2, 65, 66}.

The key performance indicator shall monitor the DRB retainability for each used mapped 5QI value, as well as for the used S-NSSAI(s). DRBs used in 3GPP EN-DC architecture shall not be covered by this KPI. For the case when a DRB have multiple QoS flows mapped and active, when a QoS flow is released it will not be counted as a DRB release (DRB still active) in this KPI.

End of Changes