3GPP TSG-SA5 Meeting #157	S5-245345

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei, Deutsche Telekom

# Title:	Update Use case #6 Investigate potential impacts to support natural language intents translation

# Document for:	Approval

Agenda Item:	6.19.3

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP TR 28.914: " Study on intent driven management service for mobile network phase 3 v1.1.0"

3	Rationale

The pCR proposes to update the Use case #6: Investigate potential impacts to support natural language intents translation to include the concrete deployment scenario and example for natural language intents translation.

4	Detailed proposal

1st Change

5.6	Use case #6: Investigate potential impacts to support natural language intents translation

5.6.1	Description

Intent management aims to provide more intelligent services by understanding and implementing users' intents. In 3GPP TS 28.312 [2], the primary focus is on intent where intent models are used. The formal intent and intent report model are defined in TS 28.312 [2] for intent driven MnS to enable that the intent should be interpreted by the machine without any ambiguity. However, with the advancement of technologies such as large language models, intents expressed in natural language are gradually becoming the focal point of research and application in intent management. There are some use cases where the use of natural language for intent expression would be preferred. For example, on the human-machine interface presenting and expressing intent using natural language can lead to a more intuitive experience for human users. One Example of natural language RAN ES intent can be “Reduce 10% RAN energy consumption without impact on user experience for Region X”

Some SDOs have also conducted research on intents described in natural language. In TM Forum IG1253 [5], it is mentioned that the intent interpreter can be used to handle intents described in natural language, and the intent interpreter assumes the role of the intent owner for the formal intent. In ETSI GR ZSM 011 [6], it is mentioned that through intent translation, the received natural language intent is mapped to the intent model, and the translation may be based on rules or AI/ML.

Regarding the existing intent management in 3GPP TS 28.312 [2], if intent interpreter is introduced to process intents described in natural language, it is necessary to clarify its position within the existing intent management framework, especially its relationship with the MnS consumer. Additionally, further exploration is needed to assess the impact on the existing intent models and procedures for intent management. Following is the potential deployment scenario to handle the intent expressed with natural language from the consumers who are not technically trained and not familiar with formal intent models, e.g., human users.

Figure 5.6.1-1 Potential deployment scenario to handle the natural language intent

In this deployment scenario, the intent interpreter is a management function as 3GPP intent driven management service, which at least support the following capabilities:

-	Receive the natural language intent from consumers and translate it into a formal intent (the formal intent contains the attributes to enable the IDMS producer to decide the actions to fulfil the intent expectation) according to intent model defined in TS 28.312 [2] and intent handling capabilities (including supported intent object, intent targets) obtained for the intent handling functions. When received the natural language intent, the Intent Interpreter needs to obtain additional information (e.g. current/predicted network performance, current coverage areas (e.g. supported coverageTAClist), expert experience knowledge) to generate the corresponding formal intent instance. Following is an example to interpret the natural language RAN ES intent to 3GPP formal RAN ES intent.

Figure 5.6.1-2 Example of interpreting natural language intent to formal intent

-	Receive intent reports generated by the MnS producer and translate them back into language intent report.

To support Intent interpreter as IDMS consumer to better interpret the natural language intent, some enhancement of intent driven MnS may be needed, for examples,

- Enhance the intent handling capabilities model to state mandatory and default values for each scenario specific intent.

- Enhance the intent feasibility check capability to validate the integrity of intent information.

- Enhance the notFulfilledReasons to include the reason of intent cannot be interpret without ambiguity.

5.6.2	Evaluation of potential solutions

Further study is needed to assess the impact on the existing intent models and procedures for intent management to support natural language intent interpretation.

2nd Change

6.6	Use case #6: Investigate potential impacts to support natural language intents translation

The use case, potential deployment scenario and impact on existing intent driven MnS are investigated in clause 5.6, which require additional investigations in future releases.

End of Changes