3GPP TSG-SA5 Meeting #157	S5-245394

Hyderabad, India, 14 - 18 October 2024

# Source:	China Mobile

# Title:	Add the solution of 5G LAN topology visualization and user plane traffic analysis for TR28.915

# Document for:	Approval

Agenda Item:	6.19.5

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP draft TR 28.915: “Management and orchestration; Study on management aspects of Network Digital Twin v0.1.0”.

[2]	SP-231727 "New Study on management aspects of Network Digital Twin"

3	Rationale

5G LAN-type service defined in 5GS enables Ethernet or IP unicast, multicast or broadcast communication via local switch, N6-based or N19-based forwarding methods. Several UPFs may construct the LAN topology via N19 interface. In the realm of traditional LAN networks, monitoring and safeguarding LAN network from the perils of loops and data storms is of paramount importance. As specified in clause 5.6.10.2 of TS23.501, it can not guarantee the Ethernet network remains loop-free. Deployments need to be verified on an individual basis that loops in the Ethernet network are avoided. Therefore, using Network Detection Technology (NDT) for visualizing 5G LAN topologies and establishing data traffic models can assist operators in forecasting potential user plane data storms.

So, this contribution proposes to add one specific description of 5G LAN topology visualization and user plane traffic analysis in Use case 8 and explain on what data needs to be visualized and why visualization is needed to TR 28.915 based on SP-231727 [2].

4	Detailed proposal

First Changes

5.8	Use case8: Visualization of network topology and traffic

5.8.1	Description

The visualization of the network is helpful in some management capabilities for the network operators.

EXAMPLE 1:	The visualization of network shows the network topology and information of each contained NFs including the overall performance statistics information (including, the number of simultaneous UEs and PDU sessions), this helps to knowledge the real time status and performance related information of the network.

EXAMPLE 2:	Based on the visualization of user or signaling traffic, it helps to quickly detect abnormal traffic and root cause of a service failure.

By collecting and the synchronizing real time data from the mobile network, the management system can create a network digital twin. The created network digital twin can provide the capability of network visualization, which not only shows the topology of the network, but also displays the simulation image of the real network which includes both  network elements (e.g. 5GC NFs or gNB) information and infrastructure resource information.

The consumer could request the NDT for the supported capability of visualization of network topology and traffic and may further receive the detail information (e.g. the location information) for the consumer to obtain the visualization information of the network.

Using the 5G LAN-type service as a specific scenario can effectively demonstrate the aforementioned use case.

5G LAN-type service defined in 5GS enables Ethernet or IP unicast, multicast or broadcast communication via local switch, N6-based or N19-based forwarding methods. Several UPFs may construct the LAN topology via N19 interface. In the realm of traditional LAN networks, monitoring and safeguarding LAN network from the perils of loops and data storms is of paramount importance. As specified below in clause 5.6.10.2 of TS23.501, in release 19, it can not guarantee the network remains loop-free. Deployments need to be verified on an individual basis that loops in the network connection are avoided.

The 5G LAN topology visualization, user plan traffic simulation and storm analysis will be helpful in management capabilities for the network operation.

- 5G LAN topology visualization

By collecting and the synchronizing real time topology data from the mobile network for 5G LAN group, the management system can use NDT to simulate a digital twin of 5G LAN group.

-	When the 5G VN group members' PDU Sessions are served by different PSA UPFs and N19-based forwarding is applied, after the SMF creates a group-level N4 Session with each involved UPF to enable N19-based forwarding and N6-based forwarding, the NDT can simulate the UPF connection topology for this specific 5G VN group.

-	When utilizing Ethernet-based 5G LANs, once the UPF detects devices behind UE, the NDT can simulate the connection status of these devices according to the UPF detected information, while also reflecting the topological connection with the UEs. With such capability, when a new device behind UE connected/removed from the UE, and even a device moved from one UE to another, the NDT can simulate the topological connection change.

2.	5G LAN user plane traffic analysis

In addition, by collecting and synchronizing real-time traffic routing rules from the 5G LAN group in mobile networks, combined with the digital twin of the 5G LAN group topology, the management system can utilize NDT to simulate user plane traffic routing, predicting user plane traffic storms caused by loops.

5.8.2	Potential requirements

REQ-VISUAL_NDT-01: NDT should have a capability to indicate its support visualization of network topology and traffic (e.g., 5G LAN network topology and traffic, including both UPF connection topology and the UE/device behind UE connection topology).

REQ-VISUAL_NDT-02: NDT should have a capability to report the visualization information of the network (e.g., 5G LAN network topology and traffic).

REQ-5GLAN_NDT-03: NDT should have a capability to model (either using emulation method or simulation method) the behaviour of user plane traffic for a 5G LAN network and predict the potential user plane traffic storms caused by loops.

REQ-5GLAN_NDT-04: NDT should have a capability to report the results for user plane traffic storm analysis.

5.8.3	Potential solutions

This solution addresses the following issues of use case 8. When the 5G LAN-type service is used in the network and the traffic analysis is required by MnS consumer, NDT is used for 5G LAN topology visualization and user plan traffic simulation and analysis. The NDT utilizes network related information to generate a report of 5G LAN topology visualization and user plane traffic analysis results with the following approach.

Figure 5.8.3.1-1: NDT for 5G LAN topology visualization and user plane traffic analysis

1.	The MnS consumer sends a request to NDT as the MnS provider for 5G LAN topology visualization and (optional) the user plane traffic modelling, including the simulated network objects (e.g. 5G VN group identifier) and the required actions on the simulated network objects (e.g. 5G LAN topology visualization, user plane traffic modeling).

2.	The NDT as the MnS provider provides a response to MnS consumer indicating the status of the request based on a feasibility check (success or failure).

3.	The NDT as the MnS consumer synchronizes the network objects related information from MnS providers for 5G LAN topology visualization and (optional) the user plane traffic modelling, The network related information for simulation the topology of the 5G LAN may include 5G VN group identifier, 5G VN group topology, N19 tunnel information, device behind UE status, device information, optionally for modelling the user plane traffic, include the routing information (e.g. packet detecting/forwarding rules).

4.	The NDT executes the 5G LAN topology simulation and visualization and optionally the user plane traffic modelling and generates the report.

5.	The NDT as the MnS provider sends the notification of report including the results to MnS consumer. The report can include:

-	Visible 5G LAN network topology: report the visualization information of the 5G LAN.

-	User plane traffic modelling: Use of network simulation to simulate the user plane traffic model.

6.	The MnS consumer sends a request to NDT as the MnS provider for 5G LAN user plane traffic analysis based on the simulated 5G LAN user plan traffic modelling.

7.	The NDT executes the 5G LAN simulation and user plane traffic analysis and generates the report.

8.	The NDT as the MnS provider sends the notification of report including the results to MnS consumer. The report can include:

-	User plane traffic analysis result: Use of network simulation to simulate the user plane traffic impact and predict the potential user plane traffic storm caused by loop and the prevention solution for that risk prediction.

5.8.4	Evaluation of potential solutions

The solution described in clause 5.8.3 provides the NRM extension needed for the NDT to provide the network topology and traffic visualization modelling of network behaviour that supports 5G LAN topology visualization and user plane traffic analysis. The solution allows NDT MnS consumer to request the NDT to do the 5G LAN simulation, visualization and user plane traffic modelling, as well as to analysis traffic based on routing information gathered from the network. Furthermore, the solution can leverage NDT capabilities to predict potential user plane data loops through traffic analysis. Therefore, the normative work on NDT support to 5G LAN topology visualization and user plane traffic analysis should progress following the outline in solution in clause in clause 5.8.3. To support the topology of VN groups and UPF traffic visibility for 5G LAN, the existing SMF IOC and the corresponding KPIs and performance measurements reported by UPF for VN group level can be utilized.

End of Changes