# 3GPP TSG-SA5 Meeting #157	S5-245413

# 14 - 18 October 2024, Hyderabad, India

# Source:	Nokia

# Title:	pCR 28.867 Conclusions and Recommendations

# Document for:	Approval

# Agenda Item:	6.19.4

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.867-041 “Closed control loop management”.

# 3	Rationale

TR28.867 has study several use cases on closed control loops. This pCR is to add conclusions and recommendations for normative work

# 4	Detailed proposal

Start of modification

## 5.10	CCL decision escalation

### 5.10.1	Description

CCLs will make decisions in different contexts (states, status, conditions, etc.) of the network and not all decisions are equally effective. In some cases, the CCL may need to provide feedback indication the challenges faced on reaching a definite decision. The feedback may include a request to escalate its decision making to another entity.

A CCL can be fully or partially autonomous derivation or execution of its decisions. The degree to which the CCL independently executes decisions or escalates them, should be flexibly configurable by the MnS consumer as a confidence threshold. The confidence threshold could be configured based on the sensitivity of the operations under its control, the trust level in the decisions of the CCL and the necessity to consider a bigger picture at times. Then, based on how much confidence the CCL has in its decisions, the CCL can escalate a decision or situation to an escalation recipient which has this bigger picture (say has wider scope), can execute a different(larger) set of actions or has better capabilities, e.g. a larger and more capable ML model.

Note: the computation of confidence within the CCL is up to implementation as it depends on the CCL's purpose and the scenario that the CCL is addressing.

The escalation recipient CCL enables the escalator CCL to request for escalation for a given network context or state with e.g. information about the escalator CCL preferences and observed constraints when driving decisions. Based on its evaluations, the escalation recipient CCL should provide to the escalator CCL a report that holds the outcomes that the CCL (acting as an escalation recipient) has derived for a given escalation request.

NOTE:	The relation with existing Assurance closed loop execution supervision use case in 3GPP TS 28.535 [2] needs to be clarified.

### 5.10.2	Potential requirements

REQ- CCL-ESC-1: The CCL MnS producer should have a capability to enabling an authorized consumer to configure the degree of autonomy of the CCL as characterization of the conditions under which the ACCL should escalate a decision.

REQ- CCL-ESC-2: The CCL MnS producer should have a capability to enabling an authorized consumer to configure the entity to which a decision should be escalated.

REQ-CCL-ESC-3: The CCL MnS producer (acting as an escalation recipient CCL) should have a capability to enabling an authorized MnS consumer (e.g. an escalator CCL) to request escalation of a decision or escalation of decision-making for a given network context or state to the CCL associated with the CCL MnS producer.

REQ-CCL-ESC-4: The 3GPP management system (or the CCL Mns producer) should have a capability enabling an MnS consumer to provide information related to its previous decisions, decision constraints, preferences, … as input to be used in resolving escalations sent towards the CCL associated with the CCL MnS producer.

REQ-CCL-ESC-5: The CCL MnS producer (acting as an escalation recipient CCL) should have a capability to provide to an authorized MnS consumer (e.g. an escalator CCL) a report that holds the outcomes that the CCL (acting as an escalation recipient) has derived for a given escalation request.

### 5.10.3	Possible solutions

Figure 5.10.3-1: End-to-end flow of CCL decision escalation

- Introduce an attribute defining the entity to which the decision is escalated to, say called the escalationRecipient.

- Introduce an attribute on the CCL for defining the condition that triggers the escalation. For example, the CCL may trigger escalation when its level of confidence in the derived decision is below some threshold, say called a confidence threshold. The confidence threshold attribute enables the CCL to autonomously make decisions for each situation and context based on its computed confidence level in the given situation. If the confidence level is lower than the confidence threshold the decision is escalated otherwise the decision is executed.

- Introduce on the CCL representing the escalationRecipient. an IOC representing the request for escalation, say called EscalationRequest, that holds all information related to the request for escalation. The EscalationRequest may include:

-	A proposedCMChange attribute which describes the configuration management changes  that has been proposed by the escalator CCL.

-	A decisionConstraints attribute indicating the constraints observed by the escalating CCL in making the decision(s). The constraints may be of type context as defined in 3GPP TS 28.312 [5] or conditions expressed using JEX/XPATH

-	The EscalationReason attribute can provide an optional description of the reason behind the escalation to provide more context and further clarification.

- The escalationRecipient may send an output representing the outcome of the escalation. Introduce in the AssuranceReport an escalationOutcome data type that holds the outcomes of the escalation. When the escalationOutcome attribute of the report is updated, the escalation recipient CCL sends a notification to the escalator CCL that the report is available:

-	The escalation outcome indicates whether the CCL should take any action and what that action is. Introduce an attribute to indicate what shall be done by the CCL. It is an ENUM with the values:

- "DONOTHING"- indicating that the CCL does not ned to take any action, i.e. the escalationRecipient is addressing the scenario.

- "APPLYACTION"- indicating that the CCL should apply a specific set of actions proposed by the escalationRecipient. The action may be placed a new attribute, say called "proposedActions that list as et of CM changes.

- "APPLYGUIDANCE"- indicating that the CCL should compute a new CM change based on the guidance from the escalationRecipient. The guidance may be placed in the proposedActions attribute.

### 5.10.4	Evaluation of solutions

The solution in clause 5.10.3 provided the procedures and infrmation objects for CCL escalation, enabling interactions for CCLs to escalate decision maming for scenarios where they are unable to decide the best actions. No alternative solution for CCL escalation is provided. It is recommended to apply the solution in in clause 5.10.3 as the solution for CCL escalation.

Next modifications

# 6	Conclusions and Recommendations

## 6.1	Closed control loop and intent

Intent and closed control loops are different, but complementary concepts as discussed in clause 4.5 in Table 4.5-1. An intent manager may utilize one or several closed control loops for implementing the assurance of intent requirements. This means intent managers can become consumers of closed control loops.

A closed control loop that provides (MnS producer) an intent API is configured using intent. This closed control loop therefore meets the definition of being an intent manager in the role of intent handler. If the closed control loop acts by sending intent, it implements the intent owner role. This means, that closed control loop can either be utilized as integral part of intent management or directly implement intent management. However, the functional scope of intent management exceeds the proposed scope of closed control loop. Intent management considers, for example, the negotiation of requirements and intent handlers are already driving deployment decisions and processes. Furthermore, the reporting on achievements and results to the source of intent is an integral part of intent management and realized through intent reports.

The normative work should follow these principles.

## 6.2	Dynamic CCL Creation

it is recommended to develop normative specification for the use case on Dynamic CCL Creation following the solution in in clause 5.1.3.

## 6.3	Triggered CCL

it is recommended to develop normative specification for the use case on Triggered CCL following the solution in in clause 5.2.3.

## 6.4	CCL creation based on Historical CCL data

it is recommended to develop normative specification for the use case on CCL creation based on Historical CCL data following the solution in in clause 5.3.3.

## 6.5	closed control loop for problem recovery

it is recommended to develop normative specification for the use case on closed control loop for problem recovery following the solution in in clause 5.4.3.

## 6.6	CCL for fault management

## 6.7	CCL conflicts management

it is recommended to develop normative specification for :

- the "hierarchical coordination with distributed execution" approach as the means for handling conflicts among CCLs.

- the use case on goal targets conflicts handling following the solution in in clause 5.6.3.3 on goal targets coordination.

- the use case on direct actions conflicts handling following the solution in in clause 5.6.3.4 on direct actions coordination.

- the use case on Indirect targets conflicts handling following the solution in in clause 5.6.3.5 on Indirect targets coordination.

- the use case on Action-execution-time conflicts handling following the solution in in clause 5.6.3.6 on Action-execution-time coordination.

## 6.8	CCL scope management

it is recommended to develop normative specification for the use case on CCL scope conflicts handling following the solution in in clause 5.7.3 on CCL scope coordination.

## 6.9	CCL-impact assessment and resolution

it is recommended to develop normative specification for the use case on CCL-impact assessment and resolution following the solution in in clause 5.8.3.

## 6.10	Consumers feedback on CCL actions

it is recommended to develop normative specification for the use case on Consumers feedback on CCL actions following the solution in in clause 5.9.3.

## 6.11	CCL decision escalation

it is recommended to develop normative specification for the use case on CCL decision escalation following the solution in in clause 5.10.3.

## 6.11	Performance Evaluation of a Closed Control Loop

Next modifications

Annex A:
PlantUML Code for figures

# A.1	Figure 5.6.3.1-2

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class CoordinationCCL <<InformationObjectClass>>

class CoordinationProfile <<dataType>>

ManagedEntity "1" *-- "1" CoordinationCCL: <<names>>

CoordinationCCL "1" -- "*" CoordinationProfile

note left of ManagedEntity

Represents the following IOCs:

Subnetwork or

ManagedFunction

end note

note right of CoordinationProfile

Represents the following capabilities:

GoalTargetCoordination

ScopeAssignmentsCoordination

DirectActionsCoordination

IndirectTargetsCoordination

executionTimeCoordination

end note

@enduml

End of modifications