3GPP TSG-SA5 Meeting #157	S5-245521

# Hyderabad, India, 14 – 18 October 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0608 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 New measurements to monitor Sub-Optimal Handovers (Intra-5GS and Inter-System) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Monitoring functionality by means counters has been added for the Handover failure cases; however, there are no counters defined for the near-failure or sub-optimal success handover cases. Without these measurements, these sub-optimal handover scenarios cannot be statiscally characterized and hence, the operator may not be able to take corrective actions to prevent/reduce these scenarios. In this CR, we propose 6 counters to take care of this major gap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6 new measurements are proposed - Almost Too Early Handovers (Intra-5GS and Inter-System) - Almost Too Late Handovers (Intra-5GS and Inter-System) - RLF After Successful Handovers (Intra-5GS and Inter-System) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operators will be able to measure, analyze and take corrective actions in scenarios where handovers succeed, but not at the optimum level. This covers both Intra-5GS NR handovers as well as Inter-System handovers/ |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Below new clauses are added 5.1.1.6.x, 5.1.1.6.y, A.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

##### 5.1.1.6.x			Handover near failures related to MRO for intra-system mobility

a)	This measurement provides the number of handover near failure events related to MRO detected during the intra-system mobility within 5GS, see TS 38.300 [49] clause 15.5.2. The measurement includes separate counters for various handover near failure types, classified as "Intra-system almost too early handover" and "Intra-system almost too late handover" and “Near failure event followed with RLF report”.

b)	CC.

c)	The measurements of almost too early handovers and almost too late handovers events are obtained respectively by accumulating the number of near failure events detected by gNB during the intra-system mobility within 5GS. The counter for almost too early handovers event is triggered based on the Successful Handover Report received from UE with SHR-cause equal to “t304-cause”, see TS 38.331[20]  clause 6.2.2. The counter is triggered in source NR cell CU and reported per source and target NR cell CU pair. The counter for almost too late handovers event is triggered based on the Successful Handover Report received from UE with SHR-cause equal to “t310/312-cause”, see TS 38.331 [20] clause 6.2.2. The counter is triggered in source NR cell CU and reported per source and target NR cell CU pair.

As the UE stores the SHR until it is fetched by the network or for 48 hours after the SHR is recorded, the network identifies the point in time successful handover to which the SHR is related with backward compatibility based on timeSinceSHR IE reported within the SHR, see TS 38.331 [20] clause 6.2.2.

The measurement of “Near failure event followed with RLF report” is obtained by accumulating the number of near failure events detected by gNB during the intra-system mobility within 5GS which are followed shortly after successful handover with RLF detected by UE in the target NR cell CU. The counter for “Near failure event followed with RLF report” is triggered when trigger conditions for almost too early and/or almost too late handovers event based on the Successful Handover Report received from UE with SHR-cause equal to “t304-cause” and/or “t310/312-cause” is met  which is followed with HO to wrong cell as defined in TS 38.300 [49], clause 15.5.2.2.2. The counter is triggered in source NR cell CU and reported per source and target NR cell CU pair. The network/ NG RAN node of the source NR cell CU identifies the SHR and RLF report sent within the HO report are from the same UE and related to the same mobility event based on the same target C-RNTI reported and timesinceSHR and timesinceFailure in the SHR and RLF report, respectively are pointing to some point in time when the mobility event happened with some tolerance configured by vendor.

d)	Each measurement is an integer value.

e)	HO.IntraSys.AlmostTooEarly
HO.IntraSys.AlmostTooLate

HO.IntraSys.NearFailureRLF

f)		NRCellCU

NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.6.y			Handover near failures related to MRO for inter-system mobility

a)	This measurement provides the number of handover failure events delated to MRO detected during the successful inter-system mobility between NG-RAN and E-UTRAN, limited to the scenarios defined in TS 38.300 [49] clause 15.5.2.2.3. The measurement includes separate counters for various handover failure types, classified as "Inter-system almost too early handover"(inter-system mobility from E-UTRAN to NG-RAN) and "Inter-system almost too late handover" (inter-system mobility from NG-RAN to E-UTRAN) and “Near failure event followed with RLF report”.

b)	CC.

c)	The measurements of almost too early inter-system handover events are obtained by accumulating the number of near failure events detected during the successful inter-system mobility from E-UTRAN to NG-RAN.  The counter for almost too early handovers event is triggered based on the Successful Handover Report received from UE with SHR-cause equal to “t304-cause”, see TS 38.331 [20] clause 6.2.2. The counter is triggered in taregt NR cell CU and reported per source Eutran and target NR cell CU pair. The measurements of almost too late inter-system handover events are obtained by accumulating the number of near failure events detected during the successful inter-system mobility from NG-RAN to E-UTRAN. The counter for almost too late handovers event is triggered based on the Successful Handover Report received from UE with SHR-cause equal to “t310/312-cause”, see TS 38.331[20] clause 6.2.2. The counter is triggered in source NR cell CU and reported per source NR and target Eutran cell CU pair.

As the UE stores the SHR until it is fetched by the network or for 48 hours after the SHR is recorded, the network identifies the point in time successful handover to which the SHR is related with backward compatibility based on timeSinceSHR IE reported within the SHR, see TS 38.331 [20] clause 6.2.2.

The measurement of “Near failure event followed with RLF report” is obtained by accumulating the number of near failure events detected by gNB during the inter-system mobility from NG-RAN to E-UTRAN which are followed shortly after successful handover with RLF detected by UE in the target Eutran cell CU. The counter for “Near failure event followed with RLF report” is triggered when trigger conditions for almost too late handovers event based on the Successful Handover Report received from UE with SHR-cause equal to “t310/312-cause” is met  which is followed with HO to wrong cell as defined in TS 38.300 [49], clause 15.5.2.2.3. The counter is triggered in source NR cell CU and reported per source NR and target Eutran cell CU pair. The network/ NG RAN node of the source NR cell CU identifies the SHR and RLF report are form the same UE and related to the same mobility event based on the same target C-RNTI reported and timesinceSHR and timesinceFailure in the SHR and HO report (which contains target C-RNTI and  timesinceFailure extracted from RLF report in LTE format)  (see 38.423), respectively are pointing to some point in time when the mobility event happened with some tolerance configured by vendor.

d)	Each measurement is an integer value.

e)	HO.InterSys.AlmostTooEarly

HO.InterSys.AlmostTooLate

HO.InterSys. NearFailureRLF

f)	NRCellCU
EutranRelation (contained by NRCellCU)

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

Start of next change

# A.x	Monitoring of Sub-Optimal Handovers

There are 2 categories of MRO procedures for both intra-RAT and inter-RAT mobility.

- Category-1: MRO related to failed mobility procedures (originally specified in R16)

- Category-2: MRO related to successful, but sub-optimal mobility procedures (added in R17/R18, TS 38.300 [49], Clause 15.5.2.7)

Corresponding to these Category-1 MRO procedures, monitoring functionality/measurements was also added in the current document for intra and inter RAT handover in clause 5.1.1.25.1 and 5.1.1.25.2, respectively.

These measurements could be used to quantify these failures and derive statistical significance of each type of failure and take appropriate actions/optimizations in the various thresholds related to the HO procedure. These measurements are based on the RLF Report and provided the monitoring of the following types of failures :

too early handovers

too late handovers

handover to wrong cell

However, there are no MRO performance measurements for the Category-2 MRO procedures given above. In other words, there is no monitoring available for SHR at all. Because of this limitation, there is no means for the network to monitor the performance of the almost failed cases and no way to do any statistical analysis of issues like below

What percentage of UEs do sub-optimal Handovers?

What percentage of UEs do “almost late” and/or “almost early” handovers?

Are there any patterns in sub-optimal Handovers (in terms of time of HO or place of HO or cell load) etc.,?

Is there any specific type of sub-optimal Handover that is more prevalent than the others?

Currently, there is no means to answer the above questions using standardized measurements defined in 3GPP. We propose to address this lacuna in this invention by proposing measurements for these scenarios and their associated counting rules

This CR proposes a method for monitoring of the risky or failed but recovered mobility cases which finally resulted in success handover via providing new performance measurements for the following scenarios

“Almost too early” Handover

“Almost too late” Handover

These near failure case which are pegged based on successful handover report (SHR) received from UE with the t310/t312-cause cause and t304-cause, respectively. The measurements are pegged per the source and target PCell pair.

We also propose a new measurement for the so called near failure event followed shortly after successful handover with RLF in the target cell, characterized by an SHR followed by an RLF Report in the Target cell.

So, 3 counters are proposed each (ie totally, 6 counters) for intra-5GS and inter-system Handover procedures

End of change