3GPP TSG-SA5 Meeting #157	S5-245621

Hyderabad, India, 14 - 18 October 2024

# Source:	Nokia

# Title:	Discussion paper on renewable energy information

# Document for:	Endorsement

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is asked to endorse the detailed proposal in section 4.

# 2	References

[1]	3GPP TR 28.880: “Study on energy efficiency and energy saving aspects of 5G networks and services”

[2]	S5‑244743: pCR TR 28.880 Potential solution for carbon emission related information - Non-split gNB Renewable Energy Factor

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

[4] 	ITU-T Recommendation L.1333 (09/22): "Carbon data intensity for network energy performance monitoring".

[5]	ETSI EN 203 228: Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[6]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[7]	SP-241388: "New WID on Energy Efficiency and Energy Saving ".

# 3	Rationale

## 3.1	Introduction

SA1 TS 22.261 [3] and SA5 TR 28.880 [1] have worked in Rel-19 on requirements related to “renewable energy”  (consumption) information reporting (e.g., monitoring, exposure etc).

The ratio (or percentage) of renewable energy source of the network elements, based on infrastructures hosting network elements (operator sites and ICT sites), is being proposed to be reported (see clause 5.3 of TR 28.880 [1] and S5‑244743[2]), within or outside the 5G network, as a metric for various purposes, e.g., to select the most appropriate NF in the network (see WT #3 of Rel-19 SA2 WID SP-241388[7]).

In both ETSI and ITU-T, KPIs have been defined mainly for use cases related to annual reporting, rather than for operational use of this information within the network to optimize it dynamically. Such KPIs not only cover the “total” ratio of renewable energy, but also cover the “grid” and “on-site” ratio of renewable energy (see ETSI EN 303 472 [6]) and the “network carbon intensity” (see ITU-T Recommendation L.1333 [4]).

From 3GPP perspective, we believe it would be interesting to follow this more detailed information to be measured and collected, considering:

- Grid (market-based) vs on-site energy consumption (i.e. via local energy production plant or backup systems) as an indication to discriminate the “cost of energy” for an operator.

- Carbon emissions, to be used for own operator’s needs or exposed to 3rd parties in relation to their subscription, e.g., for ESG reporting.

We believe that such metrics, beyond the single “total ratio of renewable energy”, could be effective and useful for 3GPP information exposure and decision-making – beyond annual KPI reporting. For example, both information above could be used at operational level throughout the network, e.g., to select NFs, adapt service, expose information etc.

This paper discusses the current planned usage of renewable energy information within 3GPP, in particular related to two underlying aspects of “renewable”: 1) carbon emissions and 2) energy costs.

It is structured as follows:

- Which usage of renewable energy information is being considered in the context of 3GPP?

- Considerations on reporting GHG emission metric, e.g., CO2e emissions, emission factor, carbon intensity.

- Considerations on reporting the energy source information for the sake of enabling management of energy cost.

## 3.2	Discussion

- Which usage of renewable energy information is being considered in the context of 3GPP?

As per SA1 clause 6.15a of TS22.261 [3], “Energy related information can include ratio of renewable energy and carbon emission information when available. Calculation of energy related information as described in the following requirements is done by means of averaging or applying a statistical model. The requirements do not imply that some form of 'real time' monitoring is required.”

In particular, the following SA1 requirements have been defined, which could apply to “the ratio of renewable energy” as part of “energy-related information”:

Requirements from clause 6.15a.2.2 of TS 22.261 [3]:

The 5G system shall provide a mechanism to include Energy related information as part of charging information.

Subject to operator policy and agreement with 3rd party, the 5G system shall provide a mechanism to support the selection of an application server based on energy related information associated with a set of application servers.

Subject to user consent and operator policy, 5G system shall be able to provide means to modify a communication service based on energy related information criteria based on subscription policies.

Requirements from clause 6.15a.5.2 of TS 22.261 [3]:

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption for serving this 3rd party.

NOTE 1: Energy consumption information can include ratio of renewable energy and carbon emission information when available. The reporting period could be set, e.g., on monthly or yearly basis and can vary based on location.

NOTE 2: The energy consumption information can be related to the network resources of network slice, NPNs, etc.

Based on these requirements, stage 2 work has started to consider energy-related information (explicitly referring to “renewable energy”) for topics such as:

- NF selection based on energy-related information, to meet those requirements targeting internal network optimisation and “to modify a communication service”

- Collection and/or (estimated) calculation of energy consumption information at, e.g. per UE granularity – mainly “to expose information on energy consumption for serving this 3rd party”.

- Considerations on reporting GHG emission metric e.g. CO2e emissions, emission factor, carbon intensity

As highlighted above, in some cases, renewable energy information (e.g. percentage or ratio) is propagated throughout the 5G network as a way to indicate a clean or “low-carbon” energy supply mix, to be exposed to third parties or used internally, e.g., to select an NF.

However, such a simple ratio is not always directly interpretable: in some cases the total ratio of renewable energy can be higher in one energy mix vs another (i.e., closer to 100%), but the overall carbon intensity of that energy mix is actually higher as well (i.e., worse). A typical reason is that the remaining (i.e., non-renewable) energy of the first energy mix has higher carbon intensity than the second one.

As an example, based on electricitymaps.com, in August 2024,

- US/Kansas energy grid provider had “65%” of “renewable” energy as part of their supply mix (71% of “low carbon”) with overall carbon intensity of 237gCO2eq/kWh

- CA/Ontario energy grid provider had “34%” of “renewable” energy as part of their supply mix (87% of “low carbon”) with overall carbon intensity of 72gCO2eq/kWh

- Reason for it is that the remaining 35% of Kansas’ energy supply mix is based mainly on coal, which has a very high carbon intensity.

- Note: For electricitymaps.com, “low carbon” = “renewable” + nuclear

- Considerations on reporting the energy source information for the sake of enabling management of energy cost

In other cases, the renewable energy percentage is propagated throughout the 5G network in order to make decisions, e.g. select an NF. However, this does not discriminate where this “renewable energy” comes from and its cost impact for the operator. Indeed operators can have “Green” PPAs (Power Purchase Agreement) for purchasing grid energy, whilst in some location their own on-site energy production (e.g. via solar panels or windmills) cover some percentage of their site energy. Without discriminating this type of (renewable) energy source, the operator cannot make an informed decision to prioritize a NE/NF simply based on the “total” percentage of renewable energy powering it.

ITU-T Recommendation L.1333 [4] considers 3 types of energy sources: “grid”, “locally generated” and “backup” energy. The last two refer to “on-site” energy generation.

ETSI ES 203 228 [5] has identified the possible source of renewable energy of a “telecommunication site” in the figure 3.2-1 below (Source figure 3 of ETSI ES 203 228 [5]). This figure clarifies the distinction between renewable energy from the grid and renewable energy produced on-site, for example at a base station. However it does not distinguish between “locally generated” and “backup” energy.

Figure 3.2-1: Schematic representation of the energy sources for a site (Source figure 3 of ETSI ES 203 228 [5])

# 4	Detailed proposal

This paper proposes to endorse the following recommendations.

Proposal #1: Definitions:

- Rely on ITU-T Recommendation L.1333 [4] definition of energy types in order to introduce metrics related to “locally generated” renewable energy, vs “backup” energy, vs “grid” energy

- Rely on ITU-T Recommendation L.1333 [4] definitions for “carbon intensity”

NOTE: although such definitions either apply to KPIs (ETSI) or to the entire “network” (ITU-T), the wording, semantic meaning and potential units should be aligned within 3GPP for metrics.

Proposal #2: For energy cost-related aspects of “renewable energy percentage”,

- Split the renewable energy percentage metrics for network elements and network functions into “locally generated”, “backup” and “grid” sources rather than only one total ratio of renewable energy.

Proposal #3: For carbon emission-related aspects of “renewable energy percentage”,

- Introduce “carbon emission factor” and “carbon intensity” information and/or metrics for at network elements and network functions level

- Split this metric also based on “locally generated”, “backup” and “grid” energy sources rather than only one total combined ratio for carbon information.