3GPP TSG-SA5 Meeting #157	S5-245687

Hyderabad , IN, 14 - 18 October 2024

# Source: Ericsson-LG Co

# Title: PCR TR 28.858 Fine-Tunning Potential Solution and Evaluation

# Document for:Approval

Agenda Item: AIML_MGT_Ph2

# 1	Decision/action requested

approval

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.105: Artificial Intelligence / Machine Learning (AI/ML) management".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

# 3	Rationale

Update Description and Use case for ML Fine tuning as well as adding Possible solution #2 to clause 5.1.4.X, which ensures compatibility with existing standards as well introduces a generic attribute TrainingType. Add evaluation description in clause 5.1.4.5.

# 4	Detailed proposal

1nd Change

### 5.1.3	ML Fine-tuning

#### 5.1.3.1 	Description

ML fine-tuning refers to the process of training an already pre-trained ML model on a more specific task or domain. This involves adjusting the MLModel parameters to better suit the new scope leveraging the knowledge it has already learned.

In contrasts,  re-training of an ML Model generally involves training the ml model from scratch without any prior specialization for a particular type of confidence. Fine-tunning , on the other hand , focuses on optimizing the ml model for a  specific type of inference based on the new data provided.

The ML Model training process defined in TS 28.105 includes provisions for fine-tuning pre-tained models to better align them with targeted tasks.

#### 5.1.3.2	Use cases

##### 5.1.3.2.1		ML fine-tuning for a pre-trained ML model

A pre-trained ML model may be initially trained on a dataset that supports various types of inference, i.e. aimlInferenceName. However, fine-tuning allows the model to specialize in a specific type/types of inference, aimlInferenceName . The producer can fine-tune the pre-trained model to optimize it for these targeted inference/s.

NOTE: It is for FFS whether fine-tuning process could be applied for the same producer who pre-trained the model or for another producer.

#### 5.1.3.3	Potential requirements

REQ-ML_ TRAIN-MLFT-1: The ML training MnS producer shall have a capability to enable an authorized consumer to request the fine-tuning of a pre-trained ML model.

Editor’s note: It is for further discussion if the same requirements in existing TS 28.105 Table 6.2a.1.3-1 [2]  related to training function also applies for fine-tuning.

#### 5.1.3.4.X.1	Possible solutions#1

The exsting MLModel IOC and MLTrainingRequest IOC can be enhanced to support fine-tuning.

Enhancement Aspects on MLTrainingRequest IOC:  Since the exsting MLTrainingRequest IOC cannot support consumer requested ML fine-tuning, both following enhancements are possible.

- FineTuningIndication, indicates the consumer requested ML fine-tuning. The support qualifier should be CM, which means this attribute should be present when the MLTrainingRequest MOI represents the request for ML Model Fine-tuning. Besides, change the support qualifier of “aIMLInferenceName” to CM, which means that this attribute should be present when the MLTrainingRequest MOI does not represent the request for ML Model Fine-tuning.

Enhancement Aspects on MLModel IOC:  Since fine-tuning is aiming at a specific inference type for a pre-trained model, the AIML capabilities would be narrowed  which has the best performance. It is proposed to introduce following attributes:

- FinetuningModelIndication, indicates that the ML Model is a Finetuning Model. Besides, change the support qualifier of “aIMLInferenceName” to CM, where the condition is that the ML Model requested to train is not a pre-training ML Model.

- PotentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

#### 5.1.3.4.X.2	Possible solutions#2

The existing ML training procedures in TS 28.105 [2] already provide all the necessary mechanisms for initial training and re-training of an ML model. The existing MLTrainingRequest IOC may be used to support fine-tuning, similarly to how it is used for the re-training procedure of an ML model.

The key distinction between retraining and fine-tuning is that retraining results in a new version of the same ML model, while fine-tuning produces results in a new ML model, which has a different mlModelId, MLModelRef, and aIMLInferenceName than the ML model that was requested to be fine-tuned.

To enable fine-tuning within the current framework, the aIMLInferenceName attribute in the MLTrainingRequest IOC needs be modified to be a writable attribute, allowing for an the model to specialize in a specific type/types of inference.

#### 5.1.3.5	Evaluation

Potential Solution #2 offers a simpler, more efficient, and flexible approach to fine-tunning an ML model by reusing existing attributes , IOCs and mechanisms in TS 28.105 [2]. It minimizes complexity, aligns with current standards making it the preferred solution over Solution #1.

Therefore, the solution outlined in clause 5.1.2.4.X is more feasible and requires no additional development in the normative specifications.

2nd Change