3GPP TSG-SA5 Meeting #157 	S5-245765

Hyderabad, India, 14 - 18 October 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0141 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel 19 CR TS 28.104 Remove Support Qualifier from attribute constraints |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_ph2 |  |  |  |  |  | Date: |  |  | 2024-08-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following changes to 32.160 the TS is updated to remove support qualifier from attribute constraints tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support Qualifier is removed from attribute constraints table throughout the spec |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with template in 32.160 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2.3, 9.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of modification

### 8.5.2	Recommended3GPPAction <<dataType>>

#### 8.5.2.1	Definition

This data type specifies the data type of recommended 3GPP action.

#### 8.5.2.2	Information elements

Table 8.5.2.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| mOInstance | Identifies the instance of a common ancestor object of the objects for which changes are recommended. | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| path | The "path" and "mOInstance" identify the object, attribute, attribute field or multi-value attribute element, that are recommended for creation, deletion or modification. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| op | It specifies the type of operation that is recommended for the MOI specified by the path. allowedValues: "add", "remove" and "replace". The operation describes what is recommended to do to the NRM. "add" shall be used for recommending the creation of an object, attribute, attribute field or multi-value attribute element. "remove" shall be used for recommending the deletion of an object, attribute, attribute field or multi-value attribute element. "replace" shall be used for recommending the replacement of an existing attribute value, attribute field value or multi-value attribute element. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| value | If an object creation is recommended with "add", the "value" shall carry a complete representation of the created object. If an object deletion is recommended with "remove", the "value" shall be absent. It may optionally carry a complete representation of the deleted object. If an attribute, attribute field or multi-value attribute element creation is recommended with "add", the "value" shall carry the value of the recommended attribute, attribute field or multi-value attribute element. If an attribute, attribute field or multi-value attribute element deletion is recommended with "remove", the "value" shall be absent. If the replacement of an attribute, attribute field or multi-value attribute element value is recommended with "replace", the "value" shall carry the new value of the attribute, attribute field or multi-value attribute element. If multiple objects are recommended for creation, the creation of parent objects shall be recommended before the child objects. | CM | type: AttributeValuePair (see TS 32.156 [18]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| additionalText | It provides the additional text for the recommended change. | O | type: String multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |

#### 8.5.2.3	Constraints

Table 8.5.2.3-1

| Name | Definition |
| --- | --- |
| value | Condition: value of op attribute is "add", or "replace". |

Next modification

### 9.3.1	MDAFunction

#### 9.3.1.1	Definition

The IOC MDAFunction represents the MDA function which supports one or more MDA capabilities. The MDA function may be supported by AI/ML. Attribute mLModelRefList indicates that AI/ML is supported for this function. Attribute AIMLInferenceFuntionRefList indicates that AI/ML Inference Function is supported for this function.

#### 9.3.1.2	Attributes

Table 9.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedMDACapabilities | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| mLModelRefList | CM | T | F | F | T |
| aIMLInferenceFunctionRefList | CM | T | F | F | T |

#### 9.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |
| mLModelRefList | The condition is "The MDA function is supported by ML Model". |
| aIMLInferenceFunctionRefList | The condition is "The MDA function is supported by AI/ML inference function". |

#### 9.3.1.4	Notifications

The common notifications defined in clause 9.6 are valid for this IOC, without exceptions or additions.

End of modifications