3GPP TSG-SA5 Meeting #157	S5-245785

Hyderabad, India, 14 - 18 October 2024

# Source:	China Unicom

# Title:	New WID on Management of Network Sharing Phase 3

# Document for:	Approval

# Agenda Item:	6.2.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	New WID on Management of Network Sharing Phase 3

{Free text. It has to be the same as in the "Title:" section above. Studies have to start by "Study on"}

Acronym:	NetShare_OAM_Ph3

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
| X | Normative – Stage 1 |
| X | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_NetShare_OAM_Ph2 | SA5 | 1020015 | Study on Management of Network Sharing Phase3 |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1030011 | (Stage 2 of) Indirect Network Sharing | SA2 work item on Network Sharing |
| 1000029 | (Stage 1 of) Indirect Network Sharing | SA1 work item on Network Sharing |

3	Justification

Network sharing is a crucial approach to decrease CAPEX of 5G constructions for operators. As stated in TS 22.261 the increased density of access nodes needed to meet future performance objectives pose considerable challenges in deployment and acquiring spectrum and antenna locations. RAN sharing is seen as a technical solution to these issues. Plenty of previous normative works have been completed during Rel-17 and Rel-18. Several TSGs, including SA1 and SA2, CT1, CT4, have been keeping track of this topic in Rel-19.

In TR 28.878, following issues for MOCN and Indirect Network Sharing have been studied.

For MOCN, firstly, the management system of MOP should have the capability to allow authorized POP users to access, read and download data of configuration, performance metrics (including performance measurement and KPI) and alarms. Secondly, different POPs require different data for personalized operation and maintenance services, including collecting different trace and performance data requirements. For example, the different trace data or performance data with collection time window, collection requirements, etc., should be reported for each POP.

For Indirect Network Sharing, each POP in the Indirect Network Sharing scenario could have requirements to manage S-RAN. The hosting operator, as MOP in the Indirect Network Sharing scenario, should have capabilities to allow authorized consumers to manage S-RAN. Correspondingly, RAN and CN configurations should be enhanced to support Indirect Network Sharing. To differ the operator-specific traffic, performance measurement in PLMN granularity in NFs of CN should be enhanced.

The use cases, potential requirements and the corresponding potential solutions of FS_NetShare_OAM_Ph3 are provided in TR 28.878. Normative work based on the study is needed for these aspects of management enhancement related to MOCN and Indirect Network Sharing.

4	Objective

The objective of the work item is to specify how to enhance the management of Network Sharing based on the study in TR 28.878, including:

WT-1: Enhance the capability to support operator-specific management data collection jobs.

WT-2: Enhance the deployment example of service-based management architecture for MOCN defined in TS 32.130.

WT-3: The management enhancements supporting Indirect Network Sharing, including:

WT-3.1: Add  descriptions to support management of Indirect Network Sharing in TS 32.130.

WT-3.2: Enhance the RAN and CN configurations and add descriptions and workflow to support configurations of  Indirect Network Sharing.

WT-3.3: Enhance the PLMN granularity performance measurement for NFs of CN.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) | Non-3GPP Dependency (EE/ZSM/TMF etc.) |
| --- | --- | --- | --- | --- | --- |
| WT-1 | 1 | 1 | no | no | no |
| WT-2 | 0.5 | 0.5 | no | no | no |
| WT-3 | 0.5 | 0.5 | no | Maybe(SA1 and SA2) | no |

Total TU estimates for the study phase: 2

Total TU estimates for the normative phase: 2

Total TU estimates: 4

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 28.552 | Enhance the PLMN granularity performance measurement for NFs of CN. | Jun 2025 (SA#108) |  |
| TS 32.130 | - Add descriptions and workflow to support management of Indirect Network Sharing. - Enhance the deployment example of service-based management architecture. | Jun 2025 (SA#108) |  |
| TS 28.541 | - Enhance the capability to support operator-specific management data collection jobs. - Enhance the RAN and CN configurations to support Indirect Network Sharing. | Jun 2025 (SA#108) |  |
| TS 28.622 | Enhance the capability to support operator-specific management data collection jobs. | Jun 2025 (SA#108) |  |

6	Work item Rapporteur(s)

7	Work item leadership

SA5

8	Aspects that involve other WGs

Coordinate with SA1 and SA2

9	Supporting Individual Members

Supporting IM name

China Unicom

China Telecom

Huawei

CATT

Intel

ZTE

AsiaInfo