3GPP TSG SA WG5 Meeting #157	S5-245907

Hyderabad, India, 14 - 18 October 2024

# Source:	Huawei

# Title:	New charging solution for roaming A2X Communication

# Document for:	Approval

Agenda Item:	7.5.4

1 	Discussion

This is a pCR to provide the Aircraft-to-Everything for UAS charging in TR 28.853.

2	References

[1]	3GPP TR 28.853: "Study on charging aspects of uncrewed aerial systems".

3	Rationale

This pCR proposes to introduce the roaming solution for Aircraft-to-Everything service for UAS Charging.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.853 [1].

First change

5.4.4.3	Solution #4.3: Roaming A2X Communication over Uu reference point

5.4.4.3.1	General description

This solution#4.3 which relying on CHF/5G Converged Charging System for Roaming A2X Communication, addresses the Key Issue#4.1.

The charging for A2X communication over Uu reference point using the PDU session and MBS session in roaming case, including HR and LBO scenarios, refers to TS 23.501 [4], TS 23.502 [5] and TS 23.247 [9].

5.4.4.3.2	Architecture description

The 5G System high level charging architecture for SMF/MB-SMF charging in roaming cases to support the A2X communication roaming scenarios.

5.4.4.3.3	Procedures description

The high-level charging procedure for A2X communication over Uu reference point in roaming case can refer to the PDU session charging as specified in the TS 32.255[10] and MBS session charging as specified in the TS 32.255[10] and TS 32.279[11].

End of change