3GPP TSG-SA5 Meeting #157	S5-245915

Hyderabad, India, 14 - 18 October 2024

# Source:	Ericsson

# Title:	Clarification of topic 3 use cases

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1]	3GPP TS 28.846 Study on charging aspects of satellite access phase 3

# 3	Rationale

Update of the use cases to use the business roles defined.

# 4	Detailed proposal

This document proposes the following changes in TR 28.846 [1].

First change

## 6.3	Topic 3: Charging for satellite resource usage between SSP and MNO

### 6.3.1	Use cases

#### 6.3.1.1	Use Case #1.1: SSP charging MNO for satellite used for backhaul

MNO has an agreement for rental of the satellite from SSP to be used for backhaul.

The charging party and charged party can be:

-	Charged party: MNO.

-	Charging party: SSP.

This charging could be based on the total data volume transferred via the satellite.

#### 6.3.1.2	Use Case #1.2: SSP charging MNO for edge computing via UPF deployed on the satellite

SSP provides the satellite infrastructure resources to MNO to enable the EAS to be running on the satellite.

The charging party and charged party can be:

-	Charged party: MNO

-	Charging party: SSP

This charging could be based on usage of satellite per EAS related to EAS deployment (EAS deployment, EAS modification, EAS termination) and infrastructure resource (virtual CPU usage, virtual memory usage, virtual disk usage, data volumes).

End of change