3GPP TSG-SA5 Meeting #157	S5-246008

Hyderabad, India, 14 - 18 October 2024

# Source:	Rakuten Mobile

# Title:	Add declarative section and evaluation of solutions for modification of a NF deployment instance

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

For approval

# 2	References

[1] 	3GPP TR 28.869: “Study on cloud aspects of management and orchestration”.

# 3	Rationale

Added a section to discuss usage of declarative descriptor and another for evaluation of solutions for modification of a NF deployment instance.

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.869 [1].

1st change

##### 5.2.4.3.2	Use of deployment management reference point based on declarative descriptor

In this solution, the 3GPP management system interacts with an orchestration and management system using the deployment management reference point as described in clause 5.2.1.3 for modification of a NF deployment instance based on declarative descriptor.

The modified requirements of a NF deployment instance are conveyed from the 3GPP management system to the orchestration and management system via a modified version of the declarative descriptor that was used for creation of the NF deployment instance. It shall be possible to reference to modified parts, or different parts in an existing declarative descriptor as part of the declarative modification request.

End of changes